# RESTRICTED ACCESS

| DISTRIBUTION : | Firm | To | Ref | Copies | 1st page | e-mail |
| --- | --- | --- | --- | --- | --- | --- |
|  | FCE | Xavier Christmann, Claude Redon, Adel Bassaid | 8 |  |  |  |
|  | FCE | Hamza Zetti, Lory Médas, Sabrine Bouazizi, Wail Amri, Kérima Adjadi, Mickael Pastor, Aki Saito | 10 |  |  |  |
|  | RBE | D. Andris, M. Ianos, A. Negrea, M. Obada, S. Dominte | 5 |  |  |  |
|  | FCE | Secretary ship | 1 |  |  |  |

SW Architecture Design & Interface Description :

# DIA sw UNIT

OBJECT:	This document is the description of the design & interfaces for DIA SW unit.

SUMMARY:	This document provides a high-level view of the DIA SW unit. The inputs of this document are provided by the software requirement. It is linked to the DAIMLER_MMA_SWarchitectureDesignInterfaceDescription.docx document.

CONCLUSION:	Applicable from R8.0 SW release

THIS DOCUMENT CONTAINS HIDDEN TEXT

EVOLUTION OF THE DOCUMENT

| Issue | Date | Author | Motive and nature of the modifications |
| --- | --- | --- | --- |
| A00 | 31/08/2016 | C. Redon | First release (extract from the full PP4G architecture document) |
| A01 | 23/09/2016 | J. Lacroix | Update of requirement reference |
| A02 | 12/10/2016 | P-O Pilot | Add new requirements for some services (Variable access / Called functions). Add DIA_runGetTxNotification runnable. |
| A03 | 18/11/2016 | C. Redon | Correction of traceability issues |
| Start extended description based on mainstream document |  |  |  |
| 1.1.1.2 | 11/07/2019 | A. Vaché | Update traceability to match PP4G extended platform requirements IDs |
| 1.1.1.3 | 19/07/2019 | A. Vaché | Remove traceability to no more existing requirements |
| 1.1.1.4 | 08/08/2019 | A. Vaché | Solve some traceability issues highlighted by reqtify |
| 1.1.1.5 | 18/11/2019 | W. AMRI | Add missing traceability, and correct PTC issues: 801001, 801041, 801077 |
| 1.1.1.10 | 26/11/2019 | W. AMRI | Update template information |
| 1.1.1.11 | 28/01/2020 | A. Diankouuka | Update requirements to be covered |
| Start DAI MMA description based on mainstream document |  |  |  |
| 1.5.4.1 | 06/01/2022 | A Negrea | First Revision |
| 1.5.4.2 | 06/01/2022 | A Negrea | Duplicate Revision |
| 1.5.4.3 | 21/01/2022 | A Negrea | Add specific DID |
| 1.5.4.4 | 02/02/2022 | A. Negrea | Add specific project DID for production |
| 1.5.4.5 | 16/02/2022 | A. Negrea | Update after review |
| 1.5.4.6 | 17/03/2022 | A. Negrea | Update according to SRM |
| 1.5.4.7 | 23/03/2022 | A. Negrea | Update according to SRM2 |
| 1.5.4.8 | 30/03/2022 | A. Negrea | Update according to SRM3 |
| 1.5.4.9 | 09/06/2022 | E. Jivan | Update for R02.05 release |
| 1.5.4.10 | 15/06/2022 | E. Jivan | Update after review |
| 1.5.4.11 | 16/06/2022 | E. Jivan | Added missing traceability DAI_EXT_TF_E_8177 |
| 1.5.4.12 | 17/06/2022 | E. Jivan | Removed routine 0330. |
| 1.5.4.13 | 17/06/2022 | E. Jivan | Updated table of contents. |
| 1.5.4.14 | 19/08/2022 | E. Jivan | Update for R3.0 and missing traceability. |
| 1.5.4.15 | 19/08/2022 | E. Jivan | Updated missing traceability. |
| 1.5.4.16 | 19/08/2022 | E. Jivan | Corrected traceability. |
| 1.5.4.17 | 23/08/2022 | E. Jivan | Updated missing req ID and readability. |
| 1.5.4.18 | 23/08/2022 | E. Jivan | Corrected size for 0x0302 |
| 1.5.4.19 | 23/08/2022 | E. Jivan | Corrected content of 0x0302 |
| 1.5.4.20 | 23/08/2022 | E. Jivan | Corrected 0x23, returned data for 0x0310 and 0x0302 |
| 1.5.4.21 | 14/11/2022 | T. Gligor | Update for R4.0 Release |
| 1.5.4.22 | 14/11/2022 | T. Gligor | Update for R4.0 Release |
| 1.5.4.23 | 14/11/2022 | R. Cristoi | Added traceability for ROE on DTC status change |
| 1.5.4.24 | 14/11/2022 | T. Gligor | Update for R4.0 Release (minor naming changes) |
| 1.5.4.25 | 22/11/2022 | T. Gligor | Update for R4.0 Release (corrections for ROE) |
| 1.5.4.26 | 24/11/2022 | T. Gligor | Updates for R4.0 Release (full document check for traceability issues) |
| 1.5.4.27 | 24/11/2022 | T. Gligor | Updates for R4.0 Release (update 3 missing functions) |
| 1.5.4.28 | 06/12/2022 | T. Gligor | Updates for R4.0 Release (after Review) |
| 1.5.4.29 | 16/12/2022 | T. Gligor | Update for R4.0 Release (add a leading 0 for 3 requirements) |
| 1.5.4.30 | 21/12/2022 | T. Gligor | Update for R5.0 Release (initial R5.0 revision) |
| 1.5.4.31 | 10/02/2023 | T. Gligor | Update interfaces for R5.0 Release |
| 1.5.4.32 | 13/02/2023 | T. Gligor | Update traceability with TF_B - Can Input requirements |
| 1.5.4.33 | 16/02/2023 | T. Gligor | Update traceability with missing TF_E requirements + review |
| 1.5.4.34 | 13/03/2023 | T. Gligor | Fix postponed findings from R5.0 Release |
| 1.5.4.35 | 12/04/2023 | E. Jivan | Added 0x0210 Can Output read |
| 1.5.4.36 | 08/05/2023 | T. Gligor | Updates for R6.0 Release |
| 1.5.4.37 | 09/05/2023 | T. Gligor | Updates for R6.0 Release (Update with TF_B Traceability) |
| 1.5.4.38 | 09/05/2023 | T. Gligor | Updates for R6.0 Release (Update with TF_B Traceability) |
| 1.5.4.39 | 11/05/2023 | T. Gligor | Updates for R6.0 Release (Update with TF_B Traceability) |
| 1.5.4.40 | 19/06/2023 | T. Gligor | Updates for R6.1 Release |
| 1.5.4.41 | 20/06/2023 | T. Gligor | Updates for R6.1 Release (Correct duplicates) |
| 1.5.4.42 | 20/10/2023 | T. Gligor | Updates for R7.1 Release |
| 1.5.4.43 | 27/10/2023 | T. Gligor | Updates for R7.1 Release (After Reqtify Report) |
| 1.5.4.44 | 29/11/2023 | M. Serban | Update for R8.1 Release (Update with TF_B Traceability) |
| 1.5.4.44 | 07/12/2023 | T. Gligor | Updates for R8.1 Release |
| 1.5.4.45 | 08/12/2023 | T. Gligor | Update for R8.1 Release (After Reqtify Report) |
| 1.5.4.46 | 08/01/2024 | T. Gligor | Update ARCH_SW_DIA_0187 (R8.1) |
| 1.5.4.47 | 11/01/2024 | M. Serban | Added missing requirement |
| 1.5.4.48 | 28/03/2024 | T. Gligor | Updates for R9.0 Release |
| 1.5.4.49 | 01/04/2024 | T. Gligor | Updates for R9.0 Release |

This document contains 224 pages.

CONTENTS

1.	Documentation	18

1.1.	Upper Level Relevant Documents	18

1.2.	Design interface description Documents	19

1.3.	Design Specification Documents	20

1.4.	Tier2 Documents	21

1.5.	HW Datasheet	22

1.6.	Other Documents	22

1.7.	Glossary And Definition	23

2.	Technical functions	24

3.	Description	37

3.1.	To handle a diagnostic tool request with immediate positive answer	38

3.2.	To handle a diagnostic tool request with differed positive answer	40

3.3.	To answer negative response	42

4.	Runnables	43

4.1.	DIA_runDidF18C_ECUSerialNumberRead	43

4.1.1.	Definition	43

4.1.2.	Data flow / Parameters	43

4.2.	DIA_runDidF18C_EcuSerialNumberConditionCheckRead	43

4.2.1.	Definition	43

4.3.	DIA_runDidF18C_EcuSerialNumberReadDataLength	44

4.3.1.	Definition	44

4.3.1.	Data flow / Parameters	44

4.4.	DIA_runDID0340_UniqueSeatbeltSerialNumberRead	44

4.4.1.	Definition	44

4.4.2.	Data flow / Parameters	45

4.5.	DIA_runDid0340_UniqueSeatbeltSerialNumberConditionCheckRead	45

4.5.1.	Definition	45

4.6.	DIA_runDID0340_UniqueSeatbeltSerialNumberWrite	45

4.6.1.	Definition	45

4.6.2.	Data flow / Parameters	46

4.7.	DIA_runDidFD2F_EcuLocationRead	46

4.7.1.	Definition	46

4.7.1.	Data flow / Parameters	46

4.8.	DIA_runDidFD10_CycleConfigurationRead	47

4.8.1.	Definition	47

4.8.2.	Data flow / Parameters	47

4.9.	DIA_runDidFD10_CycleConfigurationWrite	48

4.9.1.	Definition	48

4.9.2.	Data flow / Parameters	48

4.10.	DIA_runDidFD2A_StepsLibraryRead	49

4.10.1.	Definition	49

4.10.2.	Data flow / Parameters	49

4.11.	DIA_runDidFD2A_StepsLibraryWrite	49

4.11.1.	Definition	49

4.11.2.	Data flow / Parameters	50

4.12.	DIA_runDidFD03_AEETraceabilityNumberWrite	51

4.12.1.	Definition	51

4.12.2.	Data flow / Parameters	51

4.13.	DIA_runDid0244_PreSafeCounterRead	51

4.13.1.	Definition	51

4.13.2.	Data flow / Parameters	52

4.14.	DIA_runDid0244_PreSafeCounterConditionCheckRead	52

4.14.1.	Definition	52

4.15.	DIA_runDid0244_PreSafeCounterReadDataLength	52

4.15.1.	Definition	52

4.15.1.	Data flow / Parameters	53

4.16.	DIA_runDid0243_BSRCounterRead	53

4.16.1.	Definition	53

4.16.2.	Data flow / Parameters	53

4.17.	DIA_runDid0243_BSRCounterConditionCheckRead	54

4.17.1.	Definition	54

4.18.	DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_ConditionCheckRead	54

4.18.1.	Definition	54

4.19.	DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_ReadData	54

4.19.1.	Definition	54

4.19.2.	Data flow / Parameters	55

4.20.	DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_WriteData	55

4.20.1.	Definition	55

4.20.2.	Data flow / Parameters	56

4.21.	DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ConditionCheckRead	56

4.21.1.	Definition	56

4.22.	DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ReadData	56

4.22.1.	Definition	56

4.22.2.	Data flow / Parameters	57

4.23.	DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ReadData_WriteData	57

4.23.1.	Definition	57

4.23.2.	Data flow / Parameters	57

4.24.	DataServices_Tensioning_Cycle_BSR_CombinedSignal_ConditionCheckRead	58

4.24.1.	Definition	58

4.25.	DataServices_Tensioning_Cycle_BSR_CombinedSignal_ReadData	58

4.25.1.	Definition	58

4.25.2.	Data flow / Parameters	58

4.26.	DataServices_ Tensioning_Cycle_BSR_CombinedSignal_WriteData	59

4.26.1.	Definition	59

4.27.	DIA_runDid0310_VehicleEquipment_ConditionCheckRead	59

4.27.1.	DataServices_Vehicle_Equipment_Read_Equipment_1_ConditionCheckRead	59

4.27.2.	DataServices_Vehicle_Equipment_Read_Equipment_2_ConditionCheckRead	60

4.27.3.	DataServices_Vehicle_Equipment_Read_Equipment_3_ConditionCheckRead	60

4.27.4.	DataServices_Vehicle_Equipment_Read_Equipment_4_ConditionCheckRead	61

4.27.5.	DataServices_Vehicle_Equipment_Read_Equipment_5_ConditionCheckRead	61

4.27.6.	DataServices_Vehicle_Equipment_Read_Equipment_6_ConditionCheckRead	62

4.27.7.	DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_ConditionCheckRead	62

4.27.8.	DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_ConditionCheckRead	62

4.27.9.	DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_ConditionCheckRead	63

4.27.10.	DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_ConditionCheckRead	63

4.28.	DIA_runDid0310_VehicleEquipment_ReadData	64

4.28.1.	DataServices_Vehicle_Equipment_Read_Equipment_1_ReadData	64

4.28.2.	DataServices_Vehicle_Equipment_Read_Equipment_2_ReadData	65

4.28.3.	DataServices_Vehicle_Equipment_Read_Equipment_3_ReadData	65

4.28.4.	DataServices_Vehicle_Equipment_Read_Equipment_4_ReadData	66

4.28.5.	DataServices_Vehicle_Equipment_Read_Equipment_5_ReadData	67

4.28.6.	DataServices_Vehicle_Equipment_Read_Equipment_6_ReadData	67

4.28.7.	DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_ReadData	68

4.28.8.	DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_ReadData	68

4.28.9.	DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_ReadData	69

4.28.10.	DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_ReadData	70

4.29.	DIA_runDid0310_VehicleEquipment_WriteData	70

4.29.1.	DataServices_Vehicle_Equipment_Read_Equipment_1_WriteData	70

4.29.2.	DataServices_Vehicle_Equipment_Read_Equipment_2_WriteData	71

4.29.3.	DataServices_Vehicle_Equipment_Read_Equipment_3_WriteData	72

4.29.4.	DataServices_Vehicle_Equipment_Read_Equipment_4_WriteData	72

4.29.5.	DataServices_Vehicle_Equipment_Read_Equipment_5_WriteData	73

4.29.6.	DataServices_Vehicle_Equipment_Read_Equipment_6_WriteData	73

4.29.7.	DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_WriteData	74

4.29.8.	DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_WriteData	75

4.29.9.	DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_WriteData	75

4.29.10.	DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_WriteData	76

4.30.	DIA_runDid0311_VehicleEquipmentDevelopment_ConditionCheckRead	77

4.30.1.	DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_ConditionCheckRead	77

4.30.2.	DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_ConditionCheckRead	77

4.31.	DIA_runDid0311_VehicleEquipmentDevelopment_ReadData	78

4.31.1.	DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_ReadData	78

4.31.2.	DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_ReadData	78

4.32.	DIA_runDid0311_VehicleEquipmentDevelopment_WriteData	79

4.32.1.	DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_WriteData	79

4.32.2.	DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_WriteData	80

4.33.	DIA_runDidFD08_ResetCauseManagementRead	80

4.33.1.	Definition	81

4.33.2.	Data flow / Parameters	81

4.34.	DIA_runDidFD08_ResetCauseManagementConditionCheckRead	82

4.34.1.	Definition	82

4.35.	DIA_runDidFD09_LastWarmResetRead_LastDetectedWarmResetCause	82

4.35.1.	Definition	82

4.35.2.	Data flow / Parameters	82

4.36.	DIA_runDidFD09_LastWarmReseConditionCheckRead_LastDetectedWarmResetCause	83

4.36.1.	Definition	83

4.37.	DIA_runDidFD09_LastWarmResetRead_ResetCauseCounter	83

4.37.1.	Definition	83

4.37.2.	Data flow / Parameters	84

4.38.	DIA_runDidFD09_LastWarmResetConditionCheckRead_ResetCauseCounter	84

4.38.1.	Definition	84

4.39.	DIA_runDidFD31_HBCalibrationRead	84

4.39.1.	Definition	84

4.39.2.	Data flow / Parameters	84

4.40.	DIA_runDidFD31_HBCalibrationWriteConditionCheckRead	85

4.40.1.	Definition	85

4.41.	DIA_runDidFE00_AECStatusRead	85

4.41.1.	Definition	85

4.41.2.	Data flow / Parameters	86

4.42.	DIA_runDidFE00_AECStatusConditionCheckRead	86

4.42.1.	Definition	86

4.43.	DIA_runDidFE00_AECStatusReadDataLength	86

4.43.1.	Definition	86

4.43.1.	Data flow / Parameters	87

4.44.	DIA_runDidFEFF_MeasurementFrameConfigurationRead	87

4.44.1.	Definition	87

4.44.1.	Data flow / Parameters	87

4.45.	DIA_runDidFEFF_MeasurementFrameConfigurationConditionCheckRead	87

4.45.1.	Definition	87

4.46.	DIA_runDidFDFF_MeasurementFrameBlock12ConfigRead	88

4.46.1.	Definition	88

4.46.2.	Data flow / Parameters	88

4.47.	DIA_runDidFDFF_MeasurementFrameBlock12ConfigConditionCheckRead	89

4.47.1.	Definition	89

4.48.	DIA_runDidFDFF_MeasurementFrameBlock12ConfigWrite	89

4.48.1.	Definition	89

4.48.2.	Data flow / Parameters	89

4.49.	DIA_runSid23_ReadMemoryByAddress	90

4.49.1.	Definition	90

4.49.2.	Data flow / Parameters	90

4.50.	DIA_runSid3D_WriteMemoryByAddress	90

4.50.1.	Definition	90

4.50.1.	Data flow / Parameters	91

4.51.	DIA_runRidF703_CycleExecutionStart	92

4.51.1.	Definition	92

4.51.2.	Data flow / Parameters	92

4.52.	DIA_runRidF703_CycleExecutionStop	93

4.52.1.	Definition	93

4.52.2.	Data flow / Parameters	93

4.53.	DIA_runRid0329_TriggerBSRStart	93

4.53.1.	Definition	93

4.53.2.	Data flow / Parameters	94

4.54.	DIA_runRid0329_TriggerBSRStop	94

4.54.1.	Definition	94

4.54.2.	Data flow / Parameters	94

4.55.	DIA_runRid0329_TriggerBSRRequestResults	95

4.55.1.	Definition	95

4.55.2.	Data flow / Parameters	95

4.56.	DIA_runRid0330_TriggerTensioningCycleStart	95

4.56.1.	Definition	95

4.56.2.	Data flow / Parameters	96

4.57.	DIA_runRid0330_TriggerTensioningCycleStop	96

4.57.1.	Definition	96

4.57.2.	Data flow / Parameters	96

4.58.	DIA_runRid0330_TriggerTensioningCycleRequestResults	97

4.58.1.	Definition	97

4.58.2.	Data flow / Parameters	97

4.59.	DIA_runRid0302_TriggerPresafeDisplayRequestRequestResults	97

4.59.1.	Definition	97

4.59.2.	Data flow / Parameters	98

4.60.	DIA_runRid0302_TriggerPresafeDisplayRequestStart	98

4.60.1.	Definition	98

4.60.2.	Data flow / Parameters	99

4.61.	DIA_runRid0302_TriggerPresafeDisplayRequestStop	99

4.61.1.	Definition	99

4.61.2.	Data flow / Parameters	99

4.62.	DIA_runDid0142_AvailabilityDataRead	100

4.62.1.	Definition	100

4.62.2.	Data flow / Parameters	100

4.63.	DIA_runDid0241_PRE_SAFE_Recorder_1_ReadDataLength	100

4.63.1.	Definition	100

4.63.2.	Data flow / Parameters	101

4.64.	DIA_runDid0241_PRE_SAFE_Recorder_1_ConditionCheckRead	101

4.64.1.	Definition	101

4.64.2.	Data flow / Parameters	101

4.65.	DIA_runDid0241_PRE_SAFE_Recorder_1_ReadData	102

4.65.1.	Definition	102

4.65.2.	Data flow / Parameters	102

4.66.	DIA_runDid0241_PRE_SAFE_Recorder_1_WriteData	102

4.66.1.	Definition	102

4.66.2.	Data flow / Parameters	103

4.67.	DIA_runDid0242_PRE_SAFE_Recorder_2_ReadDataLength	103

4.67.1.	Definition	103

4.67.2.	Data flow / Parameters	103

4.68.	DIA_runDid0242_PRE_SAFE_Recorder_2_ConditionCheckRead	104

4.68.1.	Definition	104

4.68.2.	Data flow / Parameters	104

4.69.	DIA_runDid0242_PRE_SAFE_Recorder_2_ReadData	104

4.69.1.	Definition	104

4.69.2.	Data flow / Parameters	105

4.70.	DIA_runDid0242_PRE_SAFE_Recorder_2_WriteData	105

4.70.1.	Definition	105

4.70.2.	Data flow / Parameters	105

4.71.	RoELight_DTCStatusChanged_Callback	106

4.71.1.	Definition	106

4.71.2.	Data flow / Parameters	106

4.72.	DIA_runClearDiagRequestCycle	107

4.72.1.	Definition	107

4.73.	DIA_runGetDiagRequestCycle	107

4.73.1.	Definition	107

4.74.	DIA_cyclicProcessingTask	107

4.74.1.	Definition	107

4.75.	ResetServiceExecution	107

4.75.1.	Definition	107

4.75.2.	Data flow / Parameters	108

4.76.	Dcm_Svc14Handler	108

4.76.1.	Definition	108

4.76.2.	Data flow / Parameters	109

4.77.	Dcm_Svc19Handler	109

4.77.1.	Definition	109

4.77.2.	Data flow / Parameters	110

4.78.	DIA_runDidFD01_SWVersionConditionCheckRead	112

4.78.1.	Definition	112

4.79.	DIA_runDidFD01_SWVersionRead	112

4.79.1.	Definition	112

4.79.2.	Data flow / Parameters	113

4.80.	Dcm_Service27Processor	113

4.80.1.	Definition	113

4.80.2.	Data flow / Parameters	114

4.81.	DIA_runDidFD2F_EcuLocationWrite	114

4.81.1.	Definition	114

4.81.2.	Data flow / Parameters	114

4.82.	DIA_runDidFD31_HBCalibrationWrite	115

4.82.1.	Definition	115

4.82.2.	Data flow / Parameters	115

4.83.	DataServices_Current_Operating_Time_Read_Current_Operating_Time_ConditionCheckRead	116

4.83.1.	Definition	116

4.84.	DataServices_Current_Operating_Time_Read_Current_Operating_Time_ReadData	116

4.84.1.	Definition	116

4.84.2.	Data flow / Parameters	116

4.85.	DIA_runDid00140_Obsolescence_Data_Current_Value_ConditionCheckRead	117

4.85.1.	DIA_runDid00140_Obsolescence_Data_BSR_Current_Value_ConditionCheckRead	117

4.85.2.	DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Current_Value_ConditionCheckRead	117

4.85.3.	DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Current_Value_ConditionCheckRead	118

4.85.4.	DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Current_Value_ConditionCheckRead	118

4.86.	DIA_runDid00140_Obsolescence_Data_Threshold _ConditionCheckRead	118

4.86.1.	DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Green_To_Yellow_ConditionCheckRead	118

4.86.2.	DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Yellow_To_Red_ConditionCheckRead	119

4.86.3.	DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Green_To_Yellow_ConditionCheckRead	119

4.86.4.	DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Yellow_To_Red_ConditionCheckRead	120

4.86.5.	DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Green_To_Yellow_ConditionCheckRead	120

4.86.6.	DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Yellow_To_Red_ConditionCheckRead	120

4.86.7.	DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Green_To_Yellow_ConditionCheckRead	121

4.86.8.	DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Yellow_To_Red_ConditionCheckRead	121

4.87.	DIA_runDid00140_Obsolescence_Data_Current_Value_Read	122

4.87.1.	DIA_runDid00140_Obsolescence_Data_BSR_Current_Value_Read	122

4.87.2.	DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Current_Value_Read	122

4.87.3.	DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Current_Value_Read	122

4.87.4.	DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Current_Value_Read	123

4.87.5.	Data flow / Parameters	123

4.88.	DIA_runDid00140_Obsolescence_Data_Threshold_Read	123

4.88.1.	DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Green_To_Yellow_Read	123

4.88.2.	DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Yellow_To_Red_Read	124

4.88.3.	DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Green_To_Yellow_Read	124

4.88.4.	DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Yellow_To_Red_Read	125

4.88.5.	DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Green_To_Yellow_Read	125

4.88.6.	DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Yellow_To_Red_Read	125

4.88.7.	DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Green_To_Yellow_Read	126

4.88.8.	DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Yellow_To_Red_Read	126

4.88.9.	Data flow / Parameters	126

4.89.	DataServices_CPU_Load_Read_CPU_load_histograms_ConditionCheckRead	127

4.89.1.	Definition	127

4.90.	DataServices_CPU_Load_Read_CPU_load_histograms_ReadDataLength	127

4.90.1.	Definition	127

4.90.2.	Data flow / Parameters	128

4.91.	DataServices_CPU_Load_Read_CPU_load_histograms_ReadData	128

4.91.1.	Definition	128

4.91.2.	Data flow / Parameters	128

4.92.	DataServices_E2E_Timing_Read_Maximum_ConditionCheckRead	129

4.92.1.	Definition	129

4.93.	DataServices_E2E_Timing_Read_Maximum_ReadDataLength	129

4.93.1.	Definition	129

4.93.2.	Data flow / Parameters	129

4.94.	DataServices_E2E_Timing_Read_Maximum_ReadData	130

4.94.1.	Definition	130

4.94.2.	Data flow / Parameters	130

4.95.	DataServices_Stack_Load_Read_Maximum_ConditionCheckRead	130

4.95.1.	Definition	130

4.96.	DataServices_Stack_Load_Read_Maximum_ReadDataLength	131

4.96.1.	Definition	131

4.96.2.	Data flow / Parameters	131

4.97.	DataServices_Stack_Load_Read_Maximum_ReadData	131

4.97.1.	Definition	131

4.97.2.	Data flow / Parameters	132

4.98.	DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ConditionCheckRead	132

4.98.1.	Definition	132

4.99.	DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ReadDataLength	132

4.99.1.	Definition	132

4.99.2.	Data flow / Parameters	133

4.100.	DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ReadData	133

4.100.1.	Definition	133

4.100.2.	Data flow / Parameters	133

4.101.	DIA_runDid0200_ CAN_input_ConditionCheckRead	134

4.101.1.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_1_ConditionCheckRead	134

4.101.2.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_2_ConditionCheckRead	134

4.101.3.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_3_ConditionCheckRead	134

4.101.4.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_4_ConditionCheckRead	135

4.101.5.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_5_ConditionCheckRead	135

4.101.6.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_6_ConditionCheckRead	136

4.101.7.	DIA_runDid0200_CAN_input_Read_Supply_battery_voltage_ConditionCheckRead	136

4.101.8.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_7_ConditionCheckRead	136

4.101.9.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_8_ConditionCheckRead	137

4.102.	DIA_runDid0200_ CAN_input_Read	137

4.102.1.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_1_ReadData	137

4.102.2.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_2_ReadData	137

4.102.3.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_3_ReadData	138

4.102.4.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_4_ReadData	138

4.102.5.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_5_ReadData	138

4.102.6.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_6_ReadData	139

4.102.7.	DIA_runDid0200_CAN_input_Read_Supply_battery_voltage_ReadData	139

4.102.8.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_7_ReadData	140

4.102.9.	DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_8_ReadData	140

4.102.10.	Data flow / Parameters	140

4.103.	DataServices_HardwareVersion_Read_HW_ReadData	144

4.103.1.	DataServices_HardwareVersion_Read_HW_patch_level_ConditionCheckRead	144

4.103.2.	DataServices_HardwareVersion_Read_HW_patch_level_ReadData	145

4.103.3.	DataServices_HardwareVersion_Read_HW_week_ConditionCheckRead	145

4.103.4.	DataServices_HardwareVersion_Read_HW_week_ReadData	145

4.103.5.	DataServices_HardwareVersion_Read_HW_year_ConditionCheckRead	146

4.103.6.	DataServices_HardwareVersion_Read_HW_year_ReadData	146

4.103.7.	Data flow / Parameters	146

4.104.	DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ConditionCheckRead	147

4.104.1.	Definition	147

4.105.	DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ReadDataLength	147

4.105.1.	Definition	147

4.105.2.	Data flow / Parameters	148

4.106.	DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ReadData	148

4.106.1.	Definition	148

4.106.2.	Data flow / Parameters	148

4.107.	DataServices_BootSoftwareVersion_Read_Boot_SW_ReadData	149

4.107.1.	DataServices_BootSoftwareVersion_Read_Boot_SW_patch_level_ConditionCheckRead	149

4.107.2.	DataServices_BootSoftwareVersion_Read_Boot_SW_patch_level_ReadData	149

4.107.3.	DataServices_BootSoftwareVersion_Read_Boot_SW_week_ConditionCheckRead	149

4.107.4.	DataServices_BootSoftwareVersion_Read_Boot_SW_week_ReadData	150

4.107.5.	DataServices_BootSoftwareVersion_Read_Boot_SW_year_ConditionCheckRead	150

4.107.6.	DataServices_BootSoftwareVersion_Read_Boot_SW_year_ReadData	151

4.107.7.	Data flow / Parameters	151

4.108.	DIA_runDid0241_PRE_SAFE_Recorder_1_WriteData	151

4.108.1.	Definition	151

4.108.2.	Data flow / Parameters	152

4.109.	DIA_runDid0242_PRE_SAFE_Recorder_2_WriteData	152

4.109.1.	Definition	152

4.109.2.	Data flow / Parameters	153

4.110.	DataServices_PRE_SAFE_Counter_read_Read_CombinedDataElement_WriteData	153

4.110.1.	Definition	153

4.110.2.	Data flow / Parameters	153

4.111.	Dcm_Svc28Handler	154

4.111.1.	Definition	154

4.111.2.	Data flow / Parameters	154

4.112.	Dcm_Svc85Handler	155

4.112.1.	Definition	155

4.112.2.	Data flow / Parameters	155

4.113.	RoELite_ObsolescenceDataHandler	156

4.113.1.	Definition	156

4.113.1.	Data flow / Parameters	157

4.114.	RoELite_AvailabilityDataHandler	157

4.114.1.	Definition	157

4.114.2.	Data flow / Parameters	157

4.115.	DiagSessionChangeCallback	158

4.115.1.	Definition	158

4.115.2.	Data flow / Parameters	158

4.116.	DataServices_Global_Time_Sync_Measured_Values_ConditionCheckRead	159

4.116.1.	Definition	159

4.117.	DataServices_Global_Time_Sync_Measured_Values_ReadData	159

4.117.1.	Definition	159

4.117.1.	Data flow / Parameters	159

4.118.	DataServices_Global_Time_Sync_Measured_Values_ReadDataLength	160

4.118.1.	Definition	160

4.118.2.	Data flow / Parameters	160

4.119.	DIA_runDid0210_CAN_output_Read	160

4.119.1.	DIA_runDid0210_CAN_output_Read_CANoutput1_ConditionCheckRead	160

4.119.2.	DIA_runDid0210_CAN_output_Read_CANoutput1_ReadData	160

4.119.3.	DIA_runDid0210_CAN_output_Read_PS_Curve_XX_Stat_ST3_ConditionCheckRead	161

4.119.4.	DIA_runDid0210_CAN_output_Read_PS_Curve_XX_Stat_ST3_ReadData	162

4.119.5.	DIA_runDid0210_CAN_output_Read_RBTMxx_Actv_Lvl_ST35_ConditionCheckRead	162

4.119.6.	DIA_runDid0210_CAN_output_Read_RBTMxx_Actv_Lvl_ST35_ReadData	163

4.119.7.	DIA_runDid0210_CAN_output_Read_RBTMxx_SP_ActvClient_ST35_ConditionCheckRead	163

4.119.8.	DIA_runDid0210_CAN_output_Read_RBTMxx_SP_ActvClient_ST35_ReadData	164

4.119.9.	DIA_runDid0210_CAN_output_Read_RBTMxx_SP_FctPrecond_Stat_ST35_ConditionCheckRead	164

4.119.10.	DIA_runDid0210_CAN_output_Read_RBTMxx_SP_FctPrecond_Stat_ST35_ReadData	164

4.120.	DIA_runDidFD50_Hardware_Version_Information_Write	165

4.120.1.	DIA_runDidFD50_Hardware_Version_Information_Write_HW_patch_level	165

4.120.2.	DIA_runDidFD50_Hardware_Version_Information_Write_HW_week	165

4.120.3.	DIA_runDidFD50_Hardware_Version_Information_Write_HW_year	166

4.121.	DIA_ runDid0220_HW_input_Read_ConditionCheckRead	166

4.121.1.	DIA_runDid0220_HW_input_Read_Voltage_KL30t_ConditionCheckRead	166

4.121.2.	DIA_runDid0220_HW_input_Read_Voltage_Motor_ConditionCheckRead	167

4.122.	DIA_ runDid0220_HW_input_Read	167

4.122.1.	DIA_runDid0220_HW_input_Read_Voltage_KL30t_ReadData	167

4.122.2.	DIA_runDid0220_HW_input_Read_Voltage_Motor_ReadData	168

4.123.	DIA_ runDid0240_Internal_ECU_Information_Read_ ConditionCheckRead	169

4.123.1.	DIA_runDid0240_Internal_ECU_information_Read_Autosar_System_State_EcuM_State_ConditionCheckRead	169

4.123.2.	DIA_runDid0240_Internal_ECU_information_Read_ECU_Temperature_ConditionCheckRead	169

4.123.3.	DIA_runDid0240_Internal_ECU_information_Read_MOTOR_Current_ConditionCheckRead	169

4.124.	DIA_ runDid0240_Internal_ECU_Information_ Read	170

4.124.1.	DIA_runDid0240_Internal_ECU_information_Read_Autosar_System_State_EcuM_State_ReadData	170

4.124.2.	DIA_runDid0240_Internal_ECU_information_Read_ECU_Temperature_ReadData	171

4.124.3.	DIA_runDid0240_Internal_ECU_information_Read_MOTOR_Current_ReadData	171

4.125.	DIA_runDidFD03_ECUTraceablityNumberConditionCheckRead	172

4.125.1.	Definition	172

4.126.	DIA_runDidFD03_ECUTraceablityNumberRead	172

4.126.1.	Definition	172

4.126.2.	Data flow / Parameters	173

4.127.	DIA_runDidFD03_PCBAProgrammedPartNumberConditionCheckRead	173

4.127.1.	Definition	173

4.128.	DIA_runDidFD03_PCBAProgrammedPartNumberRead	173

4.128.1.	Definition	173

4.128.2.	Data flow / Parameters	174

4.129.	DIA_runDidFD03_ECUTraceablityNumberWrite	174

4.129.1.	Definition	174

4.129.2.	Data flow / Parameters	174

4.130.	DIA_runDidFD03_PCBAProgrammedPartNumberWrite	175

4.130.1.	Definition	175

4.130.2.	Data flow / Parameters	175

4.131.	DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ConditionCheckRead	175

4.131.1.	Definition	175

4.132.	DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ReadData	176

4.132.1.	Definition	176

4.132.2.	Data flow / Parameters	176

4.133.	DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ReadDataLength	177

4.133.1.	Definition	177

4.133.2.	Data flow / Parameters	177

4.134.	DataServices_Vehicle_Odometer_in_Low_Resolution_Read_Vehicle_Odometer_in_Low_Resolution_ConditionCheckRead	177

4.134.1.	Definition	177

4.135.	DataServices_Vehicle_Odometer_in_Low_Resolution_Read_Vehicle_Odometer_in_Low_Resolution_ReadData	178

4.135.1.	Definition	178

4.135.2.	Data flow / Parameters	178

4.136.	DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_ConditionCheckRead	179

4.136.1.	Definition	179

4.137.	DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_ReadData	179

4.137.1.	Definition	179

4.137.2.	Data flow / Parameters	179

4.138.	DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_ConditionCheckRead	180

4.138.1.	Definition	180

4.139.	DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_ReadData	180

4.139.1.	Definition	180

4.139.2.	Data flow / Parameters	181

4.140.	DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_WriteData	181

4.140.1.	Definition	181

4.140.2.	Data flow / Parameters	181

4.141.	DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_WriteData	182

4.141.1.	Definition	182

4.141.2.	Data flow / Parameters	182

4.142.	DataServices_Task_Response_Time_Read_Task_histograms_ConditionCheckRead	182

4.142.1.	Definition	183

4.143.	DataServices_Task_Response_Time_Read_Task_histograms_ReadData	183

4.143.1.	Definition	183

4.143.2.	Data flow / Parameters	183

4.144.	DataServices_Task_Response_Time_Read_Task_histograms_ReadDataLength	184

4.144.1.	Definition	184

4.144.2.	Data flow / Parameters	184

4.145.	DataServices_System_Time_read_Read_System_time_ConditionCheckRead	184

4.145.1.	Definition	184

4.146.	DataServices_System_Time_read_Read_System_time_ReadData	185

4.146.1.	Definition	185

4.146.2.	Data flow / Parameters	185

4.147.	DiagCommonData – EF00 / EF01 / EF03 / EF05 / F100 / F104 / F10D / F111 / F121 / F154 / F155	186

4.147.1.	EF00 - ECU Extract Version	186

4.147.2.	EF01 - Diagnostic Extract Version	186

4.147.3.	EF03 - Standard Application SW Package Information	187

4.147.4.	EF05 - Minimum SW Version	188

4.147.5.	F100 - Active Diagnostic Information	189

4.147.6.	F104 - ECU Name	190

4.147.7.	F10D - DDS Package Release	191

4.147.8.	F111 - MBC ECU Identification - Hardware Part Number	191

4.147.9.	F121 - MBC ECU Identification - Software Logical Block Part Number(s)	192

4.147.10.	F154 - Hardware Supplier Identification	193

4.147.11.	F155 - Software Supplier Identification	194

4.148.	DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ConditionCheckRead	195

4.148.1.	Definition	195

4.149.	DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ReadData	195

4.149.1.	Definition	195

4.149.2.	Data flow / Parameters	195

4.150.	DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ReadDataLength	196

4.150.1.	Definition	196

4.150.2.	Data flow / Parameters	196

4.151.	DataServices_Supported_configuration_mechanisms_Read_Config_mechanisms_ConditionCheckRead	196

4.151.1.	Definition	196

4.152.	DataServices_Supported_configuration_mechanisms_Read_Config_mechanisms_ReadData	197

4.152.1.	Definition	197

4.152.2.	Data flow / Parameters	197

4.153.	DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ConditionCheckRead	198

4.153.1.	Definition	198

4.154.	DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ReadData	198

4.154.1.	Definition	198

4.154.2.	Data flow / Parameters	198

4.155.	DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ReadDataLength	199

4.155.1.	Definition	199

4.155.2.	Data flow / Parameters	199

4.156.	DataServices_Calibration_Identifications_CAL_ID_Write_CombinedDataElement_WriteData	199

4.156.1.	Definition	199

4.156.2.	Data flow / Parameters	200

4.157.	DataServices_Calibration_Identifications_CAL_ID_Write_Number_of_data_items	200

4.157.1.	Definition	200

4.157.2.	Data flow / Parameters	201

4.158.	DataServices_Synchronize_to_Non_volatile_Memory_Start	201

4.159.	DataServices_Synchronize_to_Non_volatile_Memory_Stop	201

4.160.	DataServices_Synchronize_to_Non_volatile_Memory_RequestResults	202

4.161.	Ssa_ProcCtrl_RoutineServices_Routine_Replace_Trust_model_Certificates	203

4.162.	DataServices_Check_Reprogramming_Preconditions_Routine_Start	204

4.163.	DataServices_PreCheckProgrammingDependencies_Start	205

4.164.	DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_ReadData	206

4.164.1.	Definition	206

4.164.2.	Data flow / Parameters	207

4.165.	DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_ConditionCheckRead	207

4.165.1.	Definition	207

4.165.2.	Data flow / Parameters	207

4.166.	DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_WriteData	208

4.166.1.	Definition	208

4.166.2.	Data flow / Parameters	208

4.167.	DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_ReadData	208

4.167.1.	Definition	208

4.167.2.	Data flow / Parameters	209

4.168.	DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_ConditionCheckRead	209

4.168.1.	Definition	209

4.168.2.	Data flow / Parameters	210

4.169.	DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_WriteData	210

4.169.1.	Definition	210

4.169.2.	Data flow / Parameters	210

4.170.	RoutineServices_Routine_Erase_All_Start	211

4.171.	RoutineServices_Routine_Force_External_WDG_Start	212

4.172.	DataServices_Customer_Settings_Read_Customer_Settings_ConditionCheckRead	213

4.172.1.	Definition	213

4.173.	DataServices_Customer_Settings_Read_Customer_Settings_ReadData	213

4.173.1.	Definition	213

4.173.2.	Data flow / Parameters	214

4.174.	DataServices_Customer_Settings_Read_Customer_Settings_ReadDataLength	214

4.174.1.	Definition	214

4.174.2.	Data flow / Parameters	214

4.175.	DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ConditionCheckRead	215

4.175.1.	Definition	215

4.176.	DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ReadData	215

4.176.1.	Definition	215

4.176.2.	Data flow / Parameters	215

4.177.	DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ReadDataLength	216

4.177.1.	Definition	216

4.177.2.	Data flow / Parameters	216

4.178.	DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_WriteData	217

4.178.1.	Definition	217

4.178.2.	Data flow / Parameters	217

4.179.	DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ConditionCheckRead	219

4.179.1.	Definition	219

4.180.	DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ReadData	219

4.180.1.	Definition	219

4.180.2.	Data flow / Parameters	219

4.181.	DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ReadDataLength	220

4.181.1.	Definition	220

4.181.2.	Data flow / Parameters	220

4.182.	DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ConditionCheckRead	220

4.182.1.	Definition	220

4.183.	DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ReadData	221

4.183.1.	Definition	221

4.183.2.	Data flow / Parameters	221

4.184.	DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ReadDataLength	221

4.184.1.	Definition	221

4.184.2.	Data flow / Parameters	222

4.185.	DIA_runDID0340_UniqueSeatbeltSerialNumberWrite	222

4.185.1.	Definition	222

4.185.2.	Data flow / Parameters	222

4.186.	RoutineServices_Routine_Clear_Resource_Consumption_Data_Start_Start	223

5.	MCU resources	224

TABLE OF FIGURES

Figure 1: DIA - Static description\	19

Figure 2: DIA – To handle a diagnostic tool request with immediate positive answer	20

Figure 3: DIA – To handle a diagnostic tool request with differed positive answer	22

Figure 4: DIA – To answer negative response	23

# Documentation

## Upper Level Relevant Documents

This section presents all the documents needed to write the software architecture design document.

| NB. | Document | Reference | Company |
| --- | --- | --- | --- |
|  | TF-A: To Manage the power supply | /RevAS/30_DES_Requirements/Technical Functions/ DES_TF_A_To_Manage_The_Power_Supply | RBE/FCE |
|  | TF-B: To Manage the communication | /RevAS/30_DES_Requirements/Technical Functions/ DES_TF_B_To_Manage_The_Communication | RBE/FCE |
|  | TF-C: To Secure PP ECU functioning using Pictus MCU | /RevAS/30_DES_Requirements/Technical Functions/ DES_TF_C_To_Secure_PP_ECU_Functioning_Pictus | RBE/FCE |
|  | TF-D: To Program MCU | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_D_To_Program_MCU | RBE/FCE |
|  | TF-E: To Manage Diagnostic Requests | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_E_To_Manage_Diagnostic_Requests | RBE/FCE |
|  | TF-F: To Perform Measurements | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_F_To_Perform_Measurements | RBE/FCE |
|  | TF-G: To Drive the Motor | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_G_To_Drive_the_Motor | RBE/FCE |
|  | TF-H: To Perform Autotests | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_H_To_Perform_Autotests | RBE/FCE |
|  | TF-I: To Manage the Failure | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_I_To_Manage_The_Failure | RBE/FCE |
|  | TF-J: To Manage NVM - NVP (Non Volatile Parameters) | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_J_To_Manage_NVM | RBE/FCE |
|  | TF-K: To Ensure ECU Protection and Integration | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_K_To_Ensure_ECU_Protection_And_Integration | RBE/FCE |
|  | TF-L: To Ensure ECU Integration in Environment EMC ESD | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_L_To_Ensure_ECU_Integration_In_Environment_EMC_ESD | RBE/FCE |
|  | TF-M: To generate time base | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_M_To_Generate_Time_Base | RBE/FCE |
|  | TF-N: To evaluate belt data | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_N_To_Evaluate_Belt_Data | RBE/FCE |
|  | TF-O: To schedule the SW | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_O_To_Run_SW | RBE/FCE |
|  | TF-P: To handle network management | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_P_To Handle_Network_Management | RBE/FCE |
|  | TF-Q: To Provide Data For Expertise | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_Q_To_Provide_Data_For_Expertise | RBE/FCE |
|  | TF-R: To Decide Belt Function Execution | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_R_To_Decide_Belt_Function_Execution | RBE/FCE |
|  | TF-S: To drive the boost | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_S_To_Drive_Boost | RBE/FCE |
|  | TF-X: To generate time base | /RevAS/30_DES_Requirements/Technical Functions/DES_TF_M_To_Generate_Time_Base | RBE/FCE |

## Design interface description Documents

This section presents all the documents that are linked to this software architecture design document.

Note: All links are related to S:\drive, to have them functional, please mount the S:\drive on your Audi Tr6 extended platform sandbox.

| Nb. | Document | Reference | Company |
| --- | --- | --- | --- |
|  | EEPROM parameters | SBE_4G_NVP_layout.xls | RBE/FCE |
|  | Design Interface description of AdcIf | N/A | RBE/FCE |
|  | Design Interface Description of Auto Tests Manager | ATM - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Belt Function Decision | N/A | RBE/FCE |
|  | Design Interface Description of Belt Function Execution | BFE - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Belt Function Selection | BFS - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Belt Movement Monitoring | N/A | RBE/FCE |
|  | Design Interface Description of Belt Parking Algorithm | N/A | RBE/FCE |
|  | Design Interface Description of Belt Slack Reduction | BSR - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Basic Software Manager | N/A | RBE/FCE |
|  | Design Interface Description of Basic Software Manager Interface | N/A | RBE/FCE |
|  | Design Interface Description of Can Tranceiver Interface | N/A | RBE/FCE |
|  | Design Interface Description of Communication Interaction Layer | CIL - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Diagnostic Communication Manager Interface | N/A | RBE/FCE |
|  | Design Interface Description of Diagnostic Event Manager Interface | N/A | RBE/FCE |
|  | Design Interface Description of DiagOnCAN services management | DIA - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Electronic Control Unit Manager | N/A | RBE/FCE |
|  | Design Interface Description of Electronic Control Unit Manager Interface | N/A | RBE/FCE |
|  | Design Interface Description of End of life | EOL - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Error Handler | ERH - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Haptic Warning | HWA - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Memory Integrity Control | N/A | RBE/FCE |
|  | Design Interface Description of Mode Management | MMG - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Network Management Interface | N/A | RBE/FCE |
|  | Design Interface Description of Non-Volatile Memory Interface | N/A | RBE/FCE |
|  | Design Interface Description of Non-Volatile Parameters | NVP - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Operating System Interface | N/A | RBE/FCE |
|  | Design Interface Description of Power Abstraction Layer | PAL - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Pre-Crash Master | N/A | RBE/FCE |
|  | Design Interface Description of Physical Measures Provider | PMP - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Port Interface | N/A | RBE/FCE |
|  | Design Interface Description of Pre-Tensioning | PRE - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Production cycle function | N/A | RBE/FCE |
|  | Design Interface Description of Pulse Width Modulation Interface | N/A | RBE/FCE |
|  | Design Interface Description of Reset Cause Management | N/A | RBE/FCE |
|  | Design Interface Description of SBC | N/A | RBE/FCE |
|  | Design Interface Description of System Context Management | N/A | RBE/FCE |
|  | Design Interface Description of Standard Function Recovery (releasing function) | SFR - Design Interface Description.docx | RBE/FCE |
|  | Design Interface Description of Serial Peripheral Interface Interface | N/A | RBE/FCE |
|  | Design Interface Description of Startup | N/A | RBE/FCE |
|  | Design Interface Description of System Time Management | N/A | RBE/FCE |
|  | Design Interface Description of Vehicle Dynamics algorithm | N/A | RBE/FCE |

## Design Specification Documents

This section presents all the documents that complete this software architecture design document.

Note: All links are related to S:\drive, to have them functional, please mount the S:\drive on your sandbox.

| Nb | Document | Reference | Company |
| --- | --- | --- | --- |
|  | Design document of AdcIf | N/A | RBE/FCE |
|  | Design document of Auto Tests Manager | ATM - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Belt Function Decision | N/A | RBE/FCE |
|  | Design document of Belt Function Execution | BFE - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Belt Function Selection | BFS - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Belt Movement Monitoring | N/A | RBE/FCE |
|  | Design document of Belt Parking Algorithm | N/A | RBE/FCE |
|  | Design document of Basic Software Manager Interface | N/A | RBE/FCE |
|  | Design document of Belt Slack Reduction | BSR - Detailed Design Document.docx |  |
|  | Design document of Communication Interaction Layer | CIL - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Diagnostic Communication Manager Interface | N/A | RBE/FCE |
|  | Design document of Diagnostic Event Manager Interface | N/A | RBE/FCE |
|  | Design document of DiagOnCAN services management | DIA - Detailed Design Document.docx | RBE/FCE |
|  | Design document of End of life | EOL - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Error Handler | ERH - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Haptic Warning | HWA - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Memory Integrity Control | N/A | RBE/FCE |
|  | Design document of Mode Management | MMG - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Network Management Interface | N/A | RBE/FCE |
|  | Design document of Non-Volatile Memory Interface | N/A | RBE/FCE |
|  | Design document of Non-Volatile Parameters | N/A | RBE/FCE |
|  | Design document of Power Abstraction Layer | PAL - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Physical Measures Provider | PMP - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Port Interface | N/A | RBE/FCE |
|  | Design document of Production cycle function | N/A | RBE/FCE |
|  | Design document of Reset Cause Management | N/A | RBE/FCE |
|  | Design document of RTE If | N/A | RBE/FCE |
|  | Design document of System Context Management | N/A | RBE/FCE |
|  | Design document of Standard Function Recovery (releasing function) | SFR - Detailed Design Document.docx | RBE/FCE |
|  | Design document of Serial Peripheral Interface Interface | N/A | RBE/FCE |

## Tier2 Documents

This section presents all the documents that complete this software architecture design document.

| Nb | Document | Reference | Company |
| --- | --- | --- | --- |

## HW Datasheet

This section presents all the documents related to the HW components that complete this software architecture design document.

| Nb | Document | Reference | Company |
| --- | --- | --- | --- |
|  | Infineon-TLE9471-3ES datasheet | TLE9461-3ES-Infineon.pdf | Infineon |

## Other Documents

This section presents all the documents that also have been needed to write this software architecture design document.

| Nb | Document | Reference | Company |
| --- | --- | --- | --- |
|  | Unified Modelling Language | 2.1.1 | OMG |
|  | MCU RFQ | E2581849 | FCE |

## Glossary And Definition

This section presents all the definitions and/or abbreviations used in this document.

List of terms in alphabetical order:

| Term | Meaning |
| --- | --- |
| ADC | Analog Digital Converter |
| AEC | Autoliv Error Code |
| API | Application Programming Interface |
| ASDM | Active Safety Domain Master |
| ASIC | Application Specific Integrated Circuit |
| ASY | Active SafetY |
| BSW | Basic SW modules |
| CAN | Controller Area Network |
| C/S | Chip Select |
| COP | Computer Operating Properly |
| eCPL | Electronic Crash Pole Locking |
| DART | Ditch - Airborne - Rough Terrain |
| DFLASH | Data FLASH |
| ECC | Error Code Correction |
| ECU | Electronic Control Unit |
| EOL | End Of Life |
| EEPROM | Electric Erasable and Programmable Read only Memory |
| HFPP | High Force Pre-Pre-Tensioning belt function |
| HF-PRE | High Force PRE pre-tensioning |
| HR | Hard Releasing |
| I/O | Input/Output |
| IMU | Inartial Measurements Unit |
| ISS | Integrated Safing System |
| LFPP | Low Force Pre-Pre-Tensioning belt function |
| MSA | Motor Start/Stop Automatic |
| MCAL | Micro-Controller Abstraction Layer |
| MCU | Micro-controller Unit |
| NMG | Mode ManaGement |
| NVM | Non Volatile Memory |
| OS | Operating System |
| PCM | Pre-Crash Master |
| PFLASH | Program FLASH |
| PIT | Periodic Interrupt Timer |
| PLL | Phase-locked loop |
| RAM | Random Access Memory |
| RCWM | Rear Collision Warning and Mitigation |
| RML | Left PP ECU |
| RMR | Right PP ECU |
| RMx | Both PP ECU |
| ROM | Read Only Memory |
| RSU | Remote Sensor Unit |
| RTE | Real Time Environment |
| RTOS | Real Time Operating System |
| SFR | Standard Function Recovery |
| SODL | Side Obstacle Detection Left |
| SPI | Serial Peripheral Interface |
| SRS | Supplementary Restraint System |
| TBC | To be confirmed |
| TBD | To be defined |
| TF | Technical Function |
| TFLASH | Test FLASH of the Pictus MCU (“one time programmable” memory) |
| W/D | Watchdog |

# Technical functions

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0000 | Service 0x22 will be implemented for DID F18C | DIA_runDidF18C_EcuSerialNumberRead; DIA_runDidF18C_EcuSerialNumberConditionCheckRead DIA_runDidF18C_EcuSerialNumberReadDataLength | DAI_EXT_TF_E_8099; DAI_EXT_TF_E_8025 |
| ARCH_SW_DIA_0001 | Service 0x22 will be implemented for DID 0340 | DIA_runDid0340_UniqueSeatbeltSerialNumberRead DIA_runDid0340_UniqueSeatbeltSerialNumberConditionCheckRead | DAI_EXT_TF_E_8062; DAI_EXT_TF_E_8063; DAI_EXT_TF_E_8064; DAI_EXT_TF_E_8101; DAI_EXT_TF_E_8102 |
| ARCH_SW_DIA_0008 | Service 0x2E will be implemented for DID 0340 | DIA_runDid0340_UniqueSeatbeltSerialNumberWrite | DAI_EXT_TF_E_8078 |
| ARCH_SW_DIA_0002 | Service 0x22 will be implemented for DID FD2F. | DIA_runDidFD2F_EcuLocationRead | DAI_EXT_TF_E_8885; DAI_EXT_TF_E_8888; DAI_EXT_TF_E_8889; DAI_EXT_TF_E_8893; DAI_EXT_TF_E_8894 |
| ARCH_SW_DIA_0004 | Service 0x22 will be implemented for DID FD10. | DIA_runDidFD10_CycleConfigurationRead | DAI_EXT_TF_E_8144; DAI_EXT_TF_E_8145 |
| ARCH_SW_DIA_0005 | Service 0x2E will be implemented for DID FD10. | DIA_runDidFD10_CycleConfigurationWrite | DAI_EXT_TF_E_8269; DAI_EXT_TF_E_8270 |
| ARCH_SW_DIA_0006 | Service 0x22 will be implemented for DID FD2A. | DIA_runDidFD2A_StepsLibraryRead | DAI_EXT_TF_E_8155; DAI_EXT_TF_E_8156 |
| ARCH_SW_DIA_0007 | Service 0x2E will be implemented for DID FD2A. | DIA_runDidFD2A_StepsLibraryWrite | DAI_EXT_TF_E_8307; DAI_EXT_TF_E_8308 |
| ARCH_SW_DIA_0009 | Service 0x2E will be implemented for DID FD03 | DIA_runDidFD03_AEETraceabilityNumberWrite |  |
| ARCH_SW_DIA_0010 | Service Read DTC Information shall be implemented with subfunction 0x02 ReportDTCByStatusMask. |  | ALV_EXT_TF_E_195; ALV_EXT_TF_E_197; ALV_EXT_TF_E_202; ALV_EXT_TF_E_203; ALV_EXT_TF_E_6118; ALV_EXT_TF_E_6119; ALV_EXT_TF_E_6120; ALV_EXT_TF_E_6121; ALV_EXT_TF_E_6122; ALV_EXT_TF_E_6123; ALV_EXT_TF_E_6124; ALV_EXT_TF_E_6125; ALV_EXT_TF_E_208; ALV_EXT_TF_E_209; ALV_EXT_TF_E_6217; ALV_EXT_TF_E_6218; ALV_EXT_TF_E_6219; ALV_EXT_TF_E_6220; ALV_EXT_TF_E_6221; ALV_EXT_TF_E_6222; ALV_EXT_TF_E_6223; ALV_EXT_TF_E_6224; ALV_EXT_TF_E_211 |
| ARCH_SW_DIA_0011 | Service 0x22 will be implemented for DID 0244 | DIA_runDid0244_PreSafeCounterRead DIA_runDid0244_PreSafeCounterConditionCheckRead DIA_runDid0244_PreSafeCounterReadDataLength | DAI_EXT_TF_E_8108; DAI_EXT_TF_E_8109; DAI_EXT_TF_E_10221; DAI_EXT_TF_E_10222 |
| ARCH_SW_DIA_0012 | Service 0x22 will be implemented for DID 0243 | DIA_runDid0243_BSRCounterRead DIA_runDid0243_BSRCounterConditionCheckRead | DAI_EXT_TF_E_8130; DAI_EXT_TF_E_8131 |
| ARCH_SW_DIA_0013 | Service 0x31 01 will be implemented for RID F703 | DIA_runRidF703_CycleExecutionStart | DAI_EXT_TF_E_8966; DAI_EXT_TF_E_8962; DAI_EXT_TF_E_8991; DAI_EXT_TF_E_8960; DAI_EXT_TF_E_8955; DAI_EXT_TF_E_8947; DAI_EXT_TF_E_8991; DAI_EXT_TF_E_8999 |
| ARCH_SW_DIA_0014 | Service 0x31 02 will be implemented for RID F703 | DIA_runRidF703_CycleExecutionStop | DAI_EXT_TF_E_8963; DAI_EXT_TF_E_8995; DAI_EXT_TF_E_8992; DAI_EXT_TF_E_8955; DAI_EXT_TF_E_8947; DAI_EXT_TF_E_8960; DAI_EXT_TF_E_8966 |
| ARCH_SW_DIA_0015 | Service 0x31 01 will be implemented for RID 0329 | DIA_runRid0329_TriggerBSRStart | DAI_EXT_TF_E_9300; DAI_EXT_TF_E_9301; DAI_EXT_TF_E_9303; DAI_EXT_TF_E_9304; DAI_EXT_TF_E_9305; |
| ARCH_SW_DIA_0016 | Service 0x31 02 will be implemented for RID 0329 | DIA_runRid0329_TriggerBSRStop | DAI_EXT_TF_E_9300; DAI_EXT_TF_E_9305; |
| ARCH_SW_DIA_0017 | Service 0x31 03 will be implemented for RID 0329 | DIA_runRid0329_TriggerBSRRequestResults | DAI_EXT_TF_E_9300; DAI_EXT_TF_E_9305; |
| ARCH_SW_DIA_0018 | Service 0x31 01 will be implemented for RID 0330 | DIA_runRid0330_TriggerTensioningCycleStop | DAI_EXT_TF_E_8505; DAI_EXT_TF_E_8514; DAI_EXT_TF_E_8519; DAI_EXT_TF_E_8520; DAI_EXT_TF_E_9297 |
| ARCH_SW_DIA_0019 | Service 0x31 02 will be implemented for RID 0330 | DIA_runRid0330_TriggerTensioningCycleStart | DAI_EXT_TF_E_8505; DAI_EXT_TF_E_9297 |
| ARCH_SW_DIA_0020 | Service 0x31 03 will be implemented for RID 0330 | DIA_runRid0330_TriggerTensioningCycleRequestResults | DAI_EXT_TF_E_8505 |
| ARCH_SW_DIA_0021 | Service 0x22 will be implemented for DID 0300 | DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_ReadData DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_ConditionCheckRead | DAI_EXT_TF_E_9335; DAI_EXT_TF_E_9336; DAI_EXT_TF_E_10527; DAI_EXT_TF_E_10528; DAI_EXT_TF_E_10529 |
| ARCH_SW_DIA_0022 | Service 0x2E will be implemented for DID 0300 | DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_WriteData | DAI_EXT_TF_E_9405; DAI_EXT_TF_E_9406; DAI_EXT_TF_E_9413; DAI_EXT_TF_E_10546; DAI_EXT_TF_E_10547; DAI_EXT_TF_E_10548 |
| ARCH_SW_DIA_0023 | Service 0x22 will be implemented for DID 0301 | DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ReadData DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ConditionCheckRead | DAI_EXT_TF_E_9391; DAI_EXT_TF_E_9392 |
| ARCH_SW_DIA_0024 | Service 0x2E will be implemented for DID 0301 | DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ReadData_WriteData | DAI_EXT_TF_E_9419; DAI_EXT_TF_E_9420 |
| ARCH_SW_DIA_0025 | Service 0x22 will be implemented for DID 0302 | DataServices_Tensioning_Cycle_BSR_CombinedSignal_ConditionCheckRead DataServices_Tensioning_Cycle_BSR_CombinedSignal_ReadData | DAI_EXT_TF_E_9377; DAI_EXT_TF_E_9378; DAI_EXT_TF_E_9385; DAI_EXT_TF_E_10536; DAI_EXT_TF_E_10540 |
| ARCH_SW_DIA_0026 | Service 0x2E will be implemented for DID 0302 | DataServices_Tensioning_Cycle_BSR_CombinedSignal_WriteData | DAI_EXT_TF_E_9432; DAI_EXT_TF_E_9433; DAI_EXT_TF_E_9440; DAI_EXT_TF_E_10549; DAI_EXT_TF_E_10550 |
| ARCH_SW_DIA_0027 | Service 0x22 will be implemented for DID 0310 | DataServices_Vehicle_Equipment_Read_Equipment_1_ConditionCheckRead DataServices_Vehicle_Equipment_Read_Equipment_2_ConditionCheckRead DataServices_Vehicle_Equipment_Read_Equipment_3_ConditionCheckRead DataServices_Vehicle_Equipment_Read_Equipment_4_ConditionCheckRead DataServices_Vehicle_Equipment_Read_Equipment_5_ConditionCheckRead DataServices_Vehicle_Equipment_Read_Equipment_6_ConditionCheckRead DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_ConditionCheckRead DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_ConditionCheckRead DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_ConditionCheckRead DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_ConditionCheckRead DataServices_Vehicle_Equipment_Read_Equipment_1_ReadData DataServices_Vehicle_Equipment_Read_Equipment_2_ReadData DataServices_Vehicle_Equipment_Read_Equipment_3_ReadData DataServices_Vehicle_Equipment_Read_Equipment_4_ReadData DataServices_Vehicle_Equipment_Read_Equipment_5_ReadData DataServices_Vehicle_Equipment_Read_Equipment_6_ReadData DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_ReadData DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_ReadData DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_ReadData DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_ReadData | DAI_EXT_TF_E_9363; DAI_EXT_TF_E_9364; DAI_EXT_TF_E_10223 |
| ARCH_SW_DIA_0045 | There shall be 2 reserved bytes located between Equipment 6 and Uberspannungsabbruchschwelle for response of DID 0310 |  | DAI_EXT_TF_E_9508; DAI_EXT_TF_E_9509 |
| ARCH_SW_DIA_0028 | Service 0x2E will be implemented for DID 0310 | DataServices_Vehicle_Equipment_Read_Equipment_1_WriteData DataServices_Vehicle_Equipment_Read_Equipment_2_WriteData DataServices_Vehicle_Equipment_Read_Equipment_3_WriteData DataServices_Vehicle_Equipment_Read_Equipment_4_WriteData DataServices_Vehicle_Equipment_Read_Equipment_5_WriteData DataServices_Vehicle_Equipment_Read_Equipment_6_WriteData DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_WriteData DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_WriteData DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_WriteData DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_WriteData | DAI_EXT_TF_E_9444; DAI_EXT_TF_E_9445; DAI_EXT_TF_E_10253 |
| ARCH_SW_DIA_0046 | There shall be 2 reserved bytes located between Equipment 6 and Uberspannungsabbruchschwelle for request write of DID 0310 |  | DAI_EXT_TF_E_9477; DAI_EXT_TF_E_9478 |
| ARCH_SW_DIA_0029 | Service 0x22 will be implemented for DID 0311 | DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_ConditionCheckRead DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_ConditionCheckRead DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_ReadData DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_ReadData | DAI_EXT_TF_E_9349; DAI_EXT_TF_E_9350 |
| ARCH_SW_DIA_0030 | Service 0x2E will be implemented for DID 0311 | DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_WriteData DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_WriteData | DAI_EXT_TF_E_9455; DAI_EXT_TF_E_9456; DAI_EXT_TF_E_10255; DAI_EXT_TF_E_9463; DAI_EXT_TF_E_10254; DAI_EXT_TF_E_10255 |
| ARCH_SW_DIA_0031 | Service 0x22 will be implemented for DID FD08 | DIA_runDidFD08_ResetCauseManagementRead DIA_runDidFD08_ResetCauseManagementConditionCheckRead | DAI_EXT_TF_E_8661; DAI_EXT_TF_E_8664; DAI_EXT_TF_E_8665; DAI_EXT_TF_E_8668; DAI_EXT_TF_E_8669 |
| ARCH_SW_DIA_0032 | Service 0x22 will be implemented for DID FD09 | DIA_runDidFD09_LastWarmResetRead_LastDetectedWarmResetCause DIA_runDidFD09_LastWarmResetRead_ResetCauseCounter DIA_runDidFD09_LastWarmReseConditionChecktRead_LastDetectedWarmResetCause DIA_runDidFD09_LastWarmResetConditionCheckRead_ResetCauseCounter | DAI_EXT_TF_E_8633; DAI_EXT_TF_E_8634; DAI_EXT_TF_E_8630; DAI_EXT_TF_E_8638; DAI_EXT_TF_E_8639 |
| ARCH_SW_DIA_0033 | Service 0x22 will be implemented for DID FD31 | DIA_runDidFD31_HBCalibrationRead DIA_runDidFD31_HBCalibrationWriteConditionCheckRead | DAI_EXT_TF_E_8833; DAI_EXT_TF_E_8834; DAI_EXT_TF_E_8830; DAI_EXT_TF_E_8838; DAI_EXT_TF_E_8839 |
| ARCH_SW_DIA_0035 | Service 0x22 will be implemented for DID FE00 | DIA_runDidFE00_AECStatusRead DIA_runDidFE00_AECStatusReadDataLength DIA_runDidFE00_AECStatusConditionCheckRead | DAI_EXT_TF_E_8698; DAI_EXT_TF_E_8699; DAI_EXT_TF_E_8695; DAI_EXT_TF_E_8703; DAI_EXT_TF_E_8704 |
| ARCH_SW_DIA_0036 | Service 0x22 will be implemented for DID FEFF | DIA_runDidFEFF_MeasurementFrameConfigurationRead DIA_runDidFEFF_MeasurementFrameConfigurationConditionCheckRead | DAI_EXT_TF_E_8718; DAI_EXT_TF_E_8719; DAI_EXT_TF_E_8715; DAI_EXT_TF_E_8723; DAI_EXT_TF_E_8724 |
| ARCH_SW_DIA_0037 | Service 0x22 will be implemented for DID FDFF | DIA_runDidFDFF_MeasurementFrameBlock12ConfigRead DIA_runDidFDFF_MeasurementFrameBlock12ConfigConditionCheckRead | DAI_EXT_TF_E_8737; DAI_EXT_TF_E_8738; DAI_EXT_TF_E_8734; DAI_EXT_TF_E_8742; DAI_EXT_TF_E_8743; DAI_EXT_TF_E_8744 |
| ARCH_SW_DIA_0038 | Service 0x2E will be implemented for DID FEFF | DIA_runDidFEFF_MeasurementFrameConfigurationWrite |  |
| ARCH_SW_DIA_0039 | Service 0x2E will be implemented for DID FDFF | DIA_runDidFDFF_MeasurementFrameBlock12ConfigWrite | DAI_EXT_TF_E_9085; DAI_EXT_TF_E_9086; DAI_EXT_TF_E_9082; DAI_EXT_TF_E_9090; DAI_EXT_TF_E_9091; DAI_EXT_TF_E_9100; DAI_EXT_TF_E_9101; DAI_EXT_TF_E_9102 |
| ARCH_SW_DIA_0040 | Service 0x23 will be implemented | DIA_runSid23_ReadMemoryByAddress | DAI_EXT_TF_E_8922; DAI_EXT_TF_E_8919; DAI_EXT_TF_E_8927 |
| ARCH_SW_DIA_0041 | Service 0x3D will be implemented | DIA_runSid3D_WriteMemoryByAddress | DAI_EXT_TF_E_9209; DAI_EXT_TF_E_9206; DAI_EXT_TF_E_9214 |
| ARCH_SW_DIA_0042 | Service 10 shall support the following sessions: Default, Programming, Extended and Development Session |  | DAI_EXT_TF_E_25; DAI_EXT_TF_E_26; DAI_EXT_TF_E_27; DAI_EXT_TF_E_28 |
| ARCH_SW_DIA_0043 | Service 10 shall be available in the following sessions: Default, Programming, Extended, Development Session and DefaultSession (of the Bootloader) |  | DAI_EXT_TF_E_34; DAI_EXT_TF_E_35; DAI_EXT_TF_E_36; DAI_EXT_TF_E_37; DAI_EXT_TF_E_5997; DAI_EXT_TF_E_5998 |
| ARCH_SW_DIA_0044 | Transition to Development Session will be done only with authentication |  | DAI_EXT_TF_E_9227 |
| ARCH_SW_DIA_0220 | Service 0x31 (Routine Control) 0x31 01 0x31 02 0x31 03 for RID 0x0302 (Trigger Pre-Safe Display Request) will be implemented | DIA_runRid0302_TriggerPresafeDisplayRequestRequestResults DIA_runRid0302_TriggerPresafeDisplayRequestStart DIA_runRid0302_TriggerPresafeDisplayRequestStop | DAI_EXT_TF_E_9542; DAI_EXT_TF_E_9543; DAI_EXT_TF_E_9544; DAI_EXT_TF_E_9545; DAI_EXT_TF_E_9546; DAI_EXT_TF_E_9547; DAI_EXT_TF_E_9548; DAI_EXT_TF_E_9550; DAI_EXT_TF_E_9910; DAI_EXT_TF_E_9565; DAI_EXT_TF_E_9566; DAI_EXT_TF_E_9567; DAI_EXT_TF_E_9568; DAI_EXT_TF_E_9570; DAI_EXT_TF_E_9571; DAI_EXT_TF_E_9572; DAI_EXT_TF_E_9573 |
| ARCH_SW_DIA_0235 | Service 0x22 (Read Data By Identifier) will be implemented for DID 0x0142 (Availability Data) | DIA_runDid0142_AvailabilityDataRead | DAI_EXT_TF_E_9736; DAI_EXT_TF_E_9745; DAI_EXT_TF_E_9945; DAI_EXT_TF_E_9760; DAI_EXT_TF_E_9761; DAI_EXT_TF_E_10085 |
| ARCH_SW_DIA_0240 | Service 0x22 (Read Data By Identifier) & 0x2E (Write Data By Identifier) for DID 0x0241 will be implemented | DIA_runDid0241_PRE_SAFE_Recorder_1_ReadDataLength DIA_runDid0241_PRE_SAFE_Recorder_1_ConditionCheckRead DIA_runDid0241_PRE_SAFE_Recorder_1_ReadData DIA_runDid0241_PRE_SAFE_Recorder_1_WriteData | DAI_EXT_TF_E_9738; DAI_EXT_TF_E_9906; DAI_EXT_TF_E_9747; DAI_EXT_TF_E_9947; DAI_EXT_TF_E_9838; DAI_EXT_TF_E_11509; DAI_EXT_TF_E_11510 |
| ARCH_SW_DIA_0250 | Service 0x22 (Read Data By Identifier) & 0x2E (Write Data By Identifier) for DID 0x0242 will be implemented | DIA_runDid0242_PRE_SAFE_Recorder_2_ReadDataLength DIA_runDid0242_PRE_SAFE_Recorder_2_ConditionCheckRead DIA_runDid0242_PRE_SAFE_Recorder_2_ReadData DIA_runDid0242_PRE_SAFE_Recorder_2_WriteData | DAI_EXT_TF_E_9739; DAI_EXT_TF_E_9905; DAI_EXT_TF_E_9748; DAI_EXT_TF_E_9948; DAI_EXT_TF_E_9871; DAI_EXT_TF_E_11512; DAI_EXT_TF_E_11513 |
| ARCH_SW_DIA_0210 | ROE on Dtc Status change implemented | RoELight_DTCStatusChanged_Callback | DAI_EXT_TF_B_2051; DAI_EXT_TF_B_2052; DAI_EXT_TF_B_2053; DAI_EXT_TF_B_2074; DAI_EXT_TF_B_2116; DAI_EXT_TF_B_2117; DAI_EXT_TF_B_2118; DAI_EXT_TF_B_2119; DAI_EXT_TF_B_2120; DAI_EXT_TF_B_2176; DAI_EXT_TF_B_2121; DAI_EXT_TF_B_2122; DAI_EXT_TF_B_2123; DAI_EXT_TF_B_2124; DAI_EXT_TF_B_2125; DAI_EXT_TF_B_2076; DAI_EXT_TF_B_2058 |
| ARCH_SW_DIA_0260 | Clear Diag Request Cycle Function | DIA_runClearDiagRequestCycle |  |
| ARCH_SW_DIA_0261 | Get Diag Request Cycle Function | DIA_runGetDiagRequestCycle |  |
| ARCH_SW_DIA_0262 | Cyclic processing task of the Tensioning Routines | DIA_cyclicProcessingTask |  |
| ARCH_SW_DIA_0265 | Service 0x11 (ECU Reset) will be implemented | ResetServiceExecution | ALV_EXT_TF_E_7163; ALV_EXT_TF_E_6147; ALV_EXT_TF_E_6015; ALV_EXT_TF_E_7410; ALV_EXT_TF_E_104; ALV_EXT_TF_E_114 |
| ARCH_SW_DIA_0275 | Service 0x14 (Clear Diagnostic Information) will be implemented | Dcm_Svc14Handler | ALV_EXT_TF_E_5931; ALV_EXT_TF_E_124; ALV_EXT_TF_E_6037; ALV_EXT_TF_E_6038; ALV_EXT_TF_E_6039; ALV_EXT_TF_E_6040; ALV_EXT_TF_E_6041; ALV_EXT_TF_E_6042; ALV_EXT_TF_E_6043; ALV_EXT_TF_E_139; ALV_EXT_TF_E_140 |
| ARCH_SW_DIA_0281 | Service 0x19 (Read DTC Information) will be implemented for ReportDTCByStatusMask (RDTCBST – 0x02) | Dcm_Svc19Handler | DAI_EXT_TF_E_198; ALV_EXT_TF_E_207; DAI_EXT_TF_E_10103 |
| ARCH_SW_DIA_0285 | Service 0x22 (Read Data By Identifier) will be implemented for DID FD01 (SW Version) | DIA_runDidFD01_SWVersionConditionCheckRead DIA_runDidFD01_SWVersionRead | ALV_EXT_TF_E_652; ALV_EXT_TF_E_6260; ALV_EXT_TF_E_655; ALV_EXT_TF_E_661; ALV_EXT_TF_E_7065; DAI_EXT_TF_E_10285 |
| ARCH_SW_DIA_0295 | Service 0x27 (Security Access) will be implemented for Get Seed and Send Key | Dcm_Service27Processor | ALV_EXT_TF_E_1467; DAI_EXT_TF_E_10256; ALV_EXT_TF_E_6033; ALV_EXT_TF_E_6055; DAI_EXT_TF_E_10286; ALV_EXT_TF_E_7618; ALV_EXT_TF_E_7619; ALV_EXT_TF_E_7627 |
| ARCH_SW_DIA_0305 | Service 0x2E (Write Data By Identifier) will be implemented for DID FD2F (ECU Location) | DIA_runDidFD2F_EcuLocationWrite | DAI_EXT_TF_E_10086; DAI_EXT_TF_E_10087; DAI_EXT_TF_E_10089; DAI_EXT_TF_E_10090; ALV_EXT_TF_E_6735; DAI_EXT_TF_E_10091 |
| ARCH_SW_DIA_0315 | Service 0x2E (Write Data By Identifier) will be implemented for DID FD31 (ECU Calibration Parameters) | DIA_runDidFD31_HBCalibrationWrite | ALV_EXT_TF_E_1833; ALV_EXT_TF_E_10092; ALV_EXT_TF_E_10093; ALV_EXT_TF_E_10995; ALV_EXT_TF_E_10996; ALV_EXT_TF_E_1853 |
| ARCH_SW_DIA_0325 | Service 0x22 (Read Data By Identifier) will be implemented for DID 0133 (Current Operating Time) | DataServices_Current_Operating_Time_Read_Current_Operating_Time_ConditionCheckRead DataServices_Current_Operating_Time_Read_Current_Operating_Time_ReadData | DAI_EXT_TF_E_9735; DAI_EXT_TF_E_9744; DAI_EXT_TF_E_9944; DAI_EXT_TF_E_9757; DAI_EXT_TF_E_9758; DAI_EXT_TF_E_9759 |
| ARCH_SW_DIA_0330 | Service 0x22 (Read Data By Identifier) will be implemented for DID 0140 (Obsolescence Data) | DIA_runDid00140_Obsolescence_Data_BSR_Current_Value_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_BSR_Current_Value_Read DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Green_To_Yellow_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Green_To_Yellow_Read DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Yellow_To_Red_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Yellow_To_Red_Read DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Current_Value_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Current_Value_Read DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Green_To_Yellow_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Green_To_Yellow_Read DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Yellow_To_Red_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_ Yellow_To_Red _Read DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Current_Value_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Current_Value_Read DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Green_To_Yellow_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Green_To_Yellow_Read DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Yellow_To_Red_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Yellow_To_Red_Read DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Current_Value_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Current_Value_Read DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Green_To_Yellow_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Green_To_Yellow_Read DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Yellow_To_Red_ConditionCheckRead DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Yellow_To_Red_Read | DAI_EXT_TF_E_10179; DAI_EXT_TF_E_10180; DAI_EXT_TF_E_10184; DAI_EXT_TF_E_10187; DAI_EXT_TF_E_10189; DAI_EXT_TF_E_10190; DAI_EXT_TF_E_10191; DAI_EXT_TF_E_10194; DAI_EXT_TF_E_10195; DAI_EXT_TF_E_10193 |
| ARCH_SW_DIA_0340 | Service 0x22 (Read Data By Identifier) will be implemented for DID 01E1 (CPU Load) | DataServices_CPU_Load_Read_CPU_load_histograms_ConditionCheckRead DataServices_CPU_Load_Read_CPU_load_histograms_ReadDataLength DataServices_CPU_Load_Read_CPU_load_histograms_ReadData | DAI_EXT_TF_E_10107; DAI_EXT_TF_E_10108; DAI_EXT_TF_E_10112; DAI_EXT_TF_E_10115; DAI_EXT_TF_E_10117; DAI_EXT_TF_E_10118; DAI_EXT_TF_E_10119 |
| ARCH_SW_DIA_0345 | Service 0x22 (Read Data By Identifier) will be implemented for DID 01E2 (E2E Timing) | DataServices_E2E_Timing_Read_Maximum_ConditionCheckRead DataServices_E2E_Timing_Read_Maximum_ReadData DataServices_E2E_Timing_Read_Maximum_ReadDataLength | DAI_EXT_TF_E_10123; DAI_EXT_TF_E_10124; DAI_EXT_TF_E_10128; DAI_EXT_TF_E_10131; DAI_EXT_TF_E_10133; DAI_EXT_TF_E_10134; DAI_EXT_TF_E_10135 |
| ARCH_SW_DIA_0350 | Service 0x22 (Read Data By Identifier) will be implemented for DID 01E3 (Stack Load) | DataServices_Stack_Load_Read_Maximum_ConditionCheckRead DataServices_Stack_Load_Read_Maximum_ReadDataLength DataServices_Stack_Load_Read_Maximum_ReadData | DAI_EXT_TF_E_10139; DAI_EXT_TF_E_10140; DAI_EXT_TF_E_10144; DAI_EXT_TF_E_10147; DAI_EXT_TF_E_10149; DAI_EXT_TF_E_10150; DAI_EXT_TF_E_10151; DAI_EXT_TF_E_10274; DAI_EXT_TF_E_10277 |
| ARCH_SW_DIA_0355 | Service 0x22 (Read Data By Identifier) will be implemented for DID 01E4 (Memory Usage Cycles) | DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ConditionCheckRead DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ReadDataLength DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ReadData | DAI_EXT_TF_E_10155; DAI_EXT_TF_E_10156; DAI_EXT_TF_E_10160; DAI_EXT_TF_E_10163; DAI_EXT_TF_E_10281; DAI_EXT_TF_E_10282; DAI_EXT_TF_E_10283; DAI_EXT_TF_E_10279 |
| ARCH_SW_DIA_0360 | Service 0x22 (Read Data By Identifier) will be implemented for DID 0200 (CAN Input) | DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_1_ConditionCheckRead DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_1_ReadData DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_2_ConditionCheckRead DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_2_ReadData DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_3_ConditionCheckRead DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_3_ReadData DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_4_ConditionCheckRead DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_4_ReadData DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_5_ConditionCheckRead DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_5_ReadData DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_6_ConditionCheckRead DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_6_ReadData DIA_runDid0200_CAN_input_Read_Supply_battery_voltage_ConditionCheckRead DIA_runDid0200_CAN_input_Read_Supply_battery_voltage_ReadData DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_7_ConditionCheckRead DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_7_ReadData DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_8_ConditionCheckRead DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_8_ReadData | DAI_EXT_TF_E_10200; DAI_EXT_TF_E_10201; DAI_EXT_TF_E_10205; DAI_EXT_TF_E_10208; DAI_EXT_TF_E_10210; DAI_EXT_TF_E_10211; DAI_EXT_TF_E_10212; DAI_EXT_TF_E_10213; DAI_EXT_TF_E_10214; DAI_EXT_TF_E_10217; DAI_EXT_TF_E_10218; DAI_EXT_TF_E_10219; DAI_EXT_TF_E_10220; DAI_EXT_TF_E_10216; DAI_EXT_TF_B_2303; DAI_EXT_TF_B_2304; DAI_EXT_TF_B_2305; DAI_EXT_TF_B_2319; DAI_EXT_TF_B_2320; DAI_EXT_TF_B_2321; DAI_EXT_TF_B_2330; DAI_EXT_TF_B_2331; DAI_EXT_TF_B_2332; DAI_EXT_TF_B_2335; DAI_EXT_TF_B_2336; DAI_EXT_TF_B_2337; DAI_EXT_TF_B_2340; DAI_EXT_TF_B_2341; DAI_EXT_TF_B_2342; DAI_EXT_TF_B_2345; DAI_EXT_TF_B_2346; DAI_EXT_TF_B_2347; DAI_EXT_TF_B_2313; DAI_EXT_TF_B_2324; DAI_EXT_TF_B_2279; DAI_EXT_TF_B_2288; DAI_EXT_TF_B_2281; DAI_EXT_TF_B_2289; DAI_EXT_TF_B_2284; DAI_EXT_TF_B_2286; DAI_EXT_TF_B_2285; DAI_EXT_TF_B_2287; DAI_EXT_TF_B_2365; DAI_EXT_TF_B_2366; DAI_EXT_TF_B_2367; DAI_EXT_TF_B_2368; DAI_EXT_TF_B_2369; DAI_EXT_TF_B_2370; DAI_EXT_TF_B_2371; DAI_EXT_TF_B_2372; DAI_EXT_TF_B_2427; DAI_EXT_TF_B_2428; DAI_EXT_TF_B_2433; |
| ARCH_SW_DIA_0370 | Service 0x22 (Read Data By Identifier) will be implemented for DID F150 (Hardware Version Information) | DataServices_HardwareVersion_Read_HW_patch_level_ConditionCheckRead DataServices_HardwareVersion_Read_HW_patch_level_ReadData DataServices_HardwareVersion_Read_HW_week_ConditionCheckRead DataServices_HardwareVersion_Read_HW_week_ReadData DataServices_HardwareVersion_Read_HW_year_ConditionCheckRead DataServices_HardwareVersion_Read_HW_year_ReadData | DAI_EXT_TF_E_9733; DAI_EXT_TF_E_9749; DAI_EXT_TF_E_9942; DAI_EXT_TF_E_9830; DAI_EXT_TF_E_9831; DAI_EXT_TF_E_9832; DAI_EXT_TF_E_9833 |
| ARCH_SW_DIA_0375 | Service 0x22 (Read Data By Identifier) will be implemented for DID F151 (Software Version Information) | DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ConditionCheckRead DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ReadDataLength DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ReadData | DAI_EXT_TF_E_9732; DAI_EXT_TF_E_9750; DAI_EXT_TF_E_9943; DAI_EXT_TF_E_10267; DAI_EXT_TF_E_10268; DAI_EXT_TF_E_10271; DAI_EXT_TF_E_11467 |
| ARCH_SW_DIA_0380 | Service 0x22 (Read Data By Identifier) will be implemented for DID F153 (Boot Software Version Information) | DataServices_BootSoftwareVersion_Read_Boot_SW_patch_level_ConditionCheckRead DataServices_BootSoftwareVersion_Read_Boot_SW_patch_level_ReadData DataServices_BootSoftwareVersion_Read_Boot_SW_week_ConditionCheckRead DataServices_BootSoftwareVersion_Read_Boot_SW_week_ReadData DataServices_BootSoftwareVersion_Read_Boot_SW_year_ConditionCheckRead DataServices_BootSoftwareVersion_Read_Boot_SW_year_ReadData | DAI_EXT_TF_E_9953; DAI_EXT_TF_E_9956; DAI_EXT_TF_E_9959; DAI_EXT_TF_E_9960; DAI_EXT_TF_E_9961; DAI_EXT_TF_E_9962; DAI_EXT_TF_E_9963 |
| ARCH_SW_DIA_0385 | Service 0x2E (Write Data By Identifier) will be implemented for DID 0241 (Reset Pre-Safe Recorder 1) | DIA_runDid0241_PRE_SAFE_Recorder_1_WriteData | DAI_EXT_TF_E_9740; DAI_EXT_TF_E_9951; DAI_EXT_TF_E_9754; DAI_EXT_TF_E_9922; DAI_EXT_TF_E_9768; DAI_EXT_TF_E_9769; DAI_EXT_TF_E_10320; DAI_EXT_TF_E_10321 |
| ARCH_SW_DIA_0390 | Service 0x2E (Write Data By Identifier) will be implemented for DID 0242 (Reset Pre-Safe Recorder 2) | DIA_runDid0242_PRE_SAFE_Recorder_2_WriteData | DAI_EXT_TF_E_9741; DAI_EXT_TF_E_9950; DAI_EXT_TF_E_9752; DAI_EXT_TF_E_9920; DAI_EXT_TF_E_9770; DAI_EXT_TF_E_9771; DAI_EXT_TF_E_10322; DAI_EXT_TF_E_10323 |
| ARCH_SW_DIA_0395 | Service 0x2E (Write Data By Identifier) will be implemented for DID 0244 (Reset Pre-Safe Counter) | DataServices_PRE_SAFE_Counter_read_Read_CombinedDataElement_WriteData | DAI_EXT_TF_E_9772; DAI_EXT_TF_E_9773; DAI_EXT_TF_E_10318; DAI_EXT_TF_E_10319 |
| ARCH_SW_DIA_0405 | Service 0x28 (Communication Control) will be implemented | Dcm_Svc28Handler | DAI_EXT_TF_E_10228; DAI_EXT_TF_E_10229; DAI_EXT_TF_E_10233; DAI_EXT_TF_E_10249; DAI_EXT_TF_E_10252; DAI_EXT_TF_E_10238; DAI_EXT_TF_E_10248 |
| ARCH_SW_DIA_0415 | Service 0x85 (ControlDTCSetting Service) will be implemented | Dcm_Svc85Handler | DAI_EXT_TF_E_9971; DAI_EXT_TF_E_8092; DAI_EXT_TF_E_9974; DAI_EXT_TF_E_9975; DAI_EXT_TF_E_9976; DAI_EXT_TF_E_9980; DAI_EXT_TF_E_9981; DAI_EXT_TF_E_9982; DAI_EXT_TF_E_9995; DAI_EXT_TF_E_9996; DAI_EXT_TF_E_9997; DAI_EXT_TF_E_9984; DAI_EXT_TF_E_9987; DAI_EXT_TF_E_9985; DAI_EXT_TF_E_9988; DAI_EXT_TF_E_9986; DAI_EXT_TF_E_9989; DAI_EXT_TF_E_9990; DAI_EXT_TF_E_9991; DAI_EXT_TF_E_9992; DAI_EXT_TF_E_9993 |
| ARCH_SW_DIA_0420 | RoELite for ObsolescenceData will be implemented | RoELite_ObsolescenceDataHandler | DAI_EXT_TF_B_2074; DAI_EXT_TF_B_2120; DAI_EXT_TF_B_2296; DAI_EXT_TF_B_2353; DAI_EXT_TF_B_2354; DAI_EXT_TF_B_2356 |
| ARCH_SW_DIA_0430 | RoELite for AvailabilityData will be implemented | RoELite_AvailabilityDataHandler | DAI_EXT_TF_B_2082; DAI_EXT_TF_B_2355; DAI_EXT_TF_B_2126 |
| ARCH_SW_DIA_0435 | RoELite for DiagSessionChange will be implemented | DiagSessionChangeCallback | DAI_EXT_TF_B_2083; DAI_EXT_TF_B_2128; DAI_EXT_TF_B_2129 |
| ARCH_SW_DIA_0460 | Service 0x22 (Read Data By Identifier) will be implemented for DID 0107 (Global Time Sync Measured Values) | DataServices_Global_Time_Sync_Measured_Values_ConditionCheckRead DataServices_Global_Time_Sync_Measured_Values_ReadData DataServices_Global_Time_Sync_Measured_Values_ReadDataLength | DAI_EXT_TF_E_9734; DAI_EXT_TF_E_9743; DAI_EXT_TF_E_9941; DAI_EXT_TF_E_9774; DAI_EXT_TF_E_9775; DAI_EXT_TF_E_9776 |
| ARCH_SW_DIA_0510 | Service 0x22 (Read Data By Identifier) will be implemented for DID 0210 (CAN Output) | DIA_runDid0210_CAN_output_Read_CANoutput1_ConditionCheckRead DIA_runDid0210_CAN_output_Read_CANoutput1_ReadData DIA_runDid0210_CAN_output_Read_PS_Curve_XX_Stat_ST3_ConditionCheckRead DIA_runDid0210_CAN_output_Read_PS_Curve_XX_Stat_ST3_ReadData DIA_runDid0210_CAN_output_Read_RBTMxx_Actv_Lvl_ST35_ConditionCheckRead DIA_runDid0210_CAN_output_Read_RBTMxx_Actv_Lvl_ST35_ReadData DIA_runDid0210_CAN_output_Read_RBTMxx_SP_ActvClient_ST35_ConditionCheckRead DIA_runDid0210_CAN_output_Read_RBTMxx_SP_ActvClient_ST35_ReadData DIA_runDid0210_CAN_output_Read_RBTMxx_SP_FctPrecond_Stat_ST35_ConditionCheckRead DIA_runDid0210_CAN_output_Read_RBTMxx_SP_FctPrecond_Stat_ST35_ReadData | DAI_EXT_TF_E_10310; DAI_EXT_TF_E_10311; DAI_EXT_TF_E_10315; DAI_EXT_TF_E_10461; DAI_EXT_TF_E_10463; DAI_EXT_TF_E_10464; DAI_EXT_TF_E_10472; DAI_EXT_TF_E_10473; DAI_EXT_TF_E_10474; DAI_EXT_TF_E_10475; DAI_EXT_TF_E_10477; DAI_EXT_TF_B_2310; DAI_EXT_TF_B_2377; DAI_EXT_TF_B_2378; DAI_EXT_TF_B_2379; DAI_EXT_TF_B_2380; DAI_EXT_TF_B_2381; DAI_EXT_TF_B_2382; DAI_EXT_TF_B_2383; DAI_EXT_TF_B_2384 |
| ARCH_SW_DIA_0515 | Service 0x2E (Write Data By Identifier) will be implemented for DID FD50 (HW Version) | DIA_runDidFD50_Hardware_Version_Information_Write_HW_year DIA_runDidFD50_Hardware_Version_Information_Write_HW_week DIA_runDidFD50_Hardware_Version_Information_Write_HW_patch_level | DAI_EXT_TF_E_10293; DAI_EXT_TF_E_10541; DAI_EXT_TF_E_10337; DAI_EXT_TF_E_10339; DAI_EXT_TF_E_10349; DAI_EXT_TF_E_10544 |
| ARCH_SW_DIA_0525 | Service 0x19 (Read DTC Information) will be implemented for ReportNumberOfDTCByStatusMask (RNODTCBSM – 0x01) | Dcm_Svc19Handler | ALV_EXT_TF_E_5927; ALV_EXT_TF_E_173; ALV_EXT_TF_E_6091; ALV_EXT_TF_E_176; ALV_EXT_TF_E_179; ALV_EXT_TF_E_6100; ALV_EXT_TF_E_182; ALV_EXT_TF_E_6110; ALV_EXT_TF_E_184 |
| ARCH_SW_DIA_0530 | Service 0x19 (Read DTC Information) will be implemented for ReportSupportedDTC (RSDTC) - 0x0A | Dcm_Svc19Handler | ALV_EXT_TF_E_5933; ALV_EXT_TF_E_324; ALV_EXT_TF_E_6151; ALV_EXT_TF_E_329; ALV_EXT_TF_E_6152; ALV_EXT_TF_E_332; ALV_EXT_TF_E_6153 |
| ARCH_SW_DIA_0555 | Service 0x22 (Read Data By Identifier) will be implemented for DID 0220 (HW Input) | DIA_runDid0220_HW_input_Read_Voltage_KL30t_ConditionCheckRead DIA_runDid0220_HW_input_Read_Voltage_KL30t_ReadData DIA_runDid0220_HW_input_Read_Voltage_Motor_ConditionCheckRead DIA_runDid0220_HW_input_Read_Voltage_Motor_ReadData | DAI_EXT_TF_E_10561; DAI_EXT_TF_E_10562; DAI_EXT_TF_E_10568; DAI_EXT_TF_E_10571; DAI_EXT_TF_E_10573; DAI_EXT_TF_E_10574; DAI_EXT_TF_E_10575; DAI_EXT_TF_E_10577 |
| ARCH_SW_DIA_0565 | Service 0x22 (Read Data By Identifier) will be implemented for DID 0240 (Internal ECU Information) | DIA_runDid0240_Internal_ECU_information_Read_Autosar_System_State_EcuM_State_ConditionCheckRead DIA_runDid0240_Internal_ECU_information_Read_Autosar_System_State_EcuM_State_ReadData DIA_runDid0240_Internal_ECU_information_Read_ECU_Temperature_ConditionCheckRead DIA_runDid0240_Internal_ECU_information_Read_ECU_Temperature_ReadData DIA_runDid0240_Internal_ECU_information_Read_MOTOR_Current_ConditionCheckRead DIA_runDid0240_Internal_ECU_information_Read_MOTOR_Current_ReadData | DAI_EXT_TF_E_10582; DAI_EXT_TF_E_10583; DAI_EXT_TF_E_10587; DAI_EXT_TF_E_10590; DAI_EXT_TF_E_10592; DAI_EXT_TF_E_10593; DAI_EXT_TF_E_10594; DAI_EXT_TF_E_10595; DAI_EXT_TF_E_10597 |
| ARCH_SW_DIA_0575 | Service 0x22 (Read Data By Identifier) will be implemented for DID FD03 (AEE Traceability Number And PCBA Programmed PN) | DIA_runDidFD03_ECUTraceablityNumberConditionCheckRead DIA_runDidFD03_ECUTraceablityNumberRead DIA_runDidFD03_PCBAProgrammedPartNumberConditionCheckRead DIA_runDidFD03_PCBAProgrammedPartNumberRead | DAI_EXT_TF_E_8900; DAI_EXT_TF_E_8901; DAI_EXT_TF_E_8905; DAI_EXT_TF_E_11342; DAI_EXT_TF_E_8910; DAI_EXT_TF_E_8912; DAI_EXT_TF_E_11343 |
| ARCH_SW_DIA_0585 | Service 0x2E (Write Data By Identifier) will be implemented for DID FD03 (AEE Traceability Number And PCBA Programmed PN) | DIA_runDidFD03_ECUTraceablityNumberWrite DIA_runDidFD03_PCBAProgrammedPartNumberWrite | DAI_EXT_TF_E_11352; DAI_EXT_TF_E_11353; DAI_EXT_TF_E_11357; DAI_EXT_TF_E_11356; DAI_EXT_TF_E_11358; DAI_EXT_TF_E_11359; DAI_EXT_TF_E_11350; DAI_EXT_TF_E_11351 |
| ARCH_SW_DIA_0595 | Service 0x22 (Read Data By Identifier) will be implemented for DID 0100 (Reprogramming Attempt Counter) | DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ConditionCheckRead DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ReadData DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ReadDataLength | DAI_EXT_TF_E_10746; DAI_EXT_TF_E_11217; DAI_EXT_TF_E_10757; DAI_EXT_TF_E_10759; DAI_EXT_TF_E_11221; DAI_EXT_TF_E_11223; DAI_EXT_TF_E_11362 |
| ARCH_SW_DIA_0605 | Service 0x22 (Read Data By Identifier) will be implemented for DID 010A (Vehicle Odometer in Low Resolution) | DataServices_Vehicle_Odometer_in_Low_Resolution_Read_Vehicle_Odometer_in_Low_Resolution_ConditionCheckRead DataServices_Vehicle_Odometer_in_Low_Resolution_Read_Vehicle_Odometer_in_Low_Resolution_ReadData | DAI_EXT_TF_E_11204; DAI_EXT_TF_E_11205; DAI_EXT_TF_E_11207; DAI_EXT_TF_E_11209; DAI_EXT_TF_E_11211; DAI_EXT_TF_E_11212 |
| ARCH_SW_DIA_0615 | Service 0x22 (Read Data By Identifier) will be implemented for DID 010B (Adjust ISO 15765-2 Block Size and STmin Parameter) | DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_ConditionCheckRead DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_ReadData DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_ConditionCheckRead DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_ReadData | DAI_EXT_TF_E_10601; DAI_EXT_TF_E_10604; DAI_EXT_TF_E_10605; DAI_EXT_TF_E_10609; DAI_EXT_TF_E_10612; DAI_EXT_TF_E_10615 |
| ARCH_SW_DIA_0625 | Service 0x2E (Write Data By Identifier) will be implemented for DID 010B (Adjust ISO 15765-2 Block Size and STmin Parameter) | DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_WriteData DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_WriteData | DAI_EXT_TF_E_11075; DAI_EXT_TF_E_11076; DAI_EXT_TF_E_11327; DAI_EXT_TF_E_11078; DAI_EXT_TF_E_11079; DAI_EXT_TF_E_11080 |
| ARCH_SW_DIA_0635 | Service 0x22 (Read Data By Identifier) will be implemented for DID 01E0 (Task Response Time) | DataServices_Task_Response_Time_Read_Task_histograms_ConditionCheckRead DataServices_Task_Response_Time_Read_Task_histograms_ReadData DataServices_Task_Response_Time_Read_Task_histograms_ReadDataLength | DAI_EXT_TF_E_10620; DAI_EXT_TF_E_10621; DAI_EXT_TF_E_10625; DAI_EXT_TF_E_10628; DAI_EXT_TF_E_10631 |
| ARCH_SW_DIA_0645 | Service 0x22 (Read Data By Identifier) will be implemented for DID 02EA (System Time) | DataServices_System_Time_read_Read_System_time_ConditionCheckRead DataServices_System_Time_read_Read_System_time_ReadData | DAI_EXT_TF_E_10778; DAI_EXT_TF_E_11228; DAI_EXT_TF_E_10776; DAI_EXT_TF_E_10783; DAI_EXT_TF_E_11232 |
| ARCH_SW_DIA_0655 | Service 0x22 (Read Data By Identifier) will be implemented for DiagCommonData Structure which is composed by the following DIDs: EF00 (ECU Extract Version) EF01 (Diagnostic Extract Version) EF03 (Standard Application SW Package Information) EF05 (Minimum SW Version) F100 (Active Diagnostic Information) F104 (ECU Name) F10D (DDS Package Release) F111 (MBC ECU Identification - Hardware Part Number) F121 (MBC ECU Identification - Software Logical Block Part Number(s)) F154 (Hardware Supplier Identification) F155 (Software Supplier Identification) | DiagCommonData | DAI_EXT_TF_E_10830; DAI_EXT_TF_E_10831; DAI_EXT_TF_E_10828; DAI_EXT_TF_E_10835; DAI_EXT_TF_E_11237; DAI_EXT_TF_E_11240; DAI_EXT_TF_E_11241; DAI_EXT_TF_E_11242; DAI_EXT_TF_E_11243; DAI_EXT_TF_E_11244; DAI_EXT_TF_E_11245; DAI_EXT_TF_E_10843; DAI_EXT_TF_E_11247; DAI_EXT_TF_E_10841; DAI_EXT_TF_E_10848; DAI_EXT_TF_E_10849; DAI_EXT_TF_E_10850; DAI_EXT_TF_E_10851; DAI_EXT_TF_E_10852; DAI_EXT_TF_E_10853; DAI_EXT_TF_E_10854; DAI_EXT_TF_E_10862; DAI_EXT_TF_E_11251; DAI_EXT_TF_E_10860; DAI_EXT_TF_E_10867; DAI_EXT_TF_E_11255; DAI_EXT_TF_E_10869; DAI_EXT_TF_E_10870; DAI_EXT_TF_E_10871; DAI_EXT_TF_E_10872; DAI_EXT_TF_E_10873; DAI_EXT_TF_E_10874; DAI_EXT_TF_E_10882; DAI_EXT_TF_E_11256; DAI_EXT_TF_E_10880; DAI_EXT_TF_E_10893; DAI_EXT_TF_E_11260; DAI_EXT_TF_E_11261; DAI_EXT_TF_E_11262; DAI_EXT_TF_E_10906; DAI_EXT_TF_E_11263; DAI_EXT_TF_E_10904; DAI_EXT_TF_E_10910; DAI_EXT_TF_E_11266; DAI_EXT_TF_E_10911; DAI_EXT_TF_E_10912; DAI_EXT_TF_E_10913; DAI_EXT_TF_E_11363; DAI_EXT_TF_E_10935; DAI_EXT_TF_E_11267; DAI_EXT_TF_E_10933; DAI_EXT_TF_E_10939; DAI_EXT_TF_E_11270; DAI_EXT_TF_E_10947; DAI_EXT_TF_E_11273; DAI_EXT_TF_E_10945; DAI_EXT_TF_E_10951; DAI_EXT_TF_E_11277; DAI_EXT_TF_E_10952; DAI_EXT_TF_E_10953; DAI_EXT_TF_E_11278; DAI_EXT_TF_E_10965; DAI_EXT_TF_E_11279; DAI_EXT_TF_E_10963; DAI_EXT_TF_E_10969; DAI_EXT_TF_E_11282; DAI_EXT_TF_E_11283; DAI_EXT_TF_E_11583; DAI_EXT_TF_E_11584; DAI_EXT_TF_E_11585; DAI_EXT_TF_E_11284; DAI_EXT_TF_E_11285; DAI_EXT_TF_E_10976; DAI_EXT_TF_E_11289; DAI_EXT_TF_E_11029; DAI_EXT_TF_E_11302; DAI_EXT_TF_E_11027; DAI_EXT_TF_E_11033; DAI_EXT_TF_E_11306; DAI_EXT_TF_E_11307; DAI_EXT_TF_E_11039; DAI_EXT_TF_E_11308; DAI_EXT_TF_E_11309; DAI_EXT_TF_E_11311; DAI_EXT_TF_E_11313; DAI_EXT_TF_E_11314; DAI_EXT_TF_E_11315 |
| ARCH_SW_DIA_0715 | Service 0x22 (Read Data By Identifier) will be implemented for DID F130 (Reprogramming Capabilities) | DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ConditionCheckRead DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ReadData DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ReadDataLength | DAI_EXT_TF_E_11290; DAI_EXT_TF_E_11291; DAI_EXT_TF_E_10992; DAI_EXT_TF_E_11294; DAI_EXT_TF_E_10999; DAI_EXT_TF_E_11000; DAI_EXT_TF_E_11295; DAI_EXT_TF_E_11296 |
| ARCH_SW_DIA_0725 | Service 0x22 (Read Data By Identifier) will be implemented for DID F131 (Supported Configuration Mechanisms) | DataServices_Supported_configuration_mechanisms_Read_Config_mechanisms_ConditionCheckRead DataServices_Supported_configuration_mechanisms_Read_Config_mechanisms_ReadData | DAI_EXT_TF_E_11008; DAI_EXT_TF_E_11297; DAI_EXT_TF_E_11006; DAI_EXT_TF_E_11012; DAI_EXT_TF_E_11301 |
| ARCH_SW_DIA_0735 | Service 0x22 (Read Data By Identifier) will be implemented for DID F15B (Software Fingerprint) | DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ConditionCheckRead DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ReadData DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ReadDataLength | DAI_EXT_TF_E_11059; DAI_EXT_TF_E_11316; DAI_EXT_TF_E_11057; DAI_EXT_TF_E_11063; DAI_EXT_TF_E_11320; DAI_EXT_TF_E_11321; DAI_EXT_TF_E_11322; DAI_EXT_TF_E_11323; DAI_EXT_TF_E_11324 |
| ARCH_SW_DIA_0745 | Service 0x2E (Write Data By Identifier) will be implemented for DID F804 (Calibration Identifications (CAL ID)) | DataServices_Calibration_Identifications_CAL_ID_Write_CombinedDataElement_WriteData DataServices_Calibration_Identifications_CAL_ID_Write_Number_of_data_items | DAI_EXT_TF_E_11183; DAI_EXT_TF_E_11184; DAI_EXT_TF_E_11337; DAI_EXT_TF_E_11186; DAI_EXT_TF_E_11187; DAI_EXT_TF_E_11188 |
| ARCH_SW_DIA_0755 | Service 0x31 (Routine Control) 0x31 01 Start 0x31 02 Stop 0x31 03 Request Results for RID 0x0245 (Synchronize to Non-volatile Memory) will be implemented | DataServices_Synchronize_to_Non_volatile_Memory_Start Synchronize_to_Non_volatile_Memory_Stop Synchronize_to_Non_volatile_Memory_RequestResults | DAI_EXT_TF_E_11130; DAI_EXT_TF_E_11143; DAI_EXT_TF_E_11145; DAI_EXT_TF_E_11151; DAI_EXT_TF_E_11153; DAI_EXT_TF_E_11157; DAI_EXT_TF_E_11162; DAI_EXT_TF_E_11163; DAI_EXT_TF_E_11164 |
| ARCH_SW_DIA_0765 | Service 0x31 (Routine Control) 0x31 01 Start for RID 0x0277 (Replace Trust Model Root Certificate) will be implemented | Ssa_ProcCtrl_RoutineServices_Routine_Replace_Trust_model_Certificates | DAI_EXT_TF_E_11170; DAI_EXT_TF_E_11171; DAI_EXT_TF_E_11173; DAI_EXT_TF_E_11175; DAI_EXT_TF_E_11178; DAI_EXT_TF_E_11177 |
| ARCH_SW_DIA_0775 | Service 0x19 (Read DTC Information) will be implemented for ReportDTCSnapshotRecordByDTCNumber (RDTCSRBDTCN) - 0x04 | Dcm_Svc19Handler | DAI_EXT_TF_E_10357; DAI_EXT_TF_E_10361; DAI_EXT_TF_E_10366; DAI_EXT_TF_E_10367; DAI_EXT_TF_E_10375; DAI_EXT_TF_E_11195; DAI_EXT_TF_E_11200; DAI_EXT_TF_E_11367; DAI_EXT_TF_E_11536 |
| ARCH_SW_DIA_0785 | Service 0x19 (Read DTC Information) will be implemented for ReportDTCExtendedDataRecordByDTCNumber (RDTCEDRBDTCN) - 0x06 | Dcm_Svc19Handler | DAI_EXT_TF_E_10401; DAI_EXT_TF_E_10408; DAI_EXT_TF_E_10405; DAI_EXT_TF_E_10409; DAI_EXT_TF_E_10413; DAI_EXT_TF_E_10417; DAI_EXT_TF_E_10414; DAI_EXT_TF_E_11191; DAI_EXT_TF_E_11192; DAI_EXT_TF_E_11193; DAI_EXT_TF_E_11194 |
| ARCH_SW_DIA_0800 | Service 0x19 (Read DTC Information) will be implemented for ReportDTCFaultDetectionCounte (RDTCFDC) - 0x14 | Dcm_Svc19Handler | DAI_EXT_TF_E_10448; DAI_EXT_TF_E_10487; DAI_EXT_TF_E_10488 |
| ARCH_SW_DIA_0805 | Service 0x31 (Routine Control) 0x31 01 Start for RID 0x0203 (Check Reprogramming Preconditions) will be implemented | DataServices_Check_Reprogramming_Preconditions_Routine_Start | DAI_EXT_TF_E_11085; DAI_EXT_TF_E_11087; DAI_EXT_TF_E_11088; DAI_EXT_TF_E_11089; DAI_EXT_TF_E_11090; DAI_EXT_TF_E_11094; DAI_EXT_TF_E_11364; DAI_EXT_TF_E_11096; DAI_EXT_TF_E_11099; DAI_EXT_TF_E_11499 |
| ARCH_SW_DIA_0815 | Service 0x31 (Routine Control) 0x31 01 Start for RID 0x0207 (PreCheckProgrammingDependencies) will be implemented | DataServices_PreCheckProgrammingDependencies_Start | DAI_EXT_TF_E_11101; DAI_EXT_TF_E_11102; DAI_EXT_TF_E_11103; DAI_EXT_TF_E_11104; DAI_EXT_TF_E_11107; DAI_EXT_TF_E_11365; DAI_EXT_TF_E_11109; DAI_EXT_TF_E_11111 |
| ARCH_SW_DIA_0830 | Service 0x22 (Read Data By Identifier) will be implemented for DID FD30 (Temperature Sensor Calibration Point) | DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_ConditionCheckRead DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_ReadData | DAI_EXT_TF_E_11540; DAI_EXT_TF_E_11541; DAI_EXT_TF_E_11545; DAI_EXT_TF_E_11548; DAI_EXT_TF_E_11551 |
| ARCH_SW_DIA_0840 | Service 0x2E (Write Data By Identifier) will be implemented for DID FD30 (Temperature Sensor Calibration Point) | DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_WriteData | DAI_EXT_TF_E_11555; DAI_EXT_TF_E_11556; DAI_EXT_TF_E_11559; DAI_EXT_TF_E_11562 |
| ARCH_SW_DIA_0845 | Service 0x22 (Read Data By Identifier) will be implemented for DID FDDF (Velocity PI Regulation Coefficient) | DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_ConditionCheckRead DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_ReadData | DAI_EXT_TF_E_11379; DAI_EXT_TF_E_11380; DAI_EXT_TF_E_11384; DAI_EXT_TF_E_11387; DAI_EXT_TF_E_11390 |
| ARCH_SW_DIA_0855 | Service 0x2E (Write Data By Identifier) will be implemented for DID FDDF (Velocity PI Regulation Coefficients) | DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_WriteData | DAI_EXT_TF_E_11396; DAI_EXT_TF_E_11397; DAI_EXT_TF_E_11399; DAI_EXT_TF_E_11402 |
| ARCH_SW_DIA_0860 | Service 0x31 (Routine Control) 0x31 01 Start for RID 0xF782 (Erase All) will be implemented | RoutineServices_Routine_Erase_All_Start | DAI_EXT_TF_E_11439; DAI_EXT_TF_E_11440; DAI_EXT_TF_E_11443; DAI_EXT_TF_E_11446; DAI_EXT_TF_E_11448 |
| ARCH_SW_DIA_0870 | Service 0x31 (Routine Control) 0x31 01 Start for RID 0xF791 (External Watchdog Test) will be implemented | RoutineServices_Routine_Force_External_WDG_Start | DAI_EXT_TF_E_11572; DAI_EXT_TF_E_11573; DAI_EXT_TF_E_11575; DAI_EXT_TF_E_11577; DAI_EXT_TF_E_11579 |
| ARCH_SW_DIA_0880 | Service 0x22 (Read Data By Identifier) will be implemented for DID 0138 (Customer Settings) | DataServices_Customer_Settings_Read_Customer_Settings_ConditionCheckRead DataServices_Customer_Settings_Read_Customer_Settings_ReadData DataServices_Customer_Settings_Read_Customer_Settings_ReadDataLength | DAI_EXT_TF_E_11523; DAI_EXT_TF_E_11524; DAI_EXT_TF_E_11528; DAI_EXT_TF_E_11531; DAI_EXT_TF_E_11534; DAI_EXT_TF_E_11535 |
| ARCH_SW_DIA_0890 | Service 0x22 (Read Data By Identifier) will be implemented for DID 01A2 (Trust Model CSR Supplier Signature) | DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ReadDataLength DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ReadData | DAI_EXT_TF_E_11410; DAI_EXT_TF_E_11411; DAI_EXT_TF_E_11415; DAI_EXT_TF_E_11418; DAI_EXT_TF_E_11421; DAI_EXT_TF_E_11422 |
| ARCH_SW_DIA_0900 | Service 0x2E (Write Data By Identifier) will be implemented for DID 01A2 (Trust Model CSR Supplier Signature) | DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_WriteData | DAI_EXT_TF_E_11471; DAI_EXT_TF_E_11472; DAI_EXT_TF_E_11476; DAI_EXT_TF_E_11477; DAI_EXT_TF_E_11478; DAI_EXT_TF_E_11479 |
| ARCH_SW_DIA_1000 | Service 0x22 (Read Data By Identifier) will be implemented for DID E104 (Calibration Identifications – CAL ID) | DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ConditionCheckRead DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ReadData DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ReadDataLength | DAI_EXT_TF_E_11426; DAI_EXT_TF_E_11427; DAI_EXT_TF_E_11431; DAI_EXT_TF_E_11434; DAI_EXT_TF_E_11437 |
| ARCH_SW_DIA_1010 | Service 0x22 (Read Data By Identifier) will be implemented for DID F103 (Vedoc Relevant Information) | DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ConditionCheckRead DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ReadData DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ReadDataLength | DAI_EXT_TF_E_11452; DAI_EXT_TF_E_11453; DAI_EXT_TF_E_11457; DAI_EXT_TF_E_11460; DAI_EXT_TF_E_11463; DAI_EXT_TF_E_11464; DAI_EXT_TF_E_11465; DAI_EXT_TF_E_11466 |
| ARCH_SW_DIA_1020 | Service 0x2E (Write Data By Identifier) will be implemented for DID 0340 (Seat Belt Serial Number) | DIA_runDid0340_UniqueSeatbeltSerialNumberWrite | DAI_EXT_TF_E_11487; DAI_EXT_TF_E_11488; DAI_EXT_TF_E_11492; DAI_EXT_TF_E_11493; DAI_EXT_TF_E_11494 |
| ARCH_SW_DIA_1030 | Service 0x31 (Routine Control) 0x31 01 Start for RID 0x0211 (Clear Resource Consumption Data) will be implemented | RoutineServices_Routine_Clear_Resource_Consumption_Data_Start_Start | DAI_EXT_TF_E_11501; DAI_EXT_TF_E_11502; DAI_EXT_TF_E_11504; DAI_EXT_TF_E_11505; DAI_EXT_TF_E_11568; DAI_EXT_TF_E_11570 |

# Description

DIA is intended:

- To handle DiagOnCAN requests delegated by CAN_DCM according to [A4]

- To prepare the DiagOnCAN answer.

Note:

Some DiagOnCAN services are partially or totally managed at CAN module level.

Figure 1: DIA – Static description

## To handle a diagnostic tool request with immediate positive answer

The figure below describes how the SW manages a diagnostic tool request with an immediate positive response.

First of all, the transport protocol will rebuild the complete diagnostic tool request which will be next checked by the CAN_TP component.

If the request is recognized and the related session and security access is granted then the corresponding DIA function will be called to process the requested job. Once completed, the DIA function will return the positive response code.

This positive response code will be handled by the CAN_DCM component which will order the transmission of the positive Diagnostic answer to the CAN_TP.

As shown by the diagram, the real transmission will start 1ms later (next scheduler time-slot).

Figure 2: DIA – To handle a diagnostic tool request with immediate positive answer

## To handle a diagnostic tool request with differed positive answer

With specific diagnostic tool requests, the requested job cannot be completed within the expected DiagOnCAN protocol timings constraints.

In this case, the positive response transmission will be put off to the completion of the job.

The next figure describes how the SW manages a diagnostic tool request with a differed positive response.

First of all the transport protocol will rebuild the complete diagnostic tool request which will be next checked by the CAN_TP component.

If the request is recognized and the related session and security access is granted then the corresponding DIA function will be called to process the requested job.

Since the job is too long, the DIA function will return the “response pending” code.

This code will be handled by the CAN_DCM component which will order the transmission of the “response pending” to the CAN_TP.

As shown by the diagram, the real transmission will start 1ms later (next scheduler time-slot).

From this point, the DIA function shall be periodically called by the CAN_DCM component in order to check if the job is completed.

Once the job is completed, the DIA function will notify the CAN_DCM that positive answer has to be sent (done by CAN_TP 1ms later).

Figure 3: DIA – To handle a diagnostic tool request with differed positive answer

## To answer negative response

A negative response transmission order can be handled by 2 components: CAN_DCM and DIA.

The next figure will only consider the case when the DIA component will order to transmit a negative response.

First of all the transport protocol will rebuild the complete diagnostic tool request which will be next checked by the CAN_TP component.

If the request is recognized and the related session and security access is granted then the corresponding DIA function will be called to process the requested job.

At DIA function level additional conditions are checked in order to know if the job can be performed or not.

Here one specific condition is missing. In this case, the DIA function will notify the CAN_DCM that negative response has to be transmitted (done by CAN_TP 1ms later).

Figure 4: DIA – To answer negative response

# Runnables

## DIA_runDidF18C_ECUSerialNumberRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC((Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidF18C_ECUSerialNumberRead(Dcm_OpStatusType OpStatus, Uint8 * pu8Data)) |  |  |  |
| Object |  |  |  |
| This function shall provide the belt assembly manufacturer date. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| pu8Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0000; ARCH_SW_DIA_0051; ARCH_SW_DIA_0052 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0051 | Service should be available in Default and extended session |  | DAI_EXT_TF_E_8021; DAI_EXT_TF_E_8022 |
| ARCH_SW_DIA_0052 | NVP_BLOCK_ID_AEE_TRACEABILITY_RamBlockData will be read. |  | DAI_EXT_TF_E_8104; DAI_EXT_TF_E_8105; DAI_EXT_TF_E_8106 |

## DIA_runDidF18C_EcuSerialNumberConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidF18C_EcuSerialNumberConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall provide the belt assembly manufacturer date. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| pu8Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0000; ARCH_SW_DIA_0051; ARCH_SW_DIA_0052 |  |  |  |

## DIA_runDidF18C_EcuSerialNumberReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidF18C_EcuSerialNumberReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| This function shall provide the belt assembly manufacturer date. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| DataLength | Uint12 | Out | Length of DID |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0000; ARCH_SW_DIA_0053 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0053 | Runnable shall set response length to 12 |  | DAI_EXT_TF_E_8104 |

## DIA_runDID0340_UniqueSeatbeltSerialNumberRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0340_UniqueSeatbeltSerialNumberRead (Dcm_OpStatusType OpStatus, Uint8 * pu8Data)) |  |  |  |
| Object |  |  |  |
| This function shall provide the belt assembly manufacturer date. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| pu8Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0001; ARCH_SW_DIA_0054; ARCH_SW_DIA_0055 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0054 | The Read Belt Assembly Serial Number shall be available in the following sessions: ⦁Default Session ⦁Extended Session |  | DAI_EXT_TF_E_8062; DAI_EXT_TF_E_8063 |
| ARCH_SW_DIA_0055 | Response data length shall be 28 bytes long. |  | DAI_EXT_TF_E_8102 |

## DIA_runDid0340_UniqueSeatbeltSerialNumberConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0340_UniqueSeatbeltSerialNumberConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall provide the belt assembly manufacturer date. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| pu8Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0001; ARCH_SW_DIA_0054; ARCH_SW_DIA_0055 |  |  |  |

## DIA_runDID0340_UniqueSeatbeltSerialNumberWrite

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0340_UniqueSeatbeltSerialNumberWrite(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0008; ARCH_SW_DIA_0056 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0056 | Response data will corespond to: Unique Seatbelt Serial number, data contained in BELT_ASSEMBLY_MANUF_DATE_AND_SN NVM block. Note edit: block will be written using the 0x2E Write Service. |  | DAI_EXT_TF_E_8078 |

## DIA_runDidFD2F_EcuLocationRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD2F_EcuLocationRead(Dcm_OpStatusType_1 OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| This function shall provide the belt assembly manufacturer date. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0002; ARCH_SW_DIA_0062; ARCH_SW_DIA_0063 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0062 | Service should be available in Default and extended session |  | DAI_EXT_TF_E_8884 |
| ARCH_SW_DIA_0063 | NVP_BLOCK_ID_ECU_LOCATION_RamBlockData will be read. |  | DAI_EXT_TF_E_8895; DAI_EXT_TF_E_8894 |

## DIA_runDidFD10_CycleConfigurationRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD10_CycleConfigurationRead(Dcm_OpStatusType_1 OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| This function shall provide the belt assembly manufacturer date. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0004; ARCH_SW_DIA_0064; ARCH_SW_DIA_0082; ARCH_SW_DIA_0065 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0064 | Service should be available in Development Session |  | DAI_EXT_TF_E_8150 |
| ARCH_SW_DIA_0082 | Data Length shall be 420 bytes long. |  | DAI_EXT_TF_E_9524 |
| ARCH_SW_DIA_0065 | NVP_BLOCK_ID_CYCLE0-34_RamBlockData will be read. |  | DAI_EXT_TF_E_8167; DAI_EXT_TF_E_8168; DAI_EXT_TF_E_8169; DAI_EXT_TF_E_8170; DAI_EXT_TF_E_8171; DAI_EXT_TF_E_8172; DAI_EXT_TF_E_8173; DAI_EXT_TF_E_8174; DAI_EXT_TF_E_8175; DAI_EXT_TF_E_8176; DAI_EXT_TF_E_8177; DAI_EXT_TF_E_8178; DAI_EXT_TF_E_8179; DAI_EXT_TF_E_8180; DAI_EXT_TF_E_8181; DAI_EXT_TF_E_8182; DAI_EXT_TF_E_8183; DAI_EXT_TF_E_8184; DAI_EXT_TF_E_8185; DAI_EXT_TF_E_8186; DAI_EXT_TF_E_8187; DAI_EXT_TF_E_8188; DAI_EXT_TF_E_8189; DAI_EXT_TF_E_8190; DAI_EXT_TF_E_8191; DAI_EXT_TF_E_8192; DAI_EXT_TF_E_8193; DAI_EXT_TF_E_8194; DAI_EXT_TF_E_8195; DAI_EXT_TF_E_8196; DAI_EXT_TF_E_8528; DAI_EXT_TF_E_8527; DAI_EXT_TF_E_9525; DAI_EXT_TF_E_9526; DAI_EXT_TF_E_9527 |

## DIA_runDidFD10_CycleConfigurationWrite

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD10_CycleConfigurationWrite(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0005; ARCH_SW_DIA_0066; ARCH_SW_DIA_0084; ARCH_SW_DIA_0067 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0066 | Service should be available in Development Session |  | DAI_EXT_TF_E_8273 |
| ARCH_SW_DIA_0084 | Data Length shall be 420 bytes long. |  | DAI_EXT_TF_E_9530 |
| ARCH_SW_DIA_0067 | NVP_BLOCK_ID_CYCLE0-34_RamBlockData will be Written. |  | DAI_EXT_TF_E_8276; DAI_EXT_TF_E_8277; DAI_EXT_TF_E_8278; DAI_EXT_TF_E_8279; DAI_EXT_TF_E_8280; DAI_EXT_TF_E_8281; DAI_EXT_TF_E_8282; DAI_EXT_TF_E_8283; DAI_EXT_TF_E_8284; DAI_EXT_TF_E_8285; DAI_EXT_TF_E_8286; DAI_EXT_TF_E_8287; DAI_EXT_TF_E_8288; DAI_EXT_TF_E_8289; DAI_EXT_TF_E_8290; DAI_EXT_TF_E_8291; DAI_EXT_TF_E_8292; DAI_EXT_TF_E_8293; DAI_EXT_TF_E_8294; DAI_EXT_TF_E_8295; DAI_EXT_TF_E_8296; DAI_EXT_TF_E_8297; DAI_EXT_TF_E_8298; DAI_EXT_TF_E_8299; DAI_EXT_TF_E_8300; DAI_EXT_TF_E_8301; DAI_EXT_TF_E_8302; DAI_EXT_TF_E_8303; DAI_EXT_TF_E_8304; DAI_EXT_TF_E_8305; DAI_EXT_TF_E_8557; DAI_EXT_TF_E_8558; DAI_EXT_TF_E_9531; DAI_EXT_TF_E_9532; DAI_EXT_TF_E_9533 |

## DIA_runDidFD2A_StepsLibraryRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD2A_StepsLibraryRead(Dcm_OpStatusType_1 OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| This function shall provide the belt assembly manufacturer date. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0006; ARCH_SW_DIA_0068; ARCH_SW_DIA_0083; ARCH_SW_DIA_0069 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0068 | Service should be available in Development Session |  | DAI_EXT_TF_E_8161 |
| ARCH_SW_DIA_0083 | Data Length shall be 1160 bytes long. |  | DAI_EXT_TF_E_9528 |
| ARCH_SW_DIA_0069 | NVP_BLOCK_ID_STEPS0 -144_RamBlockData will be read. |  | DAI_EXT_TF_E_9529 |

## DIA_runDidFD2A_StepsLibraryWrite

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD2A_StepsLibraryWrite(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0007; ARCH_SW_DIA_0070; ARCH_SW_DIA_0085; ARCH_SW_DIA_0071 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0070 | Service should be available in Development Session |  | DAI_EXT_TF_E_8311 |
| ARCH_SW_DIA_0085 | Data Length shall be 1160 bytes long. |  | DAI_EXT_TF_E_9535 |
| ARCH_SW_DIA_0071 | NVP_BLOCK_ID_STEPS0-144_RamBlockData will be Written. |  | DAI_EXT_TF_E_9536 |

## DIA_runDidFD03_AEETraceabilityNumberWrite

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD03_AEETraceabilityNumberWrite(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0009; ARCH_SW_DIA_0074; ARCH_SW_DIA_0075 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0074 | Service should be available in Default and extended session |  |  |
| ARCH_SW_DIA_0075 | NVP_BLOCK_ID_AEE_TRACEABILITY_RamBlockData will be Written. |  |  |

## DIA_runDid0244_PreSafeCounterRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0244_PreSafeCounterRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| This function shall provide the PreSafe Counter data. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0011; ARCH_SW_DIA_0076; ARCH_SW_DIA_0077; ARCH_SW_DIA_0366; ARCH_SW_DIA_0367 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0076 | Service should be available in Default, Extended and Development Session |  | DAI_EXT_TF_E_8112; DAI_EXT_TF_E_8113; DAI_EXT_TF_E_8118 |
| ARCH_SW_DIA_0077 | NVP_BLOCK_ID_EXECUTION_COUNTERS_RamBlockData will be read. |  | DAI_EXT_TF_E_8119; DAI_EXT_TF_E_8123; DAI_EXT_TF_E_8126; DAI_EXT_TF_E_8127; |
| ARCH_SW_DIA_0366 | Bytes 0-1 shall return the “request counter” value |  | DAI_EXT_TF_E_10221 |
| ARCH_SW_DIA_0367 | Bytes 6-7 shall return the “Max Force counter” value |  | DAI_EXT_TF_E_10222 |

## DIA_runDid0244_PreSafeCounterConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0244_PreSafeCounterConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall provide condition check read for PreSafe Counter. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0011; ARCH_SW_DIA_0076; ARCH_SW_DIA_0077 |  |  |  |

## DIA_runDid0244_PreSafeCounterReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0244_PreSafeCounterReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| This function shall provide the PreSafe Counter data length. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| DataLength | Uint8 | Out | Length of DID |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0011; ARCH_SW_DIA_0078 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0078 | Response data length shall be 8 bytes long |  | DAI_EXT_TF_E_8115 |

## DIA_runDid0243_BSRCounterRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0243_BSRCounterRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| This function shall provide the BSR Counter data. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0012; ARCH_SW_DIA_0079; ARCH_SW_DIA_0080; ARCH_SW_DIA_0081 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0079 | Service should be available in Default, Extended and Development Session. |  | DAI_EXT_TF_E_8134; DAI_EXT_TF_E_8135; DAI_EXT_TF_E_8136 |
| ARCH_SW_DIA_0080 | Response Data Length shall be 3 bytes. |  | DAI_EXT_TF_E_8138 |
| ARCH_SW_DIA_0081 | NVP_BLOCK_ID_EXECUTION_COUNTERS_RamBlockData.ComfortCounter will be read. |  | DAI_EXT_TF_E_8139 |

## DIA_runDid0243_BSRCounterConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0243_BSRCounterConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall provide condition check read for BSR Counter. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0012; ARCH_SW_DIA_0079; ARCH_SW_DIA_0080; ARCH_SW_DIA_0081 |  |  |  |

## DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType,Ct_MBBL_SsaAppl_Coding_CODE) DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0021 |  |  |  |

## DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0021; ARCH_SW_DIA_0086; ARCH_SW_DIA_0087; ARCH_SW_DIA_0202 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0086 | Service shall be available in Default, Extended and Development session |  | DAI_EXT_TF_E_9339; DAI_EXT_TF_E_9340; DAI_EXT_TF_E_9341 |
| ARCH_SW_DIA_0087 | Response data shall be 1048 bytes long From R6.0 Response data shall be 1072 bytes long |  | DAI_EXT_TF_E_9343 |
| ARCH_SW_DIA_0202 | Data contained in NVP_BLOCK_ID_CYCLE_0 to NVP_BLOCK_ID_CYCLE_25 and NVP_BLOCK_ID_STEP_0 to NVP_BLOCK_ID_STEP_91 shall be returned |  | DAI_EXT_TF_E_9518; DAI_EXT_TF_E_9345; DAI_EXT_TF_E_9346; DAI_EXT_TF_E_9347 |

## DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_PreSafe_Cycles_Level_1_8_CombinedSignal_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0022; ARCH_SW_DIA_0088; ARCH_SW_DIA_0089; ARCH_SW_DIA_0203 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0088 | Service shall be available in Default, Extended and Development session |  | DAI_EXT_TF_E_9409; DAI_EXT_TF_E_9410; DAI_EXT_TF_E_9411 |
| ARCH_SW_DIA_0089 | Request data shall be 1048 bytes long For R6.0 Request Data shall be 1072 bytes long |  | DAI_EXT_TF_E_9413 |
| ARCH_SW_DIA_0203 | Blocks NVP_BLOCK_ID_CYCLE_0 to NVP_BLOCK_ID_CYCLE_25 and NVP_BLOCK_ID_STEP_0 to NVP_BLOCK_ID_STEP_91 shall be written |  | DAI_EXT_TF_E_9414 DAI_EXT_TF_E_9415; DAI_EXT_TF_E_9416; DAI_EXT_TF_E_9417 |

## DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0023 |  |  |  |

## DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0023; ARCH_SW_DIA_0090; ARCH_SW_DIA_0091; ARCH_SW_DIA_0092 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0090 | Service shall be available in Default, Extended and Development session |  | DAI_EXT_TF_E_9395; DAI_EXT_TF_E_9396; DAI_EXT_TF_E_9397 |
| ARCH_SW_DIA_0091 | Response data shall be 144 bytes long |  | DAI_EXT_TF_E_9399 |
| ARCH_SW_DIA_0092 | Data contained in NVP_BLOCK_ID_CYCLE_29 to NVP_BLOCK_ID_CYCLE_32 and NVP_BLOCK_ID_STEP_92 to NVP_BLOCK_ID_STEP_115 shall be returned |  | DAI_EXT_TF_E_9400; DAI_EXT_TF_E_9401; DAI_EXT_TF_E_9403 |

## DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ReadData_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Release_Cycles_BSR_and_PreSafe_CombinedSignal_ReadData_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0024; ARCH_SW_DIA_0093; ARCH_SW_DIA_0094; ARCH_SW_DIA_0095 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0093 | Service shall be available in Default, Extended and Development session |  | DAI_EXT_TF_E_9423; DAI_EXT_TF_E_9424; DAI_EXT_TF_E_9425 |
| ARCH_SW_DIA_0094 | Request data shall be 144 bytes long |  | DAI_EXT_TF_E_9427 |
| ARCH_SW_DIA_0095 | Blocks NVP_BLOCK_ID_CYCLE_29 to NVP_BLOCK_ID_CYCLE_32 and NVP_BLOCK_ID_STEP_92 to NVP_BLOCK_ID_STEP_115 shall be written |  | DAI_EXT_TF_E_9428; DAI_EXT_TF_E_9429; DAI_EXT_TF_E_9430 |

## DataServices_Tensioning_Cycle_BSR_CombinedSignal_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Tensioning_Cycle_BSR_CombinedSignal_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0025 |  |  |  |

## DataServices_Tensioning_Cycle_BSR_CombinedSignal_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Tensioning_Cycle_BSR_CombinedSignal_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0025; ARCH_SW_DIA_0096; ARCH_SW_DIA_0097; ARCH_SW_DIA_0098 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0096 | Service shall be available in Default, Extended and Development session |  | DAI_EXT_TF_E_9381; DAI_EXT_TF_E_9382; DAI_EXT_TF_E_9383 |
| ARCH_SW_DIA_0097 | Response data shall be 228 bytes long From R6.0 Response data shall be 233 bytes long |  | DAI_EXT_TF_E_9385 |
| ARCH_SW_DIA_0098 | Data contained in NVP_BLOCK_ID_CYCLE_26 to NVP_BLOCK_ID_CYCLE_28 and NVP_BLOCK_ID_STEP_116 to NVP_BLOCK_ID_STEP_144 shall be returned |  | DAI_EXT_TF_E_9386; DAI_EXT_TF_E_9389 |

## DataServices_ Tensioning_Cycle_BSR_CombinedSignal_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Tensioning_Cycle_BSR_CombinedSignal_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0026; ARCH_SW_DIA_0099; ARCH_SW_DIA_0100; ARCH_SW_DIA_0101 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0099 | Service shall be available in Default, Extended and Development session |  | DAI_EXT_TF_E_9436; DAI_EXT_TF_E_9437; DAI_EXT_TF_E_9438 |
| ARCH_SW_DIA_0100 | Request data shall be 228 bytes long For R6.0 Request Data shall be 233 bytes long |  | DAI_EXT_TF_E_9440 |
| ARCH_SW_DIA_0101 | Blocks NVP_BLOCK_ID_CYCLE_26 to NVP_BLOCK_ID_CYCLE_28 and NVP_BLOCK_ID_STEP_116 to NVP_BLOCK_ID_STEP_144 shall be written |  | DAI_EXT_TF_E_9441; DAI_EXT_TF_E_9442 |

## DIA_runDid0310_VehicleEquipment_ConditionCheckRead

### DataServices_Vehicle_Equipment_Read_Equipment_1_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_1_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027 |  |  |  |

### DataServices_Vehicle_Equipment_Read_Equipment_2_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_2_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027 |  |  |  |

### DataServices_Vehicle_Equipment_Read_Equipment_3_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_3_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027 |  |  |  |

### DataServices_Vehicle_Equipment_Read_Equipment_4_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_4_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027 |  |  |  |

### DataServices_Vehicle_Equipment_Read_Equipment_5_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_5_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027 |  |  |  |

### DataServices_Vehicle_Equipment_Read_Equipment_6_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_6_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027 |  |  |  |

### DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType,Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027 |  |  |  |

### DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType,Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027 |  |  |  |

### DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType,Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027 |  |  |  |

### DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType,Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027 |  |  |  |

## DIA_runDid0310_VehicleEquipment_ReadData

### DataServices_Vehicle_Equipment_Read_Equipment_1_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_1_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027; ARCH_SW_DIA_0102; ARCH_SW_DIA_0103; ARCH_SW_DIA_0104; ARCH_SW_DIA_0368 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0102 | Service shall be available in Default, Extended and Development session |  | DAI_EXT_TF_E_9367; DAI_EXT_TF_E_9368; DAI_EXT_TF_E_9369 |
| ARCH_SW_DIA_0103 | Response data shall be 1 byte long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0104 | Data returned shall represent Equipment 1 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment1 |  | DAI_EXT_TF_E_9496; DAI_EXT_TF_E_9497 |
| ARCH_SW_DIA_0368 | Byte 0 shall represent Equipment 1, byte encoded as follows: Byte0_bit0 – Configuration of Left right hand drive (steering style of the vehicle) 0x00 – right hand drive 0x01 – left hand drive [Default] Byte0_bit1 – Activation / Deactivation of BSR 0x00 – yes – enabled [Default] 0x01 – no – disabled Byte0_bit2 – Belt Handover ngagedngy 0x00 – yes – Belt hand over available 0x01 – no – Belt hand over not available [Default] Byte0_bit3 – Reserved [Default = 0x00] Byte0_bit4 – Activation / Deactivation Haptic Warning left 0x00 – yes – enabled [Default] 0x01 – no – disabled Byte0_bit5 – Activation / Deactivation Haptic Warning right 0x00 – yes – enabled 0x01 – no – disabled [Default] Byte0_bit6 - Activation / Deactivation of Service Provider (API) 0x00 – yes – enabled [Default] 0x01 – no – disabled Byte0_bit7 – Reserved [Default=0x00] |  | DAI_EXT_TF_E_10223 |

### DataServices_Vehicle_Equipment_Read_Equipment_2_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_2_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027; ARCH_SW_DIA_0108; ARCH_SW_DIA_0109 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0108 | Response data shall be 1 byte long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0109 | Data returned shall represent Equipment 2 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment2 |  | DAI_EXT_TF_E_9498 DAI_EXT_TF_E_9499 |

### DataServices_Vehicle_Equipment_Read_Equipment_3_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_3_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027; ARCH_SW_DIA_0112; ARCH_SW_DIA_0113 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0112 | Response data shall be 1 byte long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0113 | Data returned shall represent Equipment 3 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment3 |  | DAI_EXT_TF_E_9500; DAI_EXT_TF_E_9501 |

### DataServices_Vehicle_Equipment_Read_Equipment_4_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_4_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027; ARCH_SW_DIA_0116; ARCH_SW_DIA_0117 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0116 | Response data shall be 1 byte long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0117 | Data returned shall represent Equipment 4 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment4 |  | DAI_EXT_TF_E_9502; DAI_EXT_TF_E_9503 |

### DataServices_Vehicle_Equipment_Read_Equipment_5_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_5_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027; ARCH_SW_DIA_0120; ARCH_SW_DIA_0121 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0120 | Response data shall be 1 byte long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0121 | Data returned shall represent Equipment 5 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment5 |  | DAI_EXT_TF_E_9504; DAI_EXT_TF_E_9505 |

### DataServices_Vehicle_Equipment_Read_Equipment_6_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_6_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027; ARCH_SW_DIA_0124; ARCH_SW_DIA_0125 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0124 | Response data shall be 1 byte long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0125 | Data returned shall represent Equipment 6 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment6 |  | DAI_EXT_TF_E_9506; DAI_EXT_TF_E_9507 |

### DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027; ARCH_SW_DIA_0128; ARCH_SW_DIA_0129 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0128 | Response data shall be 2 bytes long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0129 | Data returned shall represent Uberspannungsabbruchschwelle stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment. Uberspannungsabbruchschwelle |  | DAI_EXT_TF_E_9510; DAI_EXT_TF_E_9511 |

### DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027; ARCH_SW_DIA_0132; ARCH_SW_DIA_0133 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0132 | Response data shall be 2 bytes long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0133 | Data returned shall represent Uberspannungsbereichsschwelle stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment. Uberspannungsbereichsschwelle |  | DAI_EXT_TF_E_9512; DAI_EXT_TF_E_9513 |

### DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027; ARCH_SW_DIA_0136; ARCH_SW_DIA_0137 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0136 | Response data shall be 2 bytes long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0137 | Data returned shall represent Überspannungsbereichsschwelle stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment. Überspannungsbereichsschwelle |  | DAI_EXT_TF_E_9514; DAI_EXT_TF_E_9515 |

### DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0027; ARCH_SW_DIA_0140; ARCH_SW_DIA_0141 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0140 | Response data shall be 2 bytes long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0141 | Data returned shall represent Überspannungsabbruchschwelle stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment. Überspannungsabbruchschwelle |  | DAI_EXT_TF_E_9516; DAI_EXT_TF_E_9517 |

## DIA_runDid0310_VehicleEquipment_WriteData

### DataServices_Vehicle_Equipment_Read_Equipment_1_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_1_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0028; ARCH_SW_DIA_0105; ARCH_SW_DIA_0106; ARCH_SW_DIA_0107; ARCH_SW_DIA_0393 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0105 | Service shall be available in Default, Extended and Development session |  | DAI_EXT_TF_E_9448; DAI_EXT_TF_E_9449; DAI_EXT_TF_E_9450 |
| ARCH_SW_DIA_0106 | Request data shall be 1 byte long |  | DAI_EXT_TF_E_9452 |
| ARCH_SW_DIA_0107 | Runnable shall write Equipment 1 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment1 |  | DAI_EXT_TF_E_9453; DAI_EXT_TF_E_9465 |
| ARCH_SW_DIA_0393 | The new data written by Vehicle Equipment Configuration Data service (0x2E 0310) shall be stored in non-volatile memoy and shall be available only after reset. |  | DAI_EXT_TF_E_10253 |

### DataServices_Vehicle_Equipment_Read_Equipment_2_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_2_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0028; ARCH_SW_DIA_0110; ARCH_SW_DIA_0111 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0110 | Request data shall be 1 byte long |  | DAI_EXT_TF_E_9452 |
| ARCH_SW_DIA_0111 | Runnable shall write Equipment 2 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment2 |  | DAI_EXT_TF_E_9467; DAI_EXT_TF_E_9468 |

### DataServices_Vehicle_Equipment_Read_Equipment_3_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_3_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0028; ARCH_SW_DIA_0114 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0114 | Request data shall be 1 byte long |  | DAI_EXT_TF_E_9452 |
| ARCH_SW_DIA_0115 | Runnable shall write Equipment 3 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment3 |  | DAI_EXT_TF_E_9469; DAI_EXT_TF_E_9470 |

### DataServices_Vehicle_Equipment_Read_Equipment_4_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_4_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0028; ARCH_SW_DIA_0118; ARCH_SW_DIA_0119 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0118 | Request data shall be 1 byte long |  | DAI_EXT_TF_E_9452 |
| ARCH_SW_DIA_0119 | Runnable shall write Equipment 4 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment4 |  | DAI_EXT_TF_E_9471; DAI_EXT_TF_E_9472 |

### DataServices_Vehicle_Equipment_Read_Equipment_5_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_5_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0028; ARCH_SW_DIA_0122; ARCH_SW_DIA_0123 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0122 | Request data shall be 1 byte long |  | DAI_EXT_TF_E_9452 |
| ARCH_SW_DIA_0123 | Runnable shall write Equipment 5 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment5 |  | DAI_EXT_TF_E_9473; DAI_EXT_TF_E_9474 |

### DataServices_Vehicle_Equipment_Read_Equipment_6_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Equipment_1_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0028; ARCH_SW_DIA_0126; ARCH_SW_DIA_0127 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0126 | Request data shall be 1 byte long |  | DAI_EXT_TF_E_9452 |
| ARCH_SW_DIA_0127 | Runnable shall write Equipment 6 stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment.Equipment6 |  | DAI_EXT_TF_E_9475; DAI_EXT_TF_E_9476 |

### DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Uberspannungsabbruchschwelle_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0028; ARCH_SW_DIA_0130; ARCH_SW_DIA_0131 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0130 | Request data shall be 2 bytes long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0131 | Runanble shall write Uberspannungsabbruchschwelle stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment. Uberspannungsabbruchschwelle |  | DAI_EXT_TF_E_9479; DAI_EXT_TF_E_9480 |

### DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Uberspannungsbereichsschwelle_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0028; ARCH_SW_DIA_0134; ARCH_SW_DIA_0135 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0134 | Request data shall be 2 bytes long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0135 | Runanble shall write Uberspannungsbereichsschwelle stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment. Uberspannungsbereichsschwelle |  | DAI_EXT_TF_E_9481; DAI_EXT_TF_E_9482 |

### DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Unterspannungsabbruchschwelle_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0028; ARCH_SW_DIA_0138; ARCH_SW_DIA_0139 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0138 | Request data shall be 2 bytes long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0139 | Runanble shall write Unterspannungsabbruchschwelle stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment. Unterspannungsabbruchschwelle |  | DAI_EXT_TF_E_9483; DAI_EXT_TF_E_9484 |

### DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Vehicle_Equipment_Read_Unterspannungsbereichsschwelle_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0028; ARCH_SW_DIA_0142; ARCH_SW_DIA_0143 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0142 | Request data shall be 2 bytes long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0143 | Runanble shall write Unterspannungsbereichsschwelle stored in Rte_Ct_MBBL_SsaAppl_Coding_VehicleEquipment. Unterspannungsbereichsschwelle |  | DAI_EXT_TF_E_9487; DAI_EXT_TF_E_9488 |

## DIA_runDid0311_VehicleEquipmentDevelopment_ConditionCheckRead

### DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType,Ct_MBBL_SsaAppl_Coding_CODE) DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0029 |  |  |  |

### DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType,Ct_MBBL_SsaAppl_Coding_CODE) DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0029 |  |  |  |

## DIA_runDid0311_VehicleEquipmentDevelopment_ReadData

### DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0029; ARCH_SW_DIA_0150; ARCH_SW_DIA_0151 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0150 | Response data shall be 1 byte long |  | DAI_EXT_TF_E_9357 |
| ARCH_SW_DIA_0151 | Data returned shall represent Zykluszeit Messtechnikstored in Rte_Ct_MBBL_SsaAppl_Coding_ZZZ_VehicleEquipment_Development. Zykluszeit_Messtechnik |  | DAI_EXT_TF_E_9494 |

### DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0029; ARCH_SW_DIA_0144; ARCH_SW_DIA_0145; ARCH_SW_DIA_0146 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0144 | Service shall be available in Default, Extended and Development session |  | DAI_EXT_TF_E_9353; DAI_EXT_TF_E_9354; DAI_EXT_TF_E_9355 |
| ARCH_SW_DIA_0145 | Response data shall be 2 bytes long |  | DAI_EXT_TF_E_9357 |
| ARCH_SW_DIA_0146 | Data returned shall represent Bitfield stored in Rte_Ct_MBBL_SsaAppl_Coding_ZZZ_VehicleEquipment_Development.Bitfield |  | DAI_EXT_TF_E_9492; DAI_EXT_TF_E_9493 |

## DIA_runDid0311_VehicleEquipmentDevelopment_WriteData

### DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_ZZZ_Vehicle_Equipment_Development_Read_Zykluszeit_Messtechnik_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0030; ARCH_SW_DIA_0152; ARCH_SW_DIA_0153; ARCH_SW_DIA_0394; ARCH_SW_DIA_0507 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0152 | Request data shall be 3 byte long |  | DAI_EXT_TF_E_9463 |
| ARCH_SW_DIA_0153 | Runnable shall write Zykluszeit Messtechnik stored in Rte_Ct_MBBL_SsaAppl_Coding_ZZZ_VehicleEquipment_Development. Zykluszeit_Messtechnik |  | DAI_EXT_TF_E_9490 |
| ARCH_SW_DIA_0394 | The new data written by Vehicle Equipment Configuration Data service (0x2E 0311) shall be stored in non-volatile memoy and shall be available only after reset. |  | DAI_EXT_TF_E_10255 |
| ARCH_SW_DIA_0507 | Byte 0: Reserved (shall not be used) ----------------------------------------------- Byte1_bit0 - Activation/deactivation of PRE-SAFE function 0x00 - yes - PRE-SAFE function activated 0x01 - no - PRE-SAFE function not activated Byte1_bit1 - Activation/deactivation of Display message (signal RBTM_xy_Disp_Rq_ST3) 0x00 - yes - Display message activated 0x01 - no - Display message not activated Byte1_bit2 - Activation / deactivation of counter limit. 0x00 - yes - counter limit deactivated 0x01 - no - counter limit activated Byte1_bit3 - CRC Fault monitoring 0x00 - yes - deactivated 0x01 - no - Activated Byte1_bits4-7: Reserved ----------------------------------------------- Byte 2: Reserved (shall not be used) |  | DAI_EXT_TF_E_10254 |

### DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_WriteData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_ZZZ_Vehicle_Equipment_Development_Read_Bitfield_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write NVP parameter about serial number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0030; ARCH_SW_DIA_0147; ARCH_SW_DIA_0148; ARCH_SW_DIA_0149 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0147 | Service shall be available in Default, Extended and Development session |  | DAI_EXT_TF_E_9459; DAI_EXT_TF_E_9460; DAI_EXT_TF_E_9461 |
| ARCH_SW_DIA_0148 | Request data shall be 3 byte long |  | DAI_EXT_TF_E_9463 |
| ARCH_SW_DIA_0149 | Runnable shall write Bitfield stored in Rte_Ct_MBBL_SsaAppl_Coding_ZZZ_VehicleEquipment_Development.Bitfield |  | DAI_EXT_TF_E_9464; DAI_EXT_TF_E_9489 |

## DIA_runDidFD08_ResetCauseManagementRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD08_ResetCauseManagementRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0031; ARCH_SW_DIA_0154; ARCH_SW_DIA_0155; ARCH_SW_DIA_0156 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0154 | Service shall be available in Development session |  | DAI_EXT_TF_E_8660 |
| ARCH_SW_DIA_0155 | Response data shall be 1 byte long |  | DAI_EXT_TF_E_9371 |
| ARCH_SW_DIA_0156 | Response shall return Last SW Reset Cause |  | DAI_EXT_TF_E_8670; DAI_EXT_TF_E_8671; DAI_EXT_TF_E_8672; DAI_EXT_TF_E_8673; DAI_EXT_TF_E_8674; DAI_EXT_TF_E_8675; DAI_EXT_TF_E_8676; DAI_EXT_TF_E_8677; DAI_EXT_TF_E_8678; DAI_EXT_TF_E_8679; DAI_EXT_TF_E_8680; DAI_EXT_TF_E_8681; DAI_EXT_TF_E_8682; DAI_EXT_TF_E_8683; DAI_EXT_TF_E_8684; DAI_EXT_TF_E_8685; DAI_EXT_TF_E_8686; DAI_EXT_TF_E_8687; DAI_EXT_TF_E_8688; DAI_EXT_TF_E_8689; DAI_EXT_TF_E_8690 |

## DIA_runDidFD08_ResetCauseManagementConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD08_ResetCauseManagementConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0031 |  |  |  |

## DIA_runDidFD09_LastWarmResetRead_LastDetectedWarmResetCause

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD09_LastWarmResetRead_LastDetectedWarmResetCause(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0032; ARCH_SW_DIA_0157; ARCH_SW_DIA_0158; ARCH_SW_DIA_0159 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0157 | Service shall be available in Development session |  | DAI_EXT_TF_E_8629 |
| ARCH_SW_DIA_0158 | Response data shall be 4 bytes long |  | DAI_EXT_TF_E_8640; |
| ARCH_SW_DIA_0159 | Response shall return Last Detected Warm Reset Cause |  | DAI_EXT_TF_E_8640; DAI_EXT_TF_E_8641; DAI_EXT_TF_E_8642; DAI_EXT_TF_E_8643; DAI_EXT_TF_E_8644; DAI_EXT_TF_E_8645; DAI_EXT_TF_E_8646; DAI_EXT_TF_E_8647; DAI_EXT_TF_E_8648; DAI_EXT_TF_E_8649; DAI_EXT_TF_E_8650; DAI_EXT_TF_E_8651; DAI_EXT_TF_E_8652; DAI_EXT_TF_E_8655; DAI_EXT_TF_E_8654 |

## DIA_runDidFD09_LastWarmReseConditionCheckRead_LastDetectedWarmResetCause

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD09_LastWarmReseConditionChecktRead_LastDetectedWarmResetCause(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0032 |  |  |  |

## DIA_runDidFD09_LastWarmResetRead_ResetCauseCounter

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD09_LastWarmResetRead_ResetCauseCounter(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0032; ARCH_SW_DIA_0160; ARCH_SW_DIA_0161 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0160 | Response data shall be 4 bytes long |  | DAI_EXT_TF_E_8653 |
| ARCH_SW_DIA_0161 | Response shall return Reset Cause Counter |  | DAI_EXT_TF_E_8653; DAI_EXT_TF_E_8656 |

## DIA_runDidFD09_LastWarmResetConditionCheckRead_ResetCauseCounter

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD09_LastWarmResetConditionCheckRead_ResetCauseCounter(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0032 |  |  |  |

## DIA_runDidFD31_HBCalibrationRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD31_HBCalibrationRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0033; ARCH_SW_DIA_0162; ARCH_SW_DIA_0163; ARCH_SW_DIA_0164 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0162 | Service shall be available in Development session |  | DAI_EXT_TF_E_8829 |
| ARCH_SW_DIA_0163 | Response data shall be 40 bytes long |  | DAI_EXT_TF_E_8841 |
| ARCH_SW_DIA_0164 | Response data shall return HB Calibration parameters stored in NVP_BLOCK_ID_HB_CALIBRATION |  | DAI_EXT_TF_E_8841; DAI_EXT_TF_E_8842; DAI_EXT_TF_E_8843; DAI_EXT_TF_E_8844; DAI_EXT_TF_E_8845 |

## DIA_runDidFD31_HBCalibrationWriteConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD31_HBCalibrationWriteConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0033 |  |  |  |

## DIA_runDidFE00_AECStatusRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFE00_AECStatusRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0035; ARCH_SW_DIA_0165; ARCH_SW_DIA_0166 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0165 | Service shall be available in Development session |  | DAI_EXT_TF_E_8694 |
| ARCH_SW_DIA_0166 | Response data shall return AEC_ID_X and Status_X of said AEC |  | DAI_EXT_TF_E_8707; DAI_EXT_TF_E_8708; DAI_EXT_TF_E_8705 |

## DIA_runDidFE00_AECStatusConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFE00_AECStatusConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0035 |  |  |  |

## DIA_runDidFE00_AECStatusReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFE00_AECStatusReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| DataLength | Uint168 | Out | Length of DID |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0035; ARCH_SW_DIA_0167 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0167 | Runnable shall set response length to 168 |  | DAI_EXT_TF_E_8710 |

## DIA_runDidFEFF_MeasurementFrameConfigurationRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFEFF_MeasuruementFrameConfigurationRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0036; ARCH_SW_DIA_0168; ARCH_SW_DIA_0169; ARCH_SW_DIA_0170 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0168 | Service shall be available in Development session |  | DAI_EXT_TF_E_8714 |
| ARCH_SW_DIA_0169 | Response data shall be 4 bytes long |  | DAI_EXT_TF_E_8725 |
| ARCH_SW_DIA_0170 | Response shall return Timing and Block Config stored in NVP_BLOCK_ID_MEASUREMENT_FRAME_CONFIG |  | DAI_EXT_TF_E_8726; DAI_EXT_TF_E_8728; DAI_EXT_TF_E_8729 |

## DIA_runDidFEFF_MeasurementFrameConfigurationConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFEFF_MeasuruementFrameConfigurationConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeesponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| . |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0036 |  |  |  |

## DIA_runDidFDFF_MeasurementFrameBlock12ConfigRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFDFF_MeasurementFrameBlock12ConfigRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0037; ARCH_SW_DIA_0171; ARCH_SW_DIA_0172; ARCH_SW_DIA_0173 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0171 | Service shall be available in Development session |  | DAI_EXT_TF_E_8733 |
| ARCH_SW_DIA_0172 | Response data shall be 5 bytes long |  | DAI_EXT_TF_E_8745 |
| ARCH_SW_DIA_0173 | Response shall return Ram Address and Size stored in NVP_BLOCK_ID_MEASUREMENT_FRAME_12_CONFIG |  | DAI_EXT_TF_E_8746; DAI_EXT_TF_E_8747; DAI_EXT_TF_E_8748 |

## DIA_runDidFDFF_MeasurementFrameBlock12ConfigConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFDFF_MeasurementFrameBlock12ConfigConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0037 |  |  |  |

## DIA_runDidFDFF_MeasurementFrameBlock12ConfigWrite

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFDFF_MeasurementFrameBlock12ConfigWrite(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0039; ARCH_SW_DIA_0174; ARCH_SW_DIA_0175 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0174 | Service shall be available in Development session |  | DAI_EXT_TF_E_9081 |
| ARCH_SW_DIA_0175 | Request data shall be 5 bytes long |  | DAI_EXT_TF_E_8745 |
| ARCH_SW_DIA_0176 | Runnable shall write Ram Address and Size stored in NVP_BLOCK_ID_MEASUREMENT_FRAME_12_CONFIG |  | DAI_EXT_TF_E_9095; DAI_EXT_TF_E_9096; DAI_EXT_TF_E_9097; DAI_EXT_TF_E_9098; DAI_EXT_TF_E_9099 |

## DIA_runSid23_ReadMemoryByAddress

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runSid23_ReadMemoryByAddress (Dcm_OpStatusType OpStatus, uint8 MemoryIdentifier, uint32 MemoryAddress, uint32 MemorySize, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) MemoryData, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | OperationStatus response code (if applicable) |
| MemoryIdentifier | Uint8 | In | Address and Length format identifier |
| MemoryAddress | Uint32 | In | Address to read from |
| MemorySize | Uint32 | In | Size to read |
| MemoryData | Array of unsigned 8-bits | Out | Buffer to put read data into |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0040; ARCH_SW_DIA_0177; ARCH_SW_DIA_0178; ARCH_SW_DIA_0179; ARCH_SW_DIA_0180; ARCH_SW_DIA_0181; ARCH_SW_DIA_0182 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0177 | Service shall be available in Development session |  | DAI_EXT_TF_E_8918 |
| ARCH_SW_DIA_0178 | Runnable shall return data with the length of maximum 300 byes |  | DAI_EXT_TF_E_8931 |
| ARCH_SW_DIA_0179 | MemoryIdentifier shall represent Address and Length format identifier |  | DAI_EXT_TF_E_8933; DAI_EXT_TF_E_8934; DAI_EXT_TF_E_8935; DAI_EXT_TF_E_8936 |
| ARCH_SW_DIA_0180 | MemoryAddress shall represent the address to read from |  | DAI_EXT_TF_E_8937 |
| ARCH_SW_DIA_0181 | MemorySize shall represent the size to read |  | DAI_EXT_TF_E_8938; DAI_EXT_TF_E_8939; DAI_EXT_TF_E_8942 |
| ARCH_SW_DIA_0182 | MemoryData shall return data to be read with parameters above |  | DAI_EXT_TF_E_8940; DAI_EXT_TF_E_8941 |

## DIA_runSid3D_WriteMemoryByAddress

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runSid3D_WriteMemoryByAddress (Dcm_OpStatusType OpStatus, uint8 MemoryIdentifier, uint32 MemoryAddress, uint32 MemorySize, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) MemoryData, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | OperationStatus response code (if applicable) |
| MemoryIdentifier | Uint8 | In | Address and Length format identifier |
| MemoryAddress | Uint32 | In | Address to write to |
| MemorySize | Uint32 | In | Size to write |
| MemoryData | Array of unsigned 8-bits | In | Buffer of data to be written |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0041; ARCH_SW_DIA_0183; ARCH_SW_DIA_0184; ARCH_SW_DIA_0185; ARCH_SW_DIA_0186; ARCH_SW_DIA_0187 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0183 | Service shall be available in Development session |  | DAI_EXT_TF_E_9205 |
| ARCH_SW_DIA_0184 | MemoryIdentifier shall represent Address and Length format identifier |  | DAI_EXT_TF_E_9217; DAI_EXT_TF_E_9218; DAI_EXT_TF_E_9223 |
| ARCH_SW_DIA_0185 | MemoryAddress shall represent the address to write to |  | DAI_EXT_TF_E_9219; DAI_EXT_TF_E_9224 |
| ARCH_SW_DIA_0186 | MemorySize shall represent the size to write |  | DAI_EXT_TF_E_9220; DAI_EXT_TF_E_9221; DAI_EXT_TF_E_9225; DAI_EXT_TF_E_9226 |
| ARCH_SW_DIA_0187 | New R8.1 Feature: MemoryAddress also works with the EEPROM Alias. Hence, the MemoryAddress range for the EEPROM Alias is between: 0xEE000000 - 0xFF000000 EEPROM Alias for each block can be taken from the NVP Excel file on the column U from Dynamic EEPROM. There are in total 27 Blocks that can be modified via the EEPROM Alias. When writing to an EEPROM Alias Block, the Request should look like this: 3D 24 EE 12 00 00 00 01 9C Legend: EEPROM ALIAS BLOCK ADDRESS Data Note: Service 3D supports from DCM several other ALFIDs (Address and Length Format Identifiers). For example, if you want to write 1 byte length of Data the request should be: 3D 14 EE 12 00 00 01 9C Only works via Raw Request. |  | DAI_EXT_TF_E_9222 |

## DIA_runRidF703_CycleExecutionStart

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runRidF703_CycleExecutionStart(Dcm_StartDataIn_Routine_Cycle_Execution_Start_CycleType Cycle, Dcm_OpStatusType OpStatus, P2VAR(Dcm_StartDataOut_Routine_Cycle_Execution_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Cycle | Uint8 | In | Requested Cycle |
| routineInfo | Uint8 * | Out | Routine status |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0013; ARCH_SW_DIA_0188; ARCH_SW_DIA_0189; ARCH_SW_DIA_0190 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0188 | Service shall be available in Development session |  | DAI_EXT_TF_E_8946 |
| ARCH_SW_DIA_0189 | The RoutineControlOptionRecord shall reflect the selected cycle |  | DAI_EXT_TF_E_8968 |
| ARCH_SW_DIA_0190 | RoutineInfo can have the following values: 0x00, 0x01, 0x02 and 0xFF |  | DAI_EXT_TF_E_8997 |

## DIA_runRidF703_CycleExecutionStop

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runRidF703_CycleExecutionStop(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StopDataOut_Routine_Cycle_Execution_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| routineInfo | Uint8 * | Out | Routine status |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0014; ARCH_SW_DIA_0191 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0191 | Service shall be available in Development session |  | DAI_EXT_TF_E_8946 |

## DIA_runRid0329_TriggerBSRStart

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runRid0329_TriggerBSRStart(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StartDataOut_Routine_Trigger_BSR_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_StartDataOut_Routine_Trigger_BSR_Start_ResultType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Result, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Result | Uint8 * | Out | Result of request |
| routineInfo | Uint8 * | Out | Routine status |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0015; ARCH_SW_DIA_0192; ARCH_SW_DIA_0193 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0192 | Service shall be available in Extended and Development session |  | DAI_EXT_TF_E_9308; DAI_EXT_TF_E_9309 |
| ARCH_SW_DIA_0193 | Result can have the following values: 0x00, 0x01 and 0x02 |  | DAI_EXT_TF_E_9317; DAI_EXT_TF_E_9318; DAI_EXT_TF_E_9319 |

## DIA_runRid0329_TriggerBSRStop

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runRid0329_TriggerBSRStop(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StopDataOut_Routine_Trigger_BSR_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| routineInfo | Uint8 * | Out | Routine status |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0016; ARCH_SW_DIA_0194 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0194 | Service shall be available in Extended and Development session |  | DAI_EXT_TF_E_9308; DAI_EXT_TF_E_9309 |

## DIA_runRid0329_TriggerBSRRequestResults

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runRid0329_TriggerBSRRequestResults(Dcm_OpStatusType OpStatus, P2VAR(Dcm_RequestDataOut_Routine_Trigger_BSR_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_RequestDataOut_Routine_Trigger_BSR_Start_ResultType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Result, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| routineInfo | Uint8 * | Out | Routine status |
| Result | Uint8 * | Out | Result of request |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0017; ARCH_SW_DIA_0195; ARCH_SW_DIA_0196 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0195 | Service shall be available in Extended and Development session |  | DAI_EXT_TF_E_9308; DAI_EXT_TF_E_9309 |
| ARCH_SW_DIA_0196 | Result can have the following values: 0x00, 0x01 and 0x02 |  | DAI_EXT_TF_E_9330; DAI_EXT_TF_E_9331; DAI_EXT_TF_E_9332 |

## DIA_runRid0330_TriggerTensioningCycleStart

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runRid0330_TriggerTensioningCycleStart(Dcm_StartDataIn_Routine_Trigger_Tensioning_Cycle_Start_Type_of_tensioning_cycleType Type_of_tensioning_cycle, Dcm_StartDataIn_Routine_Trigger_Tensioning_Cycle_Start_PreSafe_DurationType PreSafe_Duration, Dcm_OpStatusType OpStatus, P2VAR(Dcm_StartDataOut_Routine_Trigger_Tensioning_Cycle_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_StartDataOut_Routine_Trigger_Tensioning_Cycle_Start_ResultType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Result, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Type_of_tensioning_cycle | Uint8 | In | Type of cycle to trigger |
| PreSafe_Duration | Uint8 | In | Duration |
| Result | Uint8 * | Out | Result of request |
| routineInfo | Uint8 * | Out | Routine status |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0019; ARCH_SW_DIA_0197; ARCH_SW_DIA_0198; ARCH_SW_DIA_0199; ARCH_SW_DIA_0200 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0197 | Service shall be available in Extended and Development session |  | DAI_EXT_TF_E_8555; DAI_EXT_TF_E_8556 |
| ARCH_SW_DIA_0198 | Type of Tensioning Cycle shall accept only values between 0x01 and 0x05. |  | DAI_EXT_TF_E_8521 |
| ARCH_SW_DIA_0199 | Presafe Duration shall determine the time of the tensioning cycle |  | DAI_EXT_TF_E_8522 |
| ARCH_SW_DIA_0200 | Result can have the following values: 0x00, 0x01 and 0x02 |  | DAI_EXT_TF_E_8524; DAI_EXT_TF_E_8525; DAI_EXT_TF_E_8526 |

## DIA_runRid0330_TriggerTensioningCycleStop

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runRid0330_TriggerTensioningCycleStop(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StopDataOut_Routine_Trigger_Tensioning_Cycle_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| routineInfo | Uint8 * | Out | Routine status |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0018; ARCH_SW_DIA_0201 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0201 | Service shall be available in Extended and Development session |  | DAI_EXT_TF_E_8555; DAI_EXT_TF_E_8556 |

## DIA_runRid0330_TriggerTensioningCycleRequestResults

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runRid0330_TriggerTensioningCycleRequestResults(Dcm_OpStatusType OpStatus, P2VAR(Dcm_RequestDataOut_Routine_Trigger_Tensioning_Cycle_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_RequestDataOut_Routine_Trigger_Tensioning_Cycle_Start_ResultType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Result, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| routineInfo | Uint8 * | Out | Routine status |
| Result | Uint8 * | Out | Result of request |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0020; ARCH_SW_DIA_0204; ARCH_SW_DIA_0205 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0204 | Service shall be available in Extended and Development session |  | DAI_EXT_TF_E_8555; DAI_EXT_TF_E_8556 |
| ARCH_SW_DIA_0205 | Result can have the following values: 0x00, 0x01 and 0x02 |  | DAI_EXT_TF_E_8549; DAI_EXT_TF_E_8550; DAI_EXT_TF_E_8551 |

## DIA_runRid0302_TriggerPresafeDisplayRequestRequestResults

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runRid0302_TriggerPresafeDisplayRequestRequestResults(Dcm_OpStatusType OpStatus, P2VAR(Dcm_RequestDataOut_Routine_Trigger_Presafe_Display_Request_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_RequestDataOut_Routine_Trigger_Presafe_Display_Request_Start_ResultType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Result, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function returns the Request Results of TriggerPresafeDisplay |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| routineInfo | Uint8 * | Out | Routine status |
| Result | Uint8 * | Out | Result of request |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0220; ARCH_SW_DIA_0221; ARCH_SW_DIA_0222; ARCH_SW_DIA_0223; ARCH_SW_DIA_0224 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0221 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9550 |
| ARCH_SW_DIA_0222 | Service shall be available in the following forms of Authentication: Supplier, Development ENHANCED, Production, After-Sales ENHANCED, After-Sales |  | DAI_EXT_TF_E_9910 |
| ARCH_SW_DIA_0223 | Successful Authentication is a pre-condition for running this Service |  | DAI_EXT_TF_E_9548 |
| ARCH_SW_DIA_0224 | Result can have the following values: 0x00, 0x01 and 0x02 |  | DAI_EXT_TF_E_9566; DAI_EXT_TF_E_9566; DAI_EXT_TF_E_9567; DAI_EXT_TF_E_9568 |

## DIA_runRid0302_TriggerPresafeDisplayRequestStart

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runRid0302_TriggerPresafeDisplayRequestStart(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StartDataOut_Routine_Trigger_Presafe_Display_Request_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_StartDataOut_Routine_Trigger_Presafe_Display_Request_Start_ResultType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Result, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function starts the Request of TriggerPresafeDisplay |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| routineInfo | Uint8 * | Out | Routine status |
| Result | Uint8 * | Out | Result of request |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0220; ARCH_SW_DIA_0225; ARCH_SW_DIA_0226; ARCH_SW_DIA_0227; ARCH_SW_DIA_0228 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0225 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9550 |
| ARCH_SW_DIA_0226 | Service shall be available in the following forms of Authentication: Supplier, Development ENHANCED, Production, After-Sales ENHANCED, After-Sales |  | DAI_EXT_TF_E_9910 |
| ARCH_SW_DIA_0227 | Successful Authentication is a pre-condition for running this Service |  | DAI_EXT_TF_E_9548 |
| ARCH_SW_DIA_0228 | Result can have the following values: 0x00, 0x01 and 0x02 |  | DAI_EXT_TF_E_9566; DAI_EXT_TF_E_9567; DAI_EXT_TF_E_9568 |

## DIA_runRid0302_TriggerPresafeDisplayRequestStop

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runRid0302_TriggerPresafeDisplayRequestStop(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StopDataOut_Routine_Trigger_Presafe_Display_Request_Start_routineInfoType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) routineInfo, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function stops the Request of TriggerPresafeDisplay |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| routineInfo | Uint8 * | Out | Routine status |
| Result | Uint8 * | Out | Result of request |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0220; ARCH_SW_DIA_0229; ARCH_SW_DIA_0230; ARCH_SW_DIA_0231; ARCH_SW_DIA_0232 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0229 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9550 |
| ARCH_SW_DIA_0230 | Service shall be available in the following forms of Authentication: Supplier, Development ENHANCED, Production, After-Sales ENHANCED, After-Sales |  | DAI_EXT_TF_E_9910 |
| ARCH_SW_DIA_0231 | Successful Authentication is a pre-condition for running this Service |  | DAI_EXT_TF_E_9548 |
| ARCH_SW_DIA_0232 | Result can have the following values: 0x00, 0x01 and 0x02 |  | DAI_EXT_TF_E_9566; DAI_EXT_TF_E_9567; DAI_EXT_TF_E_9568 |

## DIA_runDid0142_AvailabilityDataRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0142_AvailabilityDataRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the AvailabilityData Service |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Data | Uint8 * | Out | Stores Aec Group Status |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0235; ARCH_SW_DIA_0236; ARCH_SW_DIA_0237; ARCH_SW_DIA_0238; ARCH_SW_DIA_0336; ARCH_SW_DIA_0337 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0236 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9745 |
| ARCH_SW_DIA_0237 | Service shall be available in the following forms of Authentication: Development ENHANCED, Production After-Sales ENHANCED, After-Sales After-Sales BASIC, Internal Diagnostic Test Tool, ePTI Test Tool |  | DAI_EXT_TF_E_9945 |
| ARCH_SW_DIA_0238 | Result can have the following values: 0x00, 0x01 and 0x02 |  | DAI_EXT_TF_E_9761 |
| ARCH_SW_DIA_0337 | Positive Response: Green – no DTC or DTC without Warning indicator. Yellow – is not used Red – Display message is requested -> DTC with Warning indicator is set |  | DAI_EXT_TF_E_10085 |

## DIA_runDid0241_PRE_SAFE_Recorder_1_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0241_PRE_SAFE_Recorder_1_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function reads the Data Length from the NVP Block of the Pre-Safe Recorder 1 Mechanism |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| DataLength | Uint22 * | Out | Size of the NvP Block |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0240; ARCH_SW_DIA_0241; ARCH_SW_DIA_0242; ARCH_SW_DIA_0243 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0241 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9747 |
| ARCH_SW_DIA_0242 | Service shall be available in the following forms of Authentication: Development ENHANCED, Production After-Sales ENHANCED, After-Sales After-Sales BASIC, Internal Diagnostic Test Tool, ePTI Test Tool |  | DAI_EXT_TF_E_9947 |
| ARCH_SW_DIA_0243 | Response data shall be 22 bytes long |  | DAI_EXT_TF_E_9838 |

## DIA_runDid0241_PRE_SAFE_Recorder_1_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0241_PRE_SAFE_Recorder_1_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function reads the Condition Check for the Pre-Safe Recorder 1 Mechanism |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| routineInfo | Uint8 * | Out | Routine status |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0240; ARCH_SW_DIA_0244; ARCH_SW_DIA_0245 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0244 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9747 |
| ARCH_SW_DIA_0245 | Service shall be available in the following forms of Authentication: Development ENHANCED, Production After-Sales ENHANCED, After-Sales After-Sales BASIC, Internal Diagnostic Test Tool, ePTI Test Tool |  | DAI_EXT_TF_E_9947 |

## DIA_runDid0241_PRE_SAFE_Recorder_1_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0241_PRE_SAFE_Recorder_1_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the Data from NVP Block for the Pre-Safe Recorder 1 Mechanism |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Data | Uint8 * | Out | NvP Block Data |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0240; ARCH_SW_DIA_0246; ARCH_SW_DIA_0247 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0246 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9747 |
| ARCH_SW_DIA_0247 | Service shall be available in the following forms of Authentication: Development ENHANCED, Production After-Sales ENHANCED, After-Sales After-Sales BASIC, Internal Diagnostic Test Tool, ePTI Test Tool |  | DAI_EXT_TF_E_9947 |

## DIA_runDid0241_PRE_SAFE_Recorder_1_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_Ret6urnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0241_PRE_SAFE_Recorder_1_WriteData(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, uint16 DataLength, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function writes the Data from NVP Block for the Pre-Safe Recorder 1 Mechanism |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 * | Out | NvP Block Data |
| DataLength | Uint22 * | Out | Size of the NvP Block Data |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0240; ARCH_SW_DIA_0248; ARCH_SW_DIA_0249; ARCH_SW_DIA_1026; ARCH_SW_DIA_1027 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0248 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9747 |
| ARCH_SW_DIA_0249 | Service shall be available in the following forms of Authentication: Development ENHANCED, Production After-Sales ENHANCED, After-Sales After-Sales BASIC, Internal Diagnostic Test Tool, ePTI Test Tool |  | DAI_EXT_TF_E_9947 |
| ARCH_SW_DIA_1026 | The "Pre-Safe Recorder 1" data shall be written in NvM after: ⦁Profile End ⦁Profile Inhibition ⦁Profile Abortion |  | DAI_EXT_TF_E_11509 |
| ARCH_SW_DIA_1027 | The "Pre-Safe Recorder 1" data shall be kept over a Hard Reset/ Power Off-On |  | DAI_EXT_TF_E_11510 |

## DIA_runDid0242_PRE_SAFE_Recorder_2_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0242_PRE_SAFE_Recorder_2_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function reads the Data Length from the NVP Block of the Pre-Safe Recorder 2 Mechanism |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| DataLength | Uint22 * | Out | Size of the NvP Block |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0250; ARCH_SW_DIA_0251; ARCH_SW_DIA_0252; ARCH_SW_DIA_0253 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0251 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9748 |
| ARCH_SW_DIA_0252 | Service shall be available in the following forms of Authentication: Development ENHANCED, Production After-Sales ENHANCED, After-Sales After-Sales BASIC, Internal Diagnostic Test Tool, ePTI Test Tool |  | DAI_EXT_TF_E_9948 |
| ARCH_SW_DIA_0253 | Response data shall be 22 bytes long |  | DAI_EXT_TF_E_9871 |

## DIA_runDid0242_PRE_SAFE_Recorder_2_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0242_PRE_SAFE_Recorder_2_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function reads the Condition Check for the Pre-Safe Recorder 2 Mechanism |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| routineInfo | Uint8 * | Out | Routine status |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0250; ARCH_SW_DIA_0254; ARCH_SW_DIA_0255 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0254 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9748 |
| ARCH_SW_DIA_0255 | Service shall be available in the following forms of Authentication: Development ENHANCED, Production After-Sales ENHANCED, After-Sales After-Sales BASIC, Internal Diagnostic Test Tool, ePTI Test Tool |  | DAI_EXT_TF_E_9948 |

## DIA_runDid0242_PRE_SAFE_Recorder_2_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0242_PRE_SAFE_Recorder_2_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the Data from NVP Block for the Pre-Safe Recorder 2 Mechanism |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Data | Uint8 * | Out | NvP Block Data |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0250; ARCH_SW_DIA_0256; ARCH_SW_DIA_0257 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0256 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9748 |
| ARCH_SW_DIA_0257 | Service shall be available in the following forms of Authentication: Development ENHANCED, Production After-Sales ENHANCED, After-Sales After-Sales BASIC, Internal Diagnostic Test Tool, ePTI Test Tool |  | DAI_EXT_TF_E_9948 |

## DIA_runDid0242_PRE_SAFE_Recorder_2_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0242_PRE_SAFE_Recorder_2_WriteData(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, uint16 DataLength, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function writes the Data from NVP Block for the Pre-Safe Recorder 2 Mechanism |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 * | Out | NvP Block Data |
| DataLength | Uint22 * | Out | Size of the NvP Block Data |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0250; ARCH_SW_DIA_0258; ARCH_SW_DIA_0259; ARCH_SW_DIA_1028; ARCH_SW_DIA_1029 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0258 | Service shall be available in Default session, Extended session Update session and Development session |  | DAI_EXT_TF_E_9748 |
| ARCH_SW_DIA_0259 | Service shall be available in the following forms of Authentication: Development ENHANCED, Production After-Sales ENHANCED, After-Sales After-Sales BASIC, Internal Diagnostic Test Tool, ePTI Test Tool |  | DAI_EXT_TF_E_9948 |
| ARCH_SW_DIA_1028 | The "Pre-Safe Recorder 2" data shall be written in NvM after: ⦁Profile End ⦁Profile Inhibition ⦁Profile Abortion |  | DAI_EXT_TF_E_11512 |
| ARCH_SW_DIA_1029 | The "Pre-Safe Recorder 2" data shall be kept over a Hard Reset/ Power Off-On |  | DAI_EXT_TF_E_11513 |

## RoELight_DTCStatusChanged_Callback

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) RoELight_DTCStatusChanged_Callback(uint32 DTC, Dem_UdsStatusByteType DTCStatusOld, Dem_UdsStatusByteType DTCStatusNew) |  |  |  |
| Object |  |  |  |
| Function implements the RoE Light DTC Status Callback |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| DTC | Uint32 * | Out | DTC |
| DTCStatusOld | Dem_UdsStatusByteType | Out | DTC Status |
| DTCStatusNew | Dem_UdsStatusByteType | Out | DTC Status |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0210; ARCH_SW_DIA_0211; ARCH_SW_DIA_0212; ARCH_SW_DIA_0213 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0211 | This frame shall be identified with the address: Left side: 0x18E1A400 – ROE_RBTM_FL_ST3 Right side: 0x18E1A500 – ROE_RBTM_FR_ST3 |  | DAI_EXT_TF_B_2051 |
| ARCH_SW_DIA_0212 | ROE_0_RBTM_xy_ST3 (Byte 1) signal shall be transmitted with the value 0x01 - DTC_STATUS_CHANGE if DTC status has been changed. Byte 2 of the ROE_RBTM_xy_ST3 frame shall contain the availability mask of the DTC. Bytes 3-5 of the ROE_RBTM_xy_ST3 frame shall contain the DTC code. Byte 6 of the ROE_RBTM_xy_ST3 frame shall contain the status of the DTC. Bytes 7-8 of the ROE_RBTM_xy_ST3 frame shall be 0x0000. |  | DAI_EXT_TF_B_2076 |
| ARCH_SW_DIA_0213 | Default value: 0 |  | DAI_EXT_TF_B_2058 |

## DIA_runClearDiagRequestCycle

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| void DIA_runClearDiagRequestCycle(void) |  |  |  |
| Object |  |  |  |
| Function implements the clearing of the DiagRequestCycle |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Rte_IrvWrite_DIA_runClearDiagRequestCycle_u8DiagRequestedCycle | Calls an Rte_Write to clear the Requested Cycle |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0260 |  |  |  |

## DIA_runGetDiagRequestCycle

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| void DIA_runGetDiagRequestCycle(u8CycleNumberType * u8DiagRequestCycle) |  |  |  |
| Object |  |  |  |
| Function acts like a getter function of the DiagRequestCycle |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| u8DiagRequestCycle | u8CycleNumberType * | Out | Requested Cycle |
| Returned value |  |  |  |
| Name | Description |  |  |
| u8DiagRequestCycle | Performs an Rte_Read of the DiagRequestCycle |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0261 |  |  |  |

## DIA_cyclicProcessingTask

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(void, DIA_AC_DiagOnCAN_CODE) DIA_cyclicProcessingTask(void) |  |  |  |
| Object |  |  |  |
| Function implements the cyclic processing task of the Tensioning Routines |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| routineStatus | Handles routineStatuses according to DIA’s task cyclicality |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0262 |  |  |  |

## ResetServiceExecution

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(void, Ct_MBBL_ResetManager_CODE) ResetServiceExecution(void) |  |  |  |
| Object |  |  |  |
| Function implements the ECU Reset States using DcmEcuReset APIs |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| routineStatus | Handles routineStatuses according to DIA’s task cyclicality |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0265; ARCH_SW_DIA_0266; ARCH_SW_DIA_0267; ARCH_SW_DIA_0268; ARCH_SW_DIA_0269; ARCH_SW_DIA_0270; ARCH_SW_DIA_0271 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0266 | The ECUReset service shall be available in the following sessions: |  | ALV_EXT_TF_E_7163 |
| ARCH_SW_DIA_0267 | The ECUReset service shall be available under the following forms of Authentication: Supplier Development ENHANCED Production After-Sales ENHANCED After-Sales |  | ALV_EXT_TF_E_6147 |
| ARCH_SW_DIA_0268 | ResetType of the Reset service shall be: 0x01 – Hardware Reset |  | ALV_EXT_TF_E_6015 |
| ARCH_SW_DIA_0269 | ResetType of the Reset service shall be: 0x03 – Software Reset |  | ALV_EXT_TF_E_7410 |
| ARCH_SW_DIA_0270 | Bytes #2: Reset type should be an echo of the “ResetType” parameter in the request message. |  | ALV_EXT_TF_E_104 |
| ARCH_SW_DIA_0271 | After a successful ECU Reset, ECU shall be in DefaultSession. |  | ALV_EXT_TF_E_114 |

## Dcm_Svc14Handler

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DCM_LOCAL_INLINE FUNC(Std_ReturnType, DCM_CODE) Dcm_Svc14Handler(Dcm_ContextPtrType pContext, Dcm_OpStatusType opStatus, Dcm_MsgContextPtrType pMsgContext, Dcm_NegativeResponseCodePtrType ErrorCode, Dcm_Svc14RepeaterProxyContextPtrType pRepContext) |  |  |  |
| Object |  |  |  |
| Function implements the clearing of all Diagnostic Information related to a specified DTC Identifier Mask |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| pContext | Dcm_ContextPtrType | Out |  |
| opStatus | Dcm_OpStatusType | Out |  |
| pMsgContext | Dcm_MsgContextPtrType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodePtrType | Out |  |
| pRepContext | Dcm_Svc14RepeaterProxyContextPtrType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0275; ARCH_SW_DIA_0276; ARCH_SW_DIA_0277; ARCH_SW_DIA_0278; ARCH_SW_DIA_0279; ARCH_SW_DIA_0280 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0276 | The ClearDiagnosticInformation service shall be available in the following sessions: Default Session (0x01) Extended Session (0x03) Supplier Session/Development Session (0x7E) Update Session (0x42) |  | ALV_EXT_TF_E_5931 |
| ARCH_SW_DIA_0277 | This service shall be available with or without any form of Authentication. |  | ALV_EXT_TF_E_124 |
| ARCH_SW_DIA_0278 | The ECU shall send a Positive Response when the ClearDiagnosticInformation service is completely processed Explanation: ClearDiagnosticInformation service is considered completely processed when the data used by the ReadDiagnosticInformation (0x19) are cleared |  | ALV_EXT_TF_E_139 |
| ARCH_SW_DIA_0279 | The ECU shall send a Positive Response even if no DTCs are stored in the server’s memory |  | ALV_EXT_TF_E_140 |
| ARCH_SW_DIA_0280 | Each bit mask corresponds to the following DTC Groups: 0x000000 = GROUP_EMISSION_RELATED 0x000002 = GROUP_POWERTRAIN 0x000004 = GROUP_CHASSIS 0x000006 = GROUP_BODY 0x000008 = GROUP_NETWORK_COM 0xA00000 = GROUP_SBE 0XFFFFFF = ALL DTC GROUP |  | ALV_EXT_TF_E_6037; ALV_EXT_TF_E_6038; ALV_EXT_TF_E_6039; ALV_EXT_TF_E_6040; ALV_EXT_TF_E_6041; ALV_EXT_TF_E_6042; ALV_EXT_TF_E_6043 |

## Dcm_Svc19Handler

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DCM_LOCAL_INLINE FUNC(Std_ReturnType, DCM_CODE) Dcm_Svc19Handler(Dcm_ContextPtrType pContext, Dcm_OpStatusType opStatus, Dcm_MsgContextPtrType pMsgContext, Dcm_NegativeResponseCodePtrType ErrorCode, Dcm_Svc19RepeaterProxyContextPtrType pRepContext) |  |  |  |
| Object |  |  |  |
| Function reads the DTC Information reported by different types of DIDs from BSW |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| pContext | Dcm_ContextPtrType | Out |  |
| opStatus | Dcm_OpStatusType | Out |  |
| pMsgContext | Dcm_MsgContextPtrType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodePtrType | Out |  |
| pRepContext | Dcm_Svc19RepeaterProxyContextPtrType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0281; ARCH_SW_DIA_0282; ARCH_SW_DIA_0283; ARCH_SW_DIA_0284; ARCH_SW_DIA_0525; ARCH_SW_DIA_0526; ARCH_SW_DIA_0527; ARCH_SW_DIA_0528; ARCH_SW_DIA_0530; ARCH_SW_DIA_0531; ARCH_SW_DIA_0532; ARCH_SW_DIA_0775; ARCH_SW_DIA_0776; ARCH_SW_DIA_0777; ARCH_SW_DIA_0778; ARCH_SW_DIA_0779; ARCH_SW_DIA_0780; ARCH_SW_DIA_0781; ARCH_SW_DIA_0782; ARCH_SW_DIA_0783; ARCH_SW_DIA_0785; ARCH_SW_DIA_0786; ARCH_SW_DIA_0787; ARCH_SW_DIA_0788; ARCH_SW_DIA_0789; ARCH_SW_DIA_0790; ARCH_SW_DIA_0791; ARCH_SW_DIA_0792; ARCH_SW_DIA_0793; ARCH_SW_DIA_0794; ARCH_SW_DIA_0795; ARCH_SW_DIA_0796; ARCH_SW_DIA_0800; ARCH_SW_DIA_0801; ARCH_SW_DIA_0802; ARCH_SW_DIA_0803 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0282 | The ReportDTCByStatusMask shall be available in the following sessions: Update session |  | DAI_EXT_TF_E_198 |
| ARCH_SW_DIA_0283 | The byte 3 shall be reserved for the DTCStatusAvailabilityMask of the ReadDiagnosticInformation service. |  | ALV_EXT_TF_E_207 |
| ARCH_SW_DIA_0284 | DTCStatusAvailabilityMask shall be 0x39. REVAS supported status bits are the following ones: - Test Failed (bit 0) - Confirmed DTC (bit 3) - Test Not Completed Sinœ Last Clear (bit 4) - Test Failed this Operation Cycle (bit 5) |  | DAI_EXT_TF_E_10103 |
| ARCH_SW_DIA_0526 | The byte 1 shall be reserved for the SID-RQ of the ReadDiagnosticInformation service: ⦁ The value 0x19 shall correspond to the SID-RQ of the ReadDiagnosticInformation service. |  | ALV_EXT_TF_E_173 |
| ARCH_SW_DIA_0527 | The byte 2 shall be reserved for the ReportType of the ReadDiagnosticInformation service: ⦁ The value 0x01 shall correspond to the ReportType: ReportNumberOfDTCByStatusMask |  | ALV_EXT_TF_E_6091 |
| ARCH_SW_DIA_0528 | The byte 3 shall be reserved for the DTCStatusMask of the ReadDiagnosticInformation service: ⦁The bit 0 shall correspond to 'Test failed' ⦁The bit 1 shall correspond to 'Test failed this Operation cycle' ⦁The bit 2 shall correspond to 'Pending DTC' ⦁The bit 3 shall correspond to 'Confirmed DTC' ⦁The bit 4 shall correspond to 'Test not completed since last clear' ⦁The bit 5 shall correspond to 'Test failed since last clear' ⦁The bit 6 shall correspond to 'Test not completed this operation cycle' ⦁The bit 7 shall correspond to 'Warning indicator requested' |  | ALV_EXT_TF_E_176 |
| ARCH_SW_DIA_0531 | The byte 1 shall be reserved for the SID-RQ of the ReadDiagnosticInformation service: ⦁The value 0x19 shall correspond to the SID-RQ of theReadDiagnosticInformation service |  | ALV_EXT_TF_E_324 |
| ARCH_SW_DIA_0532 | The byte 2 shall be reserved for the ReportType of the ReadDiagnosticInformation service: ⦁The value 0x0A shall correspond to the ReportType: ReportSupportedDTC |  | ALV_EXT_TF_E_6151 |
| ARCH_SW_DIA_0776 | The ReportDTCSnapshotRecordByDTCNumber shall be available in the following diagnostic sessions: ⦁Default Session ⦁Extended Session ⦁Development Session Service shall be available under any form of Authentication |  | DAI_EXT_TF_E_10357 |
| ARCH_SW_DIA_0777 | ⦁The value 0x19 shall correspond to the SID-RQ of theReadDiagnosticInformation service. ⦁The value 0x04 shall correspond to the ReportType: ReportDTCSnapshotRecordByDTCNumber |  | DAI_EXT_TF_E_10361 |
| ARCH_SW_DIA_0778 | The bytes #3...#5 shall be reserved for the DTCRecord of the ReadDiagnosticInformation service. |  | DAI_EXT_TF_E_10366 |
| ARCH_SW_DIA_0779 | The byte #6 shall be reserved for the DTCSnapshotRecordNumber of the ReadDiagnosticInformation service. 0x10 = First Occurence (triggered on CONFIRMED) 0x20 = Last Occurence (triggered on TEST_FAILED) 0xFF = All Snapshot records |  | DAI_EXT_TF_E_10367 |
| ARCH_SW_DIA_0780 | The byte #6 shall be reserved for the StatusOfDTC of the ReadDiagnosticInformation service. |  | DAI_EXT_TF_E_10375 |
| ARCH_SW_DIA_0781 | The byte #7 shall be reserved for the DTCSnapshotRecordNumber |  | DAI_EXT_TF_E_11195 |
| ARCH_SW_DIA_0782 | The byte #8 shall be reserved for DTCSnapshotRecordNumberOfIdentifiers |  | DAI_EXT_TF_E_11200 |
| ARCH_SW_DIA_0783 | The bytes #9 ... #9 + length(DTCSnapshotRecord #x) -1 shall be reserved for DTCSnapshotRecord Note: DTCSnapshotRecord contains several dataIdentifiers and their returned data. Each DTC has its specific dataidentifiers which can be found in CDD. x= DTCSnapshotRecordNumber |  | DAI_EXT_TF_E_11367 |
| ARCH_SW_DIA_0786 | The ReportDTCExtendedDataRecordByDTCNumber shall be available in the following diagnostic sessions: ⦁Default Session ⦁Extended Session ⦁Development Session Service shall be available under any form of Authentication |  | DAI_EXT_TF_E_10401 |
| ARCH_SW_DIA_0787 | ⦁ The value 0x19 (byte1) shall correspond to the SID-RQ of theReadDiagnosticInformation service. ⦁ The value 0x06 (byte2) shall correspond to the ReportType: ReportDTCExtendedDataRecordByDTCNumber |  | DAI_EXT_TF_E_10408 |
| ARCH_SW_DIA_0788 | The bytes #3..#5 shall be reserved for the DTCRecord of the ReadDiagnosticInformation service. |  | DAI_EXT_TF_E_10405 |
| ARCH_SW_DIA_0789 | The byte #6 shall be reserved for the DTCExtendedDataRecordNumber of the ReadDiagnosticInformation service. 0x01 = StandardEnvironmentalData 0xFF = All environmental data |  | DAI_EXT_TF_E_10409 |
| ARCH_SW_DIA_0790 | The bytes #3...#5 shall be reserved for the DTCRecord of the ReadDiagnosticInformation service. |  | DAI_EXT_TF_E_10413 |
| ARCH_SW_DIA_0791 | The byte #6 shall be reserved for the StatusOfDTC of the ReadDiagnosticInformation service. |  | DAI_EXT_TF_E_10417 |
| ARCH_SW_DIA_0792 | The byte #7 shall be reserved for the ExtDataRecordlist of the ReadDiagnosticInformation service. byte #7 shall be set to 0x01 because the list of ExtendedDataRecordNumber contains only one element: StandardEnvironmentalData |  | DAI_EXT_TF_E_10414 |
| ARCH_SW_DIA_0793 | ⦁Standardized DTCExtendedData byte #8 shall correspond to 'Occurrence Flag' 0x00 - Occurence 0x01 - Fault |  | DAI_EXT_TF_E_11191 |
| ARCH_SW_DIA_0794 | ⦁Standardized DTCExtendedData byte #9 shall correspond to 'External tester present Flag' 0x00 - Not present 0x01 - Present |  | DAI_EXT_TF_E_11192 |
| ARCH_SW_DIA_0795 | ⦁Standardized DTCExtendedData byte #10 shall correspond to 'Frequency Counter' |  | DAI_EXT_TF_E_11193 |
| ARCH_SW_DIA_0796 | ⦁Standardized DTCExtendedData byte #11 shall correspond to 'Ignition Cycle Counter' |  | DAI_EXT_TF_E_11194 |
| ARCH_SW_DIA_0801 | ⦁The value 0x14 shall correspond to the ReportType: ReportDTCFaultDetectionCounter |  | DAI_EXT_TF_E_10448 |
| ARCH_SW_DIA_0802 | The bytes #3..#5 shall be reserved for the DTCRecord of the ReadDiagnosticInformation service. |  | DAI_EXT_TF_E_10487 |
| ARCH_SW_DIA_0803 | The byte #6 shall be reserved for the DTCFaultDetectionCounter of the ReadDiagnosticInformation service. (The DTCFaultDetectionCounter reports the number of fault detection counts of a DTC.) See SYS_DAI_MF09_692 for more details |  | DAI_EXT_TF_E_10488 |
| ARCH_SW_DIA_0826 | Each Root Cause will be reported on 1 byte and will have a dependency with AEC Groups and DTCs: ⦁1 byte: AEC ID (# 0x00 - These DTCs doesn't have a specific root cause besided the description of the DTC) |  | DAI_EXT_TF_E_11536 |

## DIA_runDidFD01_SWVersionConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD01_SWVersionConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function implements Service 0x22 for DID FD01 |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0285 |  |  |  |

## DIA_runDidFD01_SWVersionRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD01_SWVersionRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function implements Service 0x22 for DID FD01 |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0285; ARCH_SW_DIA_0286; ARCH_SW_DIA_0287; ARCH_SW_DIA_0288; ARCH_SW_DIA_0289; ARCH_SW_DIA_0290; ARCH_SW_DIA_0291 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0286 | The Read SW Version shall be available in the following sessions: Supplier Session/Development Session (0x7E) |  | ALV_EXT_TF_E_652 |
| ARCH_SW_DIA_0287 | Service Read SW Version shall be processed with or without Any form of Authentication. |  | ALV_EXT_TF_E_6260 |
| ARCH_SW_DIA_0288 | DataIdentifier of SW Version shall be 0xFD01 |  | ALV_EXT_TF_E_655 |
| ARCH_SW_DIA_0289 | Read SW Version service Response DataRecord shall have 12 bytes length |  | ALV_EXT_TF_E_661 |
| ARCH_SW_DIA_0290 | DataRecord: SW Version Bytes “SN1” to “SN12” are coded in ASCII and shall reflect the SW Identification (letters and digits allowed) |  | ALV_EXT_TF_E_7065 |
| ARCH_SW_DIA_0291 | SNx shall be ordered from MSB to LSB, ASCI coded i.e. bytes #4 to #15 form a string in that order that can be read directly. It will be formatted as follow: ⦁ AABCCZWH.YYY Where: AA-reflects product code, e.g. ER for ECU PP (a.k.a. ERR) B -reflects BM identification according to current software version, e.g. 0 for PP4G Extended CC-reflects customer code, e.g. 18 for DAI Group Z-reflects software type code, 6 for DAI MMA W-Digit subject to change in case the SW release have been modified between SW Freeze and M2 Milestone. H-defines a FLASHED software version YYY-reflects the software version over 3 digits |  | DAI_EXT_TF_E_10285 |

## Dcm_Service27Processor

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DCM_CALLOUT_CODE) Dcm_Service27Processor(Dcm_ContextPtrType pContext, Dcm_OpStatusType opStatus, Dcm_MsgContextPtrType pMsgContext, Dcm_NegativeResponseCodePtrType ErrorCode) |  |  |  |
| Object |  |  |  |
| Function calculates the Security Access for generated Seed and comparison Key Handling of Data |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| pContext | Dcm_ContextPtrType | Out |  |
| opStatus | Dcm_OpStatusType | Out |  |
| pMsgContext | Dcm_MsgContextPtrType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodePtrType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0295; ARCH_SW_DIA_0296; ARCH_SW_DIA_0297; ARCH_SW_DIA_0298; ARCH_SW_DIA_0299; ARCH_SW_DIA_0300; ARCH_SW_DIA_0301; ARCH_SW_DIA_0302; ARCH_SW_DIA_0303 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0296 | The Security Access service shall only be available in the following sessions: Programming Session (0x02) |  | ALV_EXT_TF_E_1467 |
| ARCH_SW_DIA_0297 | Security Access service shall be available under the following forms of Authentication: |  | DAI_EXT_TF_E_10256 |
| ARCH_SW_DIA_0298 | SecurityAccessType of the SecurityAccess service shall be 0x11 for Get Programming Seed. |  | ALV_EXT_TF_E_6033 |
| ARCH_SW_DIA_0299 | Bytes #2-#3: SecurityAccessType shall be: 0x11 |  | ALV_EXT_TF_E_6055 |
| ARCH_SW_DIA_0300 | Bytes #3-#34: Shall contain SecuritySeed |  | DAI_EXT_TF_E_10286 |
| ARCH_SW_DIA_0301 | SecurityAccessType of the SecurityAccess service shall be 0x12 for Get Programming Key. |  | ALV_EXT_TF_E_7618 |
| ARCH_SW_DIA_0302 | The bytes from 3 to 66 shall be reserved for the SecurityKey of the SecurityAccess service. |  | ALV_EXT_TF_E_7619 |
| ARCH_SW_DIA_0303 | Bytes #2-#3: SecurityAccessType shall be: 0x12 |  | ALV_EXT_TF_E_7627 |

## DIA_runDidFD2F_EcuLocationWrite

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD2F_EcuLocationWrite(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType_1 OpStatus, P2VAR(Dcm_NegativeResponseCodeType_1, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function that writes EcuLocation inside the NvM (NVP_BLOCK_ID_ECU_LOCATION_RamBlockData) |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 | Out |  |
| OpStatus | Dcm_OpStatusType_1 | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType_1 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0305; ARCH_SW_DIA_0306; ARCH_SW_DIA_0307; ARCH_SW_DIA_0308; ARCH_SW_DIA_0309; ARCH_SW_DIA_0310; ARCH_SW_DIA_0311 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0306 | The Write ECU Location service shall be available in the following sessions: Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10086 |
| ARCH_SW_DIA_0307 | Service Write ECU Location shall be available under the following forms of Authentication: Supplier Development ENHANCED Production |  | DAI_EXT_TF_E_10087 |
| ARCH_SW_DIA_0308 | Data Identifier of Write ECU Location shall be: 0xFD2F |  | DAI_EXT_TF_E_10089 |
| ARCH_SW_DIA_0309 | The SIDE parameter shall be 1 byte and shall correspond to the side the ECU shall be configured to: 0x99 – RIGHT SIDE 0x9C – LEFT SIDE |  | DAI_EXT_TF_E_10090 |
| ARCH_SW_DIA_0310 | Bytes #2-#3: DataIdentifier of the Write ECU location service shall be: 0xFD2F |  | ALV_EXT_TF_E_6735 |
| ARCH_SW_DIA_0311 | If positive answer is sent, then value of SIDE shall be taken into account after reset. |  | DAI_EXT_TF_E_10091 |

## DIA_runDidFD31_HBCalibrationWrite

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD31_HBCalibrationWrite(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function that writes HB Calibration Parameters inside NvM (NVP_BLOCK_ID_HB_CALIBRATION_RamBlockData) |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 | Out |  |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0315; ARCH_SW_DIA_0316; ARCH_SW_DIA_0317; ARCH_SW_DIA_0318; ARCH_SW_DIA_0319; ARCH_SW_DIA_0320; ARCH_SW_DIA_0321 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0316 | The ECU calibration parameters shall be modified in non-volatile and RAM memories. It means the changes will immediately affect the current SW session (ECU reset is not necessary to take into account the modifications). |  | ALV_EXT_TF_E_1833 |
| ARCH_SW_DIA_0317 | The Write ECU Calibration Parameters service shall be available in the following sessions: Supplier Session/Development Session (0x7E) |  | ALV_EXT_TF_E_10092 |
| ARCH_SW_DIA_0318 | Service Write ECU Location shall be available under the following forms of Authentication: Supplier Development ENHANCED Production |  | ALV_EXT_TF_E_10093 |
| ARCH_SW_DIA_0319 | Data Identifier of Write ECU Calibration Parameters shall be: 0xFD31 |  | ALV_EXT_TF_E_10995 |
| ARCH_SW_DIA_0320 | Write ECU Calibration Parameters Request data shall be 40 bytes long. |  | ALV_EXT_TF_E_10996 |
| ARCH_SW_DIA_0321 | Bytes #2-#3: DataIdentifier of the Write ECU Calibration Parameters shall be: 0xFD31 |  | ALV_EXT_TF_E_1853 |

## DataServices_Current_Operating_Time_Read_Current_Operating_Time_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Current_Operating_Time_Read_Current_Operating_Time_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of the Current Operating Time |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0325 |  |  |  |

## DataServices_Current_Operating_Time_Read_Current_Operating_Time_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Current_Operating_Time_Read_Current_Operating_Time_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Current Operating Time using the API provided by StbM - StbM_GetCurrentTime |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0325; ARCH_SW_DIA_0326; ARCH_SW_DIA_0327; ARCH_SW_DIA_0467; ARCH_SW_DIA_0468; ARCH_SW_DIA_0469; ARCH_SW_DIA_0470 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0467 | DataIdentifier shall be 0x0133 |  | DAI_EXT_TF_E_9735 |
| ARCH_SW_DIA_0468 | The DID shall be available in the following sessions: ⦁Default Session ⦁Programming ⦁Extended session ⦁Update Session ⦁Development Session |  | DAI_EXT_TF_E_9744 |
| ARCH_SW_DIA_0469 | This service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁Internal Diagnostic Test Tool |  | DAI_EXT_TF_E_9944 |
| ARCH_SW_DIA_0326 | Response data shall be 5 bytes long. |  | DAI_EXT_TF_E_9757 |
| ARCH_SW_DIA_0470 | Bytes 0-3 of the positive response shall represent the Current Operating Time. |  | DAI_EXT_TF_E_9758 |
| ARCH_SW_DIA_0327 | Byte 4 of the positive response shall represent the Operating Time Status, defined as follows: ⦁ Bits 0 – 3: converted value based on timeBaseStatus ⦁ Bits 4 – 7: value of userByte0 |  | DAI_EXT_TF_E_9759 |

## DIA_runDid00140_Obsolescence_Data_Current_Value_ConditionCheckRead

### DIA_runDid00140_Obsolescence_Data_BSR_Current_Value_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_BSR_Current_Value_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Current_Value_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Current_Value_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Current_Value_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Current_Value_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Current_Value_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Current_Value_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

## DIA_runDid00140_Obsolescence_Data_Threshold _ConditionCheckRead

### DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Green_To_Yellow_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Green_To_Yellow_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Yellow_To_Red_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Yellow_To_Red_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Green_To_Yellow_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Green_To_Yellow_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Yellow_To_Red_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Yellow_To_Red_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Green_To_Yellow_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Green_To_Yellow_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Yellow_To_Red_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Yellow_To_Red_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Green_To_Yellow_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Green_To_Yellow_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Yellow_To_Red_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Yellow_To_Red_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Conditions of the Obsolescence Data DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330 |  |  |  |

## DIA_runDid00140_Obsolescence_Data_Current_Value_Read

### DIA_runDid00140_Obsolescence_Data_BSR_Current_Value_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_BSR_Current_Value_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from BSR |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0331; ARCH_SW_DIA_0332; ARCH_SW_DIA_0333 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Current_Value_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Current_Value_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from Haptic_Warning |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0331; ARCH_SW_DIA_0332; ARCH_SW_DIA_0333 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Current_Value_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Current_Value_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from 4 different Block Numbers PRE_CRASH |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0331; ARCH_SW_DIA_0332; ARCH_SW_DIA_0333 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Current_Value_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Current_Value_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from PRE_SAFE_LVL_4_5 |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0331; ARCH_SW_DIA_0332; ARCH_SW_DIA_0333 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0331 | The Read Obsolescence Data service shall be available in the following sessions: Default Session (0x01) Programming (0x02) Extended Session (0x03) Supplier Session/Development Session (0x7E) Update Session (0x42) |  | DAI_EXT_TF_E_10179 |
| ARCH_SW_DIA_0332 | Read Obsolescence Data service shall be available under the following forms of Authentication: Supplier Development ENHANCED Production After-Sales ENHANCED After-Sales After-Sales BASIC Internal Diagnostic Test Tool ePTI Test Tool |  | DAI_EXT_TF_E_10180 |
| ARCH_SW_DIA_0333 | DataIdentifier of the Read Obsolescence Data shall be: 0x0140 |  | DAI_EXT_TF_E_10184 |

## DIA_runDid00140_Obsolescence_Data_Threshold_Read

### DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Green_To_Yellow_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Green_To_Yellow_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from BSR |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0334; ARCH_SW_DIA_0335 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Yellow_To_Red_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_BSR_Threshold_Yellow_To_Red_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from BSR |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0334; ARCH_SW_DIA_0335 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Green_To_Yellow_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Green_To_Yellow_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from Haptic_Warning |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0334; ARCH_SW_DIA_0335 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Yellow_To_Red_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_Haptic_Warning_Treshold_Yellow_To_Red_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from Haptic_Warning |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0334; ARCH_SW_DIA_0335 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Green_To_Yellow_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Green_To_Yellow_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from PRE_CRASH |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0334; ARCH_SW_DIA_0335 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Yellow_To_Red_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_CRASH_Treshold_Yellow_To_Red_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from PRE_CRASH |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0334; ARCH_SW_DIA_0335 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Green_To_Yellow_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Green_To_Yellow_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from PRE_SAFE_Lvl_4_5 |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0334; ARCH_SW_DIA_0335 |  |  |  |

### DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Yellow_To_Red_Read

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid00140_Obsolescence_Data_PRE_SAFE_Lvl_4_5_Treshold_Yellow_To_Red_Read (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Obsolescence Data collected from PRE_SAFE_Lvl_4_5 |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0330; ARCH_SW_DIA_0334; ARCH_SW_DIA_0335 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0334 | Response data length shall be 24 bytes long: Bytes #2-#3: DataIdentifier of the Read Obsolescence Data shall be: 0x0140 Bytes #4 - #5 = Remaining BSR Activations [2bytes] : Bytes #6 - #7 = Threshold Green to Yellow for Remaining BSR Activation [2bytes] = 0x00 0x00 Bytes #8 - #9 = Threshold Yellow to Red for Remaining BSR Activation [2bytes] = 0x00 0x00 Bytes #10 - #11 = Remaining Low Force Activations [2bytes] : Bytes #12 - #13 = Threshold Green to Yellow for Remaining Low Force Activation [2bytes] = 0x00 0x00 Bytes #14 - #15 = Threshold Yellow to Red for Remaining Low Force Activation [2bytes] = 0x00 0x00 Bytes #16 - #17 = Remaining high Force Activations [2bytes] : Bytes #18 - #19 = Threshold Green to Yellow for Remaining High Force Activation [2bytes] = 0x00 0x00 Bytes #20 - #21 = Threshold Yellow to Red for Remaining High Force Activation [2bytes] = 0x00 0x00 Bytes #22 - #23 = Remaining Max Force Activations [2bytes] : Bytes #24 - #25 = Threshold Green to Yellow for Remaining Max Force Activation [2bytes] = 0x00 0x00 Bytes #26 - #27 = Threshold Yellow to Red for Remaining Max Force Activation [2bytes] = 0x00 0x00 Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10187; DAI_EXT_TF_E_10189; DAI_EXT_TF_E_10190; DAI_EXT_TF_E_10191; DAI_EXT_TF_E_10194; DAI_EXT_TF_E_10195 |
| ARCH_SW_DIA_0335 | Read Obsolescence Data service shall return the number of remaing BSR(Comfort), Low Force, High Force, Max Force possible activation. |  | DAI_EXT_TF_E_10193 |

## DataServices_CPU_Load_Read_CPU_load_histograms_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_CPU_Load_Read_CPU_load_histograms_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the CPU Load Read Condition |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0340 |  |  |  |

## DataServices_CPU_Load_Read_CPU_load_histograms_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_CPU_Load_Read_CPU_load_histograms_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the length of the CPU Load DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint12 | Out | RTM_NUMBER_OF_CORES * (1 + RESOURCE_MEASUREMENT_DIAG_STUB_NUMBER_OF_CPULOAD_PERCENTILES |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0340; ARCH_SW_DIA_0344 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_0344 | Response data length shall be 12 bytes long: Bytes #2-#3: DataIdentifier of the Read CPU Load shall be: 0x01E1 Byte #4 = CPU ID [1byte]: [0x00] Core 1 Bytes #5 - #15 = CPU Load [11 bytes]: [Byte1] 0-9%, PercentUpdate [Byte2] 10-19%, PercentUpdate [Byte3] 20-29%, PercentUpdate [Byte4] 30-39%, PercentUpdate [Byte5] 40-49%, PercentUpdate [Byte6] 50-59%, PercentUpdate [Byte7] 60-69%, PercentUpdate [Byte8] 70-79%, PercentUpdate [Byte9] 80-89%, PercentUpdate [Byte10] 90-99%, PercentUpdate [Byte11] 100%, PercentUpdate Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10115; DAI_EXT_TF_E_10117; DAI_EXT_TF_E_10118; DAI_EXT_TF_E_10119 |
| --- | --- | --- | --- |

## DataServices_CPU_Load_Read_CPU_load_histograms_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_CPU_Load_Read_CPU_load_histograms_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns CPU Load in a histogram format |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0340; ARCH_SW_DIA_0341; ARCH_SW_DIA_0342; ARCH_SW_DIA_0343 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0341 | The Read CPU Load service shall be available in the following sessions: Default Session (0x01) Programming (0x02) Extended Session (0x03) Supplier Session/Development Session (0x7E) Update Session (0x42) |  | DAI_EXT_TF_E_10107 |
| ARCH_SW_DIA_0342 | Read CPU Load service shall be available under the following forms of Authentication: Supplier Development ENHANCED Internal Diagnostic Test Tool |  | DAI_EXT_TF_E_10108 |
| ARCH_SW_DIA_0343 | DataIdentifier of the Read CPU Load shall be: 0x01E1 |  | DAI_EXT_TF_E_10112 |

## DataServices_E2E_Timing_Read_Maximum_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_E2E_Timing_Read_Maximum_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function that checks the E2E Timing Read Condition |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0345 |  |  |  |

## DataServices_E2E_Timing_Read_Maximum_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_E2E_Timing_Read_Maximum_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function that calculates the E2E Timing Read Data Length |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint2 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0345; ARCH_SW_DIA_0349 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_0349 | Response data length shall be 2 bytes long: Bytes #2-#3: DataIdentifier of the Read E2E Timing shall be: 0x01E2 Byte #4 = E2E ID [1byte] : [0x00] Please insert KPI name (Please insert E2E KPI value) [0x01] Please insert KPI name (Please insert E2E KPI value) [0x02] Please insert KPI name (Please insert E2E KPI value) [0x03] Please insert KPI name (Please insert E2E KPI value) Bytes #5 = Maximum [1 byte] Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10131; DAI_EXT_TF_E_10133; DAI_EXT_TF_E_10134; DAI_EXT_TF_E_10135 |
| --- | --- | --- | --- |

## DataServices_E2E_Timing_Read_Maximum_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_E2E_Timing_Read_Maximum_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that calculates the E2E Timing per ID and its Maximum Percent |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0345; ARCH_SW_DIA_0346; ARCH_SW_DIA_0347; ARCH_SW_DIA_0348 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0346 | The Read E2E Timing service shall be available in the following sessions: Default Session (0x01) Programming (0x02) Extended Session (0x03) Supplier Session/Development Session (0x7E) Update Session (0x42) |  | DAI_EXT_TF_E_10123 |
| ARCH_SW_DIA_0347 | Read E2E Timing service shall be available under the following forms of Authentication: Supplier Development ENHANCED Internal Diagnostic Test Tool |  | DAI_EXT_TF_E_10124 |
| ARCH_SW_DIA_0348 | DataIdentifier of the Read E2E Timing shall be: 0x01E2 |  | DAI_EXT_TF_E_10128 |

## DataServices_Stack_Load_Read_Maximum_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_Stack_Load_Read_Maximum_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function that checks the Read Condition of the Stack Load DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0350 |  |  |  |

## DataServices_Stack_Load_Read_Maximum_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_Stack_Load_Read_Maximum_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function that calculates the Length of the Stack Load DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint22 | Out | RTM_NUMBER_OF_TASKS(11u) * 2 |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0350; ARCH_SW_DIA_0354 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_0354 | Response data length shall be 2 bytes long: Bytes #2-#3: DataIdentifier of the Read Stack Load shall be: 0x01E3 Byte #4 = Task Stack ID [1byte] [0x00] Core 1 Byte #5 = Maximum[1 byte] Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10147; DAI_EXT_TF_E_10149; DAI_EXT_TF_E_10150; DAI_EXT_TF_E_10151 |
| --- | --- | --- | --- |

## DataServices_Stack_Load_Read_Maximum_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_Stack_Load_Read_Maximum_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that calculates maximum Task Stack Usage per TaskID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0350; ARCH_SW_DIA_0351; ARCH_SW_DIA_0352; ARCH_SW_DIA_0353 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0351 | The Read Stack Load service shall be available in the following sessions: Default Session (0x01) Programming (0x02) Extended Session (0x03) Supplier Session/Development Session (0x7E) Update Session (0x42) |  | DAI_EXT_TF_E_10139 |
| ARCH_SW_DIA_0352 | Read Stack Load service shall be available under the following forms of Authentication: Supplier Development ENHANCED Internal Diagnostic Test Tool |  | DAI_EXT_TF_E_10140 |
| ARCH_SW_DIA_0353 | DataIdentifier of the Read Stack Load shall be: 0x01E3 |  | DAI_EXT_TF_E_10144 |

## DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of the Memory Usage Cycles DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0355 |  |  |  |

## DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the Length of the Memory Usage Cycles DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | 5u * Fee_Config.FeeBlkCnt | Out | 8-bit blockId + 32 bit writeCounter = 5 Byte per block |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0355; ARCH_SW_DIA_0359 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_0359 | Response data length shall be 5 bytes long: DataIdentifier of the Read Memory Usage Cycles shall be: 0x01E4 For each block there is a 5 byte response: Memory Block ID [1byte]: ⦁[0x00] Please insert memory ID (e.g. Data Flash Bank0) Memory Usage Cycles [4 bytes] Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10163; DAI_EXT_TF_E_10281; DAI_EXT_TF_E_10282; DAI_EXT_TF_E_10283 |
| --- | --- | --- | --- |

## DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_Memory_Usage_Cycles_Read_Memory_Usage_Cycles_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that calculates Memory Usage Cycles reported by FEE |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0355; ARCH_SW_DIA_0356; ARCH_SW_DIA_0357; ARCH_SW_DIA_0358 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0356 | The Read Memory usage Cycles service shall be available in the following sessions: Default Session (0x01) Programming (0x02) Extended Session (0x03) Supplier Session/Development Session (0x7E) Update Session (0x42) |  | DAI_EXT_TF_E_10155 |
| ARCH_SW_DIA_0357 | Read Memory usage Cycles service shall be available under the following forms of Authentication: Supplier Development ENHANCED Internal Diagnostic Test Tool |  | DAI_EXT_TF_E_10156 |
| ARCH_SW_DIA_0358 | DataIdentifier of the Read Memory usage Cycles shall be: 0x01E4 |  | DAI_EXT_TF_E_10160 |

## DIA_runDid0200_ CAN_input_ConditionCheckRead

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_1_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_1_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 1st Bitfield of DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_2_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_2_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 2nd Bitfield of DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_3_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_3_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 3rd Bitfield of DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_4_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_4_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 4th Bitfield of DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_5_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_5_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 5th Bitfield of DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_6_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_6_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 6th Bitfield of DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Supply_battery_voltage_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Supply_battery_voltage_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the Supply Battery Voltage of DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_7_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_7_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 7th Bitfield of DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_8_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_8_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 8th Bitfield of DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360 |  |  |  |

## DIA_runDid0200_ CAN_input_Read

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_1_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_1_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 1st Bitfield for DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360; ARCH_SW_DIA_0361; ARCH_SW_DIA_0362; ARCH_SW_DIA_0363; ARCH_SW_DIA_0364; ARCH_SW_DIA_0365 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_2_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_2_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 2nd Bitfield for DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360; ARCH_SW_DIA_0361; ARCH_SW_DIA_0362; ARCH_SW_DIA_0363; ARCH_SW_DIA_0364; ARCH_SW_DIA_0365 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_3_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_3_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 3rd Bitfield for DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360; ARCH_SW_DIA_0361; ARCH_SW_DIA_0362; ARCH_SW_DIA_0363; ARCH_SW_DIA_0364; ARCH_SW_DIA_0365; ARCH_SW_DIA_0441; ARCH_SW_DIA_0442; ARCH_SW_DIA_0443 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_4_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_4_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 4th Bitfield for DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360; ARCH_SW_DIA_0361; ARCH_SW_DIA_0362; ARCH_SW_DIA_0363; ARCH_SW_DIA_0364; ARCH_SW_DIA_0365 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_5_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_5_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 5th Bitfield for DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360; ARCH_SW_DIA_0361; ARCH_SW_DIA_0362; ARCH_SW_DIA_0363; ARCH_SW_DIA_0364; ARCH_SW_DIA_0365; ARCH_SW_DIA_0447; ARCH_SW_DIA_0448; ARCH_SW_DIA_0449; ARCH_SW_DIA_0453; ARCH_SW_DIA_0454; ARCH_SW_DIA_0455; ARCH_SW_DIA_0456; ARCH_SW_DIA_0457; ARCH_SW_DIA_0458 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_6_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_6_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 6th Bitfield for DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360; ARCH_SW_DIA_0361; ARCH_SW_DIA_0362; ARCH_SW_DIA_0363; ARCH_SW_DIA_0364; ARCH_SW_DIA_0365 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Supply_battery_voltage_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Supply_battery_voltage_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Supply Battery Voltage for DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360; ARCH_SW_DIA_0361; ARCH_SW_DIA_0362; ARCH_SW_DIA_0363; ARCH_SW_DIA_0364; ARCH_SW_DIA_0365; ARCH_SW_DIA_0450; ARCH_SW_DIA_0451; ARCH_SW_DIA_0452 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_7_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_7_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 7th Bitfield for DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360; ARCH_SW_DIA_0361; ARCH_SW_DIA_0362; ARCH_SW_DIA_0363; ARCH_SW_DIA_0364; ARCH_SW_DIA_0365; ARCH_SW_DIA_0444; ARCH_SW_DIA_0445; ARCH_SW_DIA_0446; ARCH_SW_DIA_0824; ARCH_SW_DIA_0825 |  |  |  |

### DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_8_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0200_CAN_input_Read_Neues_Bitfeld_8_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 8th Bitfield for DID 0200 CAN Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0360; ARCH_SW_DIA_0361; ARCH_SW_DIA_0362; ARCH_SW_DIA_0363; ARCH_SW_DIA_0364; ARCH_SW_DIA_0365; ARCH_SW_DIA_0441; ARCH_SW_DIA_0442; ARCH_SW_DIA_0443; ARCH_SW_DIA_0444; ARCH_SW_DIA_0445; ARCH_SW_DIA_0446; ARCH_SW_DIA_0447; ARCH_SW_DIA_0448; ARCH_SW_DIA_0449; ARCH_SW_DIA_0450; ARCH_SW_DIA_0451; ARCH_SW_DIA_0452; ARCH_SW_DIA_0453; ARCH_SW_DIA_0454; ARCH_SW_DIA_0455; ARCH_SW_DIA_0456; ARCH_SW_DIA_0457; ARCH_SW_DIA_0458; ARCH_SW_DIA_0497; ARCH_SW_DIA_0498; ARCH_SW_DIA_0499; ARCH_SW_DIA_0500; ARCH_SW_DIA_0501; ARCH_SW_DIA_0502; ARCH_SW_DIA_0503; ARCH_SW_DIA_0504; ARCH_SW_DIA_0505; ARCH_SW_DIA_0506; ARCH_SW_DIA_0535; ARCH_SW_DIA_0536; ARCH_SW_DIA_0537; ARCH_SW_DIA_0538; ARCH_SW_DIA_0539; ARCH_SW_DIA_0540; ARCH_SW_DIA_0541; ARCH_SW_DIA_0542; ARCH_SW_DIA_0824; ARCH_SW_DIA_0825 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0361 | The Read CAN Input service shall be available in the following sessions: Default Session (0x01) Extended Session (0x03) Supplier Session/Development Session (0x7E) Update Session (0x42) |  | DAI_EXT_TF_E_10200 |
| ARCH_SW_DIA_0362 | Read CAN Input service shall be available under the following forms of Authentication: Supplier Development ENHANCED Production After-Sales ENHANCED After-Sales After-Sales BASIC Internal Diagnostic Test Tool ePTI Test Tool |  | DAI_EXT_TF_E_10201 |
| ARCH_SW_DIA_0363 | DataIdentifier of the Read CAN Input shall be: 0x0200 |  | DAI_EXT_TF_E_10205 |
| ARCH_SW_DIA_0364 | Response data length shall be 9 bytes long: Bytes #2-#3: DataIdentifier of the Read CAN Input shall be: 0x0200 Byte #4 : bits 0-2: Ignition switch state (Isw_Stat_ST3 signal) bit 3 : Reserved (set to 0) bits 4-5 : Buckle switch driver state (Bckl_Sw_D_Stat_ST3) bits 6-7 : Buckle switch front passenger state (Bckl_Sw_FP_Stat_ST3) Byte #5 : bits 0-1 : Presafe enabled (Presf_Enbl_ST3) bits 2-3 : Presafe actuator RBTM_FL tensioning suppress request (PresfAct_TensSupp_Rq_ST3) bits 4-7 : Presafe level (Presf_Lvl_ST3) Byte #6 : bits 0-1 : Belt hand over left state (BeltHdOvr_FL_Stat_ST3) bits 2-3 : Belt hand over right state (BeltHdOvr_FR_Stat_ST3) bits 4-6 : Engine start state (PT4_PTCoor_EngStartPN14_Stat_ST3) bit 7 : Reserved (set to 0) Byte #7 : bit 0 : Impact event X (Impact_X_ST3) bit 1 : Reserved (set to 0) bit 2 : Turn over event 1 RO_type1 (Impact_RO_type1_ST3) bit 3 : Turn over event 2 RO_type2 (Impact_RO_type2_ST3) bits 4-5 : Steering variant (EVC_List03_StStyle_ST3) bits 6-7 : Reserved (set to 0) Byte #8 : bits 0-7 : Supply battery voltage (ngagedng of PN14_SupBat_Volt_ST3 signal with 0.1V resolution) Byte #9 : bits 0-2: Supply battery cutoff switch status ((PN14_DDP_Sw_Stat_ST3) bits3-5 : Powernet Production mode status (PN14_ProdMd_Stat_ST3) bits 6-7 : Powernet transport mode status (PN14_TransMd_Stat_ST3) Byte #10 : bits 0-1: Belt slack decreasing mode request (BltSlckDec_Md_Rq_HU_ST3) bits 2-3 : Powertrain ready (PT4_PTCoor_PT_Rdy_ST3) bits 4-7 : Reverse gear ngaged (PT4_PTCoor_DrvPosn_Stat_ST3) Byte #11 : bits 0-5: RBTMFL_SP_Lvl_Rq_ST35 bits 6-7: DidA external tester present (DidA_ExtTest_Pres_ST3) Byte #12 : bits 0-5: RBTMFR_SP_Lvl_Rq_ST35 bits 6-7: Presafe acceleration threshold (PresfAccelReset_NotExcd_ST3) Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10208; DAI_EXT_TF_E_10210; DAI_EXT_TF_E_10211; DAI_EXT_TF_E_10212; DAI_EXT_TF_E_10213; DAI_EXT_TF_E_10214; DAI_EXT_TF_E_10217; DAI_EXT_TF_E_10218; DAI_EXT_TF_E_10219; DAI_EXT_TF_E_10220 |
| ARCH_SW_DIA_0365 | Read CAN Input service shall return the current values of the mentioned CAN signals. |  | DAI_EXT_TF_E_10216 |
| ARCH_SW_DIA_0441 | Signal PT4_PTCoor_EngStartPN14_Stat_ST3 Position: ⦁ StartByte = 4; ⦁ StartBit = 4; ⦁ SignalLength = 3; Note:StartByte starts from 1 and StartBit starts from 0 |  | DAI_EXT_TF_B_2303 |
| ARCH_SW_DIA_0442 | Signal Values: 0 = NO_COMBUSTIONSTART_RQ (No combustion start required) 1 = CUSTOMER_START_RQ(Customer start required) 2 = AUTO_START_NO_PN_SUP_RQ(Automatic start without Powernet support required) 3 = AUTO_START_PN_SUP_RQ(Automatic start with Powernet support required) 7 = Signal not Available (SNA) |  | DAI_EXT_TF_B_2304 |
| ARCH_SW_DIA_0443 | PT4_PTCoor_EngStartPN14_Stat_ST3 signal shall be read by both Front LEFT ECU and Front RIGHT ECU and reported via CAN Input Service (0x0200) |  | DAI_EXT_TF_B_2305 |
| ARCH_SW_DIA_0444 | Signal DidA_ExtTest_Pres_ST3 Position: ⦁ StartByte = 1 ⦁ StartBit = 2; ⦁ SignalLength = 2 Note:StartByte starts from 1 and StartBit starts from 0 |  | DAI_EXT_TF_B_2319 |
| ARCH_SW_DIA_0445 | Signal Values: 0x01= FALSE 0x02 = TRUE 0x03 = default / Signal not Available (SNA) |  | DAI_EXT_TF_B_2320 |
| ARCH_SW_DIA_0446 | DidA_ExtTest_Pres_ST3 signal shall be read by both Front LEFT ECU and Front RIGHT ECU and reported via CAN Input Service (0x0200) |  | DAI_EXT_TF_B_2321 |
| ARCH_SW_DIA_0824 | If DidA_ExtTest_Pres_ST3 signal is received with SNA value then the value that shall be reported is 0x01 - External Tester Not Present |  | DAI_EXT_TF_B_2427 |
| ARCH_SW_DIA_0825 | If timeout of DIAG_Stat_ST3 frame (10000ms) is detected then the value that shall be reported is 0x01 - External Tester Not Present |  | DAI_EXT_TF_B_2428 |
| ARCH_SW_DIA_0447 | Signal PN14_DDP_Sw_Stat_ST3 Position: ⦁ StartByte = 1 ⦁ StartBit = 2; ⦁ SignalLength = 3 ⦁ Note:StartByte starts from 1 and StartBit starts from 0 |  | DAI_EXT_TF_B_2330 |
| ARCH_SW_DIA_0448 | Signal Values: 0 = CLS TDP (switch is closed) 1 = PRE_RESET (Pre-Reset warning) 2 = RESET (Reset active) 3 = PRE_OPN (Pre-Open warning) 4 = OPN (TDP switch is open) 7 = default / Signal not Available (SNA) |  | DAI_EXT_TF_B_2331 |
| ARCH_SW_DIA_0449 | PN14_DDP_Sw_Stat_ST3 signal shall be read by both Front LEFT ECU and Front RIGHT ECU and reported via CAN Input Service (0x0200) |  | DAI_EXT_TF_B_2332 |
| ARCH_SW_DIA_0450 | Signal PN14_SupBat_Volt_ST3 Position: ⦁ StartByte = 3 ⦁ StartBit = 6; ⦁ SignalLength = 10 Note:StartBytes starts from 1 and StartBits starts from 0 |  | DAI_EXT_TF_B_2335 |
| ARCH_SW_DIA_0451 | Signal Values: 0x00...0x3FE (0..25.55 V) 0x03FF = default / Signal not Available (SNA) unit: 0.025 V |  | DAI_EXT_TF_B_2336 |
| ARCH_SW_DIA_0452 | PN14_SupBat_Volt_ST3 signal shall be read by both Front LEFT ECU and Front RIGHT ECU and reported via CAN Input Service (0x0200) |  | DAI_EXT_TF_B_2337 |
| ARCH_SW_DIA_0453 | Signal PN14_ProdMd_Stat_ST3 Position: ⦁ StartByte = 1 ⦁ StartBit = 5; ⦁ SignalLength = 3; Note:StartBytes starts from 1 and StartBits starts from 0 |  | DAI_EXT_TF_B_2340 |
| ARCH_SW_DIA_0454 | Signal Values: 0 = OFF (Productionmode off) 1 = ON (Productionmode active) 2 = SF01 (Productionmode Special Funcion 1) 3 = SF02 (Productionmode Special Funcion 2) 4 = SF03 (Productionmode Special Funcion 3) 7 = default / Signal not Available (SNA) |  | DAI_EXT_TF_B_2341 |
| ARCH_SW_DIA_0455 | PN14_ProdMd_Stat_ST3 signal shall be read by both Front LEFT ECU and Front RIGHT ECU and reported via CAN Input Service (0x0200) |  | DAI_EXT_TF_B_2342 |
| ARCH_SW_DIA_0456 | Signal PN14_TransMd_Stat_ST3 Position: ⦁ StartByte = 1 ⦁ StartBit = 0; ⦁ SignalLength = 2; Note:StartBytes starts from 1 and StartBits starts from 0 |  | DAI_EXT_TF_B_2345 |
| ARCH_SW_DIA_0457 | Signal Values: 0 = OFF (Transport mode permanently off) 1 = TEMP_OFF (Transport mode temporarily off) 2 = ACTV (Transport mode active) 3 = default / Signal not Available (SNA) |  | DAI_EXT_TF_B_2346 |
| ARCH_SW_DIA_0458 | PN14_TransMd_Stat_ST3 signal shall be read by both Front LEFT ECU and Front RIGHT ECU and reported via CAN Input Service (0x0200) |  | DAI_EXT_TF_B_2347 |
| ARCH_SW_DIA_0497 | CAN Frame DIAG_Stat_ST3 shall be identified with the address 0x6D0 |  | DAI_EXT_TF_B_2313 |
| ARCH_SW_DIA_0498 | CAN Frame PN14_Master_Stat1_ST3 shall be identified with the address 0x464 |  | DAI_EXT_TF_B_2324 |
| ARCH_SW_DIA_0499 | Diagnostic request frame DIAG_RBTM_FL_03_ExtEth_RQ_ST3 identifier shall be 0x18E1A488 if ECU is configured as FRONT LEFT side |  | DAI_EXT_TF_B_2279 |
| ARCH_SW_DIA_0500 | DIAG_RBTM_FL_03_ExtEth_RQ_ST3 Frame size: variable, max 8 bytes |  | DAI_EXT_TF_B_2288 |
| ARCH_SW_DIA_0501 | Diagnostic request frame DIAG_RBTM_FR_03_ExtEth_RQ_ST3 identifier shall be 0x18E1A588 if ECU is configured as FRONT RIGHT side |  | DAI_EXT_TF_B_2281 |
| ARCH_SW_DIA_0502 | DIAG_RBTM_FR_03_ExtEth_RQ_ST3 Frame size: variable, max 8 bytes |  | DAI_EXT_TF_B_2289 |
| ARCH_SW_DIA_0503 | If a diagnostic service request is received via CAN Frame DIAG_RBTM_xy_03_ExtEth_RQ_ST3 then RGS shall respond via DIAG_RBTM_FL_03_ExtEth_RS_ST3 (Id: 0x18E1A480) frame |  | DAI_EXT_TF_B_2284 |
| ARCH_SW_DIA_0504 | DIAG_RBTM_FL_03_ExtEth_RS_ST3 Frame size: variable max 8 bytes |  | DAI_EXT_TF_B_2286 |
| ARCH_SW_DIA_0505 | If a diagnostic service request is received via CAN Frame DIAG_RBTM_FR_03_ExtEth_RQ_ST3 then RGS shall respond via DIAG_RBTM_FR_03_ExtEth_RS_ST3 (Id: 0x18E1A580) frame |  | DAI_EXT_TF_B_2285 |
| ARCH_SW_DIA_0506 | DIAG_RBTM_FR_03_ExtEth_RS_ST3 Frame size: variable max 8 bytes |  | DAI_EXT_TF_B_2287 |
| ARCH_SW_DIA_0535 | Diagnostic request frame DIAG_RBTM_FL_03_ExtCAN_RQ_ST3 identifier shall be 0x18E1A489 if ECU is configured as FRONT LEFT side. |  | DAI_EXT_TF_B_2365 |
| ARCH_SW_DIA_0536 | DIAG_RBTM_FL_03_ExtCAN_RQ_ST3 Frame size: variable, max 8 bytes |  | DAI_EXT_TF_B_2366 |
| ARCH_SW_DIA_0537 | Diagnostic request frame DIAG_RBTM_FR_03_ExtCAN_RQ_ST3 identifier shall be 0x18E1A589 if ECU is configured as FRONT LEFT side |  | DAI_EXT_TF_B_2367 |
| ARCH_SW_DIA_0538 | DIAG_RBTM_FR_03_ExtCAN_RQ_ST3 Frame size: variable, max 8 bytes |  | DAI_EXT_TF_B_2368 |
| ARCH_SW_DIA_0539 | Diagnostic request frame DIAG_RBTM_FL_03_IntEth_RQ_ST3 identifier shall be 0x18E1A48D if ECU is configured as FRONT LEFT side |  | DAI_EXT_TF_B_2369 |
| ARCH_SW_DIA_0540 | DIAG_RBTM_FL_03_IntEth_RQ_ST3 Frame size: variable, max 8 bytes |  | DAI_EXT_TF_B_2370 |
| ARCH_SW_DIA_0541 | Diagnostic request frame DIAG_RBTM_FR_03_IntEth_RQ_ST3 identifier shall be 0x18E1A58D if ECU is configured as FRONT LEFT side |  | DAI_EXT_TF_B_2371 |
| ARCH_SW_DIA_0542 | DIAG_RBTM_FR_03_IntEth_RQ_ST3 Frame size: variable, max 8 bytes |  | DAI_EXT_TF_B_2372 |

## DataServices_HardwareVersion_Read_HW_ReadData

### DataServices_HardwareVersion_Read_HW_patch_level_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_HardwareVersion_Read_HW_patch_level_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of Patch Level of the HW |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0370 |  |  |  |

### DataServices_HardwareVersion_Read_HW_patch_level_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_HardwareVersion_Read_HW_patch_level_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Patch Level of the HW |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0370; ARCH_SW_DIA_0371; ARCH_SW_DIA_0372; ARCH_SW_DIA_0373 |  |  |  |

### DataServices_HardwareVersion_Read_HW_week_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_HardwareVersion_Read_HW_week_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of Week in which HW was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0370 |  |  |  |

### DataServices_HardwareVersion_Read_HW_week_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_HardwareVersion_Read_HW_week_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Week when the HW was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0370; ARCH_SW_DIA_0371; ARCH_SW_DIA_0372; ARCH_SW_DIA_0373 |  |  |  |

### DataServices_HardwareVersion_Read_HW_year_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_HardwareVersion_Read_HW_year_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of Year in which HW was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0370 |  |  |  |

### DataServices_HardwareVersion_Read_HW_year_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_HardwareVersion_Read_HW_year_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Year when the HW was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0370; ARCH_SW_DIA_0371; ARCH_SW_DIA_0372; ARCH_SW_DIA_0373; ARCH_SW_DIA_0374; ARCH_SW_DIA_0471; ARCH_SW_DIA_0472; ARCH_SW_DIA_0473 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0471 | DataIdentifier shall be 0xF150 |  | DAI_EXT_TF_E_9733 |
| ARCH_SW_DIA_0472 | The DID shall be available in the following sessions: ⦁Default Session ⦁Programming ⦁Extended session ⦁Update Session ⦁Development Session |  | DAI_EXT_TF_E_9749 |
| ARCH_SW_DIA_0473 | This service shall be available with or without Any form of Authentication. |  | DAI_EXT_TF_E_9942 |
| ARCH_SW_DIA_0371 | Response data shall be 3 bytes long |  | DAI_EXT_TF_E_9830 |
| ARCH_SW_DIA_0372 | Byte 0 of the positive response shall represent the year when the HW was designed. |  | DAI_EXT_TF_E_9831 |
| ARCH_SW_DIA_0373 | Byte 1 of the positive response shall represent the week when the HW was designed. |  | DAI_EXT_TF_E_9832 |
| ARCH_SW_DIA_0374 | Byte 2 of the positive response shall represent the current patch level of the HW. |  | DAI_EXT_TF_E_9833 |

## DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of the Year / Week / Patch Level when the software was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0375 |  |  |  |

## DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function checks the Length of the F151 DID |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint6 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0375; ARCH_SW_DIA_0376; ARCH_SW_DIA_0377; ARCH_SW_DIA_0378; ARCH_SW_DIA_0379 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_0376 | Response data shall be 6 bytes long |  | DAI_EXT_TF_E_10267 |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0377 | Byte 0 of the positive response shall represent the year when the Boot SW Version was designed. Byte 1 of the positive response shall represent the week when the Boot SW Version was designed. Byte 2 of the positive response shall represent the current patch level of the Boot SW. |  | DAI_EXT_TF_E_10268 |
| ARCH_SW_DIA_0378 | Byte 3 of the positive response shall represent the year when the Application SW Version was designed. Byte 4 of the positive response shall represent the week when the Application SW Version was designed. Byte 5 of the positive response shall represent the current patch level of the Application SW. |  | DAI_EXT_TF_E_10271 |
| ARCH_SW_DIA_0379 | Byte 6 of the positive response shall represent the year when the HSM SW Version was designed. Byte 7 of the positive response shall represent the week when the HSM SW Version was designed. Byte 8 of the positive response shall represent the current patch level of the HSM SW. |  | DAI_EXT_TF_E_11467 |

## DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_SoftwareVersion_Read_Software_Logical_Block_Version_Information_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Year / Week / Patch Level when software was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0375; ARCH_SW_DIA_0474; ARCH_SW_DIA_0475; ARCH_SW_DIA_0476 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0474 | DataIdentifier shall be 0xF151 |  | DAI_EXT_TF_E_9732 |
| ARCH_SW_DIA_0475 | The DID shall be available in the following sessions: ⦁Default Session ⦁Programming ⦁Extended session ⦁Update Session ⦁Development Session |  | DAI_EXT_TF_E_9750 |
| ARCH_SW_DIA_0476 | This service shall be available with or without Any form of Authentication. |  | DAI_EXT_TF_E_9943 |

## DataServices_BootSoftwareVersion_Read_Boot_SW_ReadData

### DataServices_BootSoftwareVersion_Read_Boot_SW_patch_level_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_BootSoftwareVersion_Read_Boot_SW_patch_level_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of the Patch Level when the Boot SW was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0380 |  |  |  |

### DataServices_BootSoftwareVersion_Read_Boot_SW_patch_level_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_BootSoftwareVersion_Read_Boot_SW_patch_level_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Patch Level when the Boot SW was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0380; ARCH_SW_DIA_0381; ARCH_SW_DIA_0382; ARCH_SW_DIA_0383 |  |  |  |

### DataServices_BootSoftwareVersion_Read_Boot_SW_week_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_BootSoftwareVersion_Read_Boot_SW_week_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of the Week when the Boot SW was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0380 |  |  |  |

### DataServices_BootSoftwareVersion_Read_Boot_SW_week_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_BootSoftwareVersion_Read_Boot_SW_week_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Week when the Boot SW was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0380; ARCH_SW_DIA_0381; ARCH_SW_DIA_0382; ARCH_SW_DIA_0383 |  |  |  |

### DataServices_BootSoftwareVersion_Read_Boot_SW_year_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_BootSoftwareVersion_Read_Boot_SW_year_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of the Year when the Boot SW was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0380 |  |  |  |

### DataServices_BootSoftwareVersion_Read_Boot_SW_year_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_BootSoftwareVersion_Read_Boot_SW_year _ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the Year when the Boot SW was designed |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0380; ARCH_SW_DIA_0381; ARCH_SW_DIA_0382; ARCH_SW_DIA_0383; ARCH_SW_DIA_0477; ARCH_SW_DIA_0478; ARCH_SW_DIA_0479 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0477 | DataIdentifier shall be 0xF153 |  | DAI_EXT_TF_E_9953 |
| ARCH_SW_DIA_0478 | The DID shall be available in the following sessions: ⦁Default Session ⦁Programming ⦁Extended session ⦁Update Session ⦁Development Session |  | DAI_EXT_TF_E_9956 |
| ARCH_SW_DIA_0479 | This service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_9959 |
| ARCH_SW_DIA_0381 | Response data shall be 3 bytes long |  | DAI_EXT_TF_E_9960 |
| ARCH_SW_DIA_0382 | Byte 0 of the positive response shall represent the year when the Boot SW was designed. |  | DAI_EXT_TF_E_9961 |
| ARCH_SW_DIA_0383 | Byte 1 of the positive response shall represent the week when the Boot SW was designed. |  | DAI_EXT_TF_E_9962 |
| ARCH_SW_DIA_0384 | Byte 2 of the positive response shall represent the current patch level of the Boot SW. |  | DAI_EXT_TF_E_9963 |

## DIA_runDid0241_PRE_SAFE_Recorder_1_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0241_PRE_SAFE_Recorder_1_WriteData(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, uint16 DataLength, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function resets/erases the values of the Pre-Safe Recorder 1 structure |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 |  |  |
| DataLength | Uint22 |  |  |
| OpStatus | Dcm_OpStatusType |  |  |
| ErrorCode | Dcm_NegativeResponseCodeType |  |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0385; ARCH_SW_DIA_0386; ARCH_SW_DIA_0387; ARCH_SW_DIA_0480; ARCH_SW_DIA_0481; ARCH_SW_DIA_0482; ARCH_SW_DIA_0483 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0480 | DataIdentifier shall be 0x0241 |  | DAI_EXT_TF_E_9740 |
| ARCH_SW_DIA_0481 | Using this service shall lead to the Pre-Safe Recorder 1 to be set to default value. |  | DAI_EXT_TF_E_9951 |
| ARCH_SW_DIA_0482 | The DID shall be available in the following sessions: ⦁Extended session ⦁Update Session ⦁Development Session |  | DAI_EXT_TF_E_9754 |
| ARCH_SW_DIA_0483 | This service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales |  | DAI_EXT_TF_E_9922 |
| ARCH_SW_DIA_0386 | Request data shall be 1 byte long. |  | DAI_EXT_TF_E_9768 |
| ARCH_SW_DIA_0387 | Byte 0 shall be defined as the Erase Pre-Safe Memory command and shall always have value 0x01. |  | DAI_EXT_TF_E_9769 |

## DIA_runDid0242_PRE_SAFE_Recorder_2_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, RTE_DIA_AC_DIAGONCAN_APPL_CODE) DIA_runDid0241_PRE_SAFE_Recorder_2_WriteData(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, uint16 DataLength, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function resets/erases the values of the Pre-Safe Recorder 2 structure |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 |  |  |
| DataLength | Uint22 |  |  |
| OpStatus | Dcm_OpStatusType |  |  |
| ErrorCode | Dcm_NegativeResponseCodeType |  |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0390; ARCH_SW_DIA_0391; ARCH_SW_DIA_0392; ARCH_SW_DIA_0484; ARCH_SW_DIA_0485; ARCH_SW_DIA_0486; ARCH_SW_DIA_0487 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0484 | DataIdentifier shall be 0x0242 |  | DAI_EXT_TF_E_9741 |
| ARCH_SW_DIA_0485 | Using this service shall lead to the Pre-Safe Recorder 2 to be set to default value. |  | DAI_EXT_TF_E_9950 |
| ARCH_SW_DIA_0486 | The DID shall be available in the following sessions: ⦁Extended session ⦁Update Session ⦁Development Session |  | DAI_EXT_TF_E_9752 |
| ARCH_SW_DIA_0487 | This service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales |  | DAI_EXT_TF_E_9920 |
| ARCH_SW_DIA_0391 | Request data shall be 1 byte long. |  | DAI_EXT_TF_E_9770 |
| ARCH_SW_DIA_0392 | Byte 0 shall be defined as the Erase Pre-Safe Memory command and shall always have value 0x01. |  | DAI_EXT_TF_E_9771 |

## DataServices_PRE_SAFE_Counter_read_Read_CombinedDataElement_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_PRE_SAFE_Counter_read_Read_CombinedDataElement_WriteData(P2CONST(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_DATA) Data, uint16 DataLength, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function resets/erases the values of the Pre Safe Counter structure |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 | Out |  |
| DataLength | Uint16 | Out |  |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0395; ARCH_SW_DIA_0396; ARCH_SW_DIA_0397; ARCH_SW_DIA_0488; ARCH_SW_DIA_0489; ARCH_SW_DIA_0490; ARCH_SW_DIA_0491 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0488 | DataIdentifier shall be 0x0244 |  | DAI_EXT_TF_E_9742 |
| ARCH_SW_DIA_0489 | Using this service shall lead to the Pre-Safe Counter to be set to default value (i.e. 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00) |  | DAI_EXT_TF_E_9914 |
| ARCH_SW_DIA_0490 | The DID shall be available in the following sessions: ⦁Extended session ⦁Update Session ⦁Development Session |  | DAI_EXT_TF_E_9753 |
| ARCH_SW_DIA_0491 | This service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production |  | DAI_EXT_TF_E_9917 |
| ARCH_SW_DIA_0396 | Request data shall be 1 byte long. |  | DAI_EXT_TF_E_9772 |
| ARCH_SW_DIA_0397 | Byte 0 shall be defined as the Pre-Safe Request Counter Reset command and shall always have value 0x01. |  | DAI_EXT_TF_E_9773 |

## Dcm_Svc28Handler

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DCM_LOCAL_INLINE FUNC(Std_ReturnType, DCM_CODE) Dcm_Svc28Handler(Dcm_ContextPtrType pContext, Dcm_OpStatusType opStatus, Dcm_MsgContextPtrType pMsgContext, Dcm_NegativeResponseCodePtrType ErrorCode, Dcm_Svc28RepeaterProxyContextPtrType pRepContext) |  |  |  |
| Object |  |  |  |
| Function handles the Communication Control Service |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| opStatus | Dcm_OpStatusType | Out |  |
| pMsgContext | Dcm_MsgContextPtrType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodePtrType | Out |  |
| pRepContext | Dcm_Svc28RepeaterProxyContextPtrType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0400; ARCH_SW_DIA_0401; ARCH_SW_DIA_0402; ARCH_SW_DIA_0403; ARCH_SW_DIA_0404; ARCH_SW_DIA_0405; ARCH_SW_DIA_0406; ARCH_SW_DIA_0407 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0401 | The Communication control service shall be available in the following sessions: Extended Session (0x03) Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10228 |
| ARCH_SW_DIA_0402 | Communication control service shall be available under the following forms of Authentication: Supplier Development ENHANCED Production After-Sales ENHANCED |  | DAI_EXT_TF_E_10229 |
| ARCH_SW_DIA_0403 | Control Type = 0x00 correspond to the Enable Rx and Tx Note: This value indicates that the reception and transmission of messages shall be enabled for the specified communicationType. |  | DAI_EXT_TF_E_10233 |
| ARCH_SW_DIA_0404 | Control Type = 0x01 correspond to the Enable Rx and Disable Tx Note: This value indicates that the reception of messages shall be enabled and the transmission shall be disabled for the specified communicationType |  | DAI_EXT_TF_E_10249 |
| ARCH_SW_DIA_0405 | Communication Type = 0x01 correspond to Normal Communication type. Note: Normal Communication is referring to all application-related communication signals |  | DAI_EXT_TF_E_10252 |
| ARCH_SW_DIA_0406 | Byte #2: Control Type of the Communication control shall be an echo of "Control Type" parameter from the request message. |  | DAI_EXT_TF_E_10238 |
| ARCH_SW_DIA_0407 | If positive response of the Communication Control service has been received, then the transmission / reception of messages shall be done according the "Control Type" and "Communication Type" requested. |  | DAI_EXT_TF_E_10248 |

## Dcm_Svc85Handler

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DCM_LOCAL_INLINE FUNC(Std_ReturnType, DCM_CODE) Dcm_Svc85Handler(Dcm_ContextPtrType pContext, Dcm_OpStatusType opStatus, Dcm_MsgContextPtrType pMsgContext, Dcm_NegativeResponseCodePtrType ErrorCode, Dcm_Svc85RepeaterProxyContextPtrType pRepContext) |  |  |  |
| Object |  |  |  |
| Function handles the ControlDTCSetting Service |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| pContext | Dcm_ContextPtrType |  |  |
| opStatus | Dcm_OpStatusType opStatus |  |  |
| pMsgContext | Dcm_MsgContextPtrType |  |  |
| ErrorCode | Dcm_NegativeResponseCodePtrType |  |  |
| pRepContext | Dcm_Svc85RepeaterProxyContextPtrType |  |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0410; ARCH_SW_DIA_0411; ARCH_SW_DIA_0412; ARCH_SW_DIA_0413; ARCH_SW_DIA_0414; ARCH_SW_DIA_0415; ARCH_SW_DIA_0416; ARCH_SW_DIA_0417; ARCH_SW_DIA_0418; ARCH_SW_DIA_0492; ARCH_SW_DIA_0493; ARCH_SW_DIA_0494; ARCH_SW_DIA_0495; ARCH_SW_DIA_0496 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0492 | The DID shall be available in the following sessions: ⦁Extended session ⦁Development Session |  | DAI_EXT_TF_E_9971 |
| ARCH_SW_DIA_0493 | This service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED |  | DAI_EXT_TF_E_8092 |
| ARCH_SW_DIA_0494 | ⦁The value 0x85 shall correspond to the SID-RQ of the ControlDTCSetings Service |  | DAI_EXT_TF_E_9974 |
| ARCH_SW_DIA_0495 | ⦁The value 0x01 shall correspond to the DTCSettingType: DTC Setting Enabled |  | DAI_EXT_TF_E_9975 |
| ARCH_SW_DIA_0496 | ⦁The value 0x02 shall correspond to the DTCSettingType: DTC Setting Disabled |  | DAI_EXT_TF_E_9976 |
| ARCH_SW_DIA_0411 | The byte 1 shall be reserved for the SID-PR of the ControlDTCSetting service: The value 0xC5 shall correspond to the SID-PR of the ControlDTCSetting service. |  | DAI_EXT_TF_E_9980 |
| ARCH_SW_DIA_0412 | The byte 2 shall be reserved for the DTCSettingType of the ControlDTCSetting service: The value 0x01 shall correspond to the DTCSettingType: DTC Setting Enabled. The value 0x02 shall correspond to the DTCSettingType: DTC Setting Disabled. |  | DAI_EXT_TF_E_9979; DAI_EXT_TF_E_9981; DAI_EXT_TF_E_9982 |
| ARCH_SW_DIA_0413 | If positive response has been received for "DTC Setting Disabled" then the status of the DTCs shall be frozen. Note: ControlDTCSetting deactivates only the updating of the StatusOfDTC bits. The service is not used to deactivate substitute functions. |  | DAI_EXT_TF_E_9995 |
| ARCH_SW_DIA_0414 | If positive response has been received for "DTC Setting Enabled" then the status of the DTCs shall be resumed. |  | DAI_EXT_TF_E_9996 |
| ARCH_SW_DIA_0415 | If a clearDiagnosticInformation (14 hex) service is sent by the client, the ControlDTCSetting shall not prohibit resetting the server's DTC memory. If a successful ECUReset is performed, then this re-enables the setting of DTCs. |  | DAI_EXT_TF_E_9997 |
| ARCH_SW_DIA_0416 | The byte 1 shall be reserved for the SID-NR of the ControlDTCSetting service: ⦁ The value 0x7F shall correspond to the SID-NR of the ControlDTcSetting service. |  | DAI_EXT_TF_E_9984; DAI_EXT_TF_E_9987 |
| ARCH_SW_DIA_0417 | The byte 2 shall be reserved for the SID-RQ-NR of the ControlDTCSetting service: The value 0x85 shall correspond to the SID-RQ-NR of the ControlDTCSetting service. |  | DAI_EXT_TF_E_9985; DAI_EXT_TF_E_9988 |
| ARCH_SW_DIA_0418 | The byte 3 shall be reserved for the NRC-CDTCS of the ControlDTCSetting service: The value 0x12 shall correspond to the NRC-CDTCS: SubFunctionNotSupported The value 0x13 shall correspond to the NRC-CDTCS: IncorrectMessageLengthOrInvalidFormat The value 0x22 shall correspond to the NRC-CDTCS: ConditionsNotCorrect The value 0x7E shall correspond to the NRC-CDTCS: SubfunctionNotSupportedInActiveSession The value 0x7F shall correspond to the NRC-CDTCS: ServiceNotSupportedInActiveSession |  | DAI_EXT_TF_E_9986; DAI_EXT_TF_E_9989; DAI_EXT_TF_E_9990; DAI_EXT_TF_E_9991; DAI_EXT_TF_E_9992; DAI_EXT_TF_E_9993 |

## RoELite_ObsolescenceDataHandler

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(void, DiagFunction_CODE) RoELite_ObsolescenceDataHandler(void) |  |  |  |
| Object |  |  |  |
| Function collects the ObsolescenceData provided by EOL and returns it on the RoELite PDU Structure |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0420; ARCH_SW_DIA_0421; ARCH_SW_DIA_0422; ARCH_SW_DIA_0423; ARCH_SW_DIA_0424; ARCH_SW_DIA_0425; ARCH_SW_DIA_0426 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0421 | Frame contents shall be transmitted depending on the event detect (Type). |  | DAI_EXT_TF_B_2074 |
| ARCH_SW_DIA_0422 | If more than one RoE-lite PDU needs to be transmitted within one Transmission Cycle, the associated RoE-Lite PDUs shall be transmitted as depicted in figure [Transmitting RoE-lite PDUs from the transmission queue]. |  | DAI_EXT_TF_B_2120 |
| ARCH_SW_DIA_0423 | ROE_0_RBTM_xy_ST3 (Byte 1) signal shall be transmitted with the value 0x02 - Obsolescence_DATA if availability data has been changed. Byte 2 of the ROE_RBTM_xy_ST3 frame shall contain the block number. Bytes 3-4 of the ROE_RBTM_xy_ST3 frame shall contain the current value Bytes 5-6 of the ROE_RBTM_xy_ST3 frame shall be threshold "Green to Yellow" Bytes 7-8 of the ROE_RBTM_xy_ST3 frame shall be threshold "Yellow to Red" |  | DAI_EXT_TF_B_2296 |
| ARCH_SW_DIA_0424 | Block number: BSR: 0x00 PRE-SAFE 1-3: 0x01 PRE-SAFE 4-5: 0x02 PRE-SAFE 6-8: 0x03 |  | DAI_EXT_TF_B_2353 |
| ARCH_SW_DIA_0425 | Threshold "Green to Yellow" = "Yellow to Red" = 0 |  | DAI_EXT_TF_B_2354 |
| ARCH_SW_DIA_0426 | Only when the Obsolescence changes from - “Green” to “Red” the ECU shall send the corresponding RoE-Lite PDU. |  | DAI_EXT_TF_B_2356 |

## RoELite_AvailabilityDataHandler

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(void, DiagFunction_CODE) RoELite_AvailabilityDataHandler(void) |  |  |  |
| Object |  |  |  |
| Function collects the AvailabilityData provided by MMG and returns it on the RoELite PDU Structure |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0430; ARCH_SW_DIA_0431; ARCH_SW_DIA_0432; ARCH_SW_DIA_0433 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0431 | ROE_0_RBTM_xy_ST3 (Byte 1) signal shall be transmitted with the value 0x04 - AVAILABILITY_DATA if availability data has been changed. Byte 2 of the ROE_RBTM_xy_ST3 frame shall contain the function number. Bytes 3 of the ROE_RBTM_xy_ST3 frame shall contain the category (yelow, green, red) Bytes 4-8 of the ROE_RBTM_xy_ST3 frame shall be 0x0000000000. |  | DAI_EXT_TF_B_2082 |
| ARCH_SW_DIA_0432 | Function number: BSR: 0x00 PRE-SAFE 1-3: 0x00 PRE-SAFE 4-5: 0x00 PRE-SAFE 6-8: 0x00 |  | DAI_EXT_TF_B_2355 |
| ARCH_SW_DIA_0433 | Only when the Availability changes from - “Green” to “Red” the ECU shall send the corresponding RoE-Lite PDU. |  | DAI_EXT_TF_B_2126 |

## DiagSessionChangeCallback

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(void, DCM_CALLOUT_CODE) DiagSessionChangeCallback(Dcm_SesCtrlType formerState, Dcm_SesCtrlType newState) |  |  |  |
| Object |  |  |  |
| Function takes from DCM any change of the Diag Session and returns it on the RoELite PDU Structure |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| formerState | Dcm_SesCtrlType | In |  |
| newState | Dcm_SesCtrlType | In/Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| roe_DiagSessionChange.ROE_X | RoELite PDU Structure |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0435; ARCH_SW_DIA_0436; ARCH_SW_DIA_0437; ARCH_SW_DIA_0438 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0436 | ROE_0_RBTM_xy_ST3 (Byte 1) signal shall be transmitted with the value 05 - DIAG_SESSION_CHANGE if diagnostic sesion has been changed. Byte 2 of the ROE_RBTM_xy_ST3 frame shall contain the new Diagnostic session. Bytes 3-8 of the ROE_RBTM_xy_ST3 frame shall be 0x000000000000. |  | DAI_EXT_TF_B_2083 |
| ARCH_SW_DIA_0437 | On ECU start-up the ECU shall transmit a RoE-lite PDU containing the DiagnosticSessionType of the target session. (This includes ECU start-ups for example due to ECUReset or watchdog resets.) |  | DAI_EXT_TF_B_2128 |
| ARCH_SW_DIA_0438 | RoE-lite PDUs indicating Diagnostic Session Changes shall only be sent from Application Software, not from Boot Software. |  | DAI_EXT_TF_B_2129 |

## DataServices_Global_Time_Sync_Measured_Values_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Global_Time_Sync_Measured_Values_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of Global Time Sync Measured Values |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0460 |  |  |  |

## DataServices_Global_Time_Sync_Measured_Values_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Global_Time_Sync_Measured_Values_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function calculates the Global Time Sync Measured Values |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0460; ARCH_SW_DIA_0461; ARCH_SW_DIA_0462; ARCH_SW_DIA_0463; ARCH_SW_DIA_0464; ARCH_SW_DIA_0465 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0461 | DataIdentifier shall be 0x0107 |  | DAI_EXT_TF_E_9734 |
| ARCH_SW_DIA_0462 | The DID shall be available in the following sessions: ⦁Default Session ⦁Programming ⦁Extended session ⦁Update Session ⦁Development Session |  | DAI_EXT_TF_E_9743 |
| ARCH_SW_DIA_0463 | This service shall be available with or without Any form of Authentication. |  | DAI_EXT_TF_E_9941 |
| ARCH_SW_DIA_0464 | Byte 0 of the positive response shall represent the Source Domain Information for Interpretation |  | DAI_EXT_TF_E_9775 |
| ARCH_SW_DIA_0465 | Bytes 1-201 of the positive response shall represent the Global Time Sync Measured Values. |  | DAI_EXT_TF_E_9776 |

## DataServices_Global_Time_Sync_Measured_Values_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Global_Time_Sync_Measured_Values_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the length of the Global Time Sync Measured Values |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint201 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0460; ARCH_SW_DIA_0466 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0466 | Response data shall be 201 bytes long. |  | DAI_EXT_TF_E_9774 |

## DIA_runDid0210_CAN_output_Read

### DIA_runDid0210_CAN_output_Read_CANoutput1_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0210_CAN_output_Read_CANoutput1_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

### DIA_runDid0210_CAN_output_Read_CANoutput1_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE)DIA_runDid0210_CAN_output_Read_CANoutput1_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the Presafe Display Request and Belt Slack Decreasing Mode |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0510; ARCH_SW_DIA_0511; ARCH_SW_DIA_0543; ARCH_SW_DIA_0544; ARCH_SW_DIA_0545; ARCH_SW_DIA_0546; ARCH_SW_DIA_0547; ARCH_SW_DIA_0548; ARCH_SW_DIA_0549; ARCH_SW_DIA_0550; ARCH_SW_DIA_0551 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0511 | Response data shall contain Presafe Display Request, Belt Slack Decreasing Mode. |  | DAI_EXT_TF_E_10464 |
| ARCH_SW_DIA_0543 | The last value of BSR Function Mode BltSlckDec_Md_FR_Stat_ST3) shall keep over a next power cycle ( any type of reset ) |  | DAI_EXT_TF_B_2310 |
| ARCH_SW_DIA_0544 | If a diagnostic service request is received via CAN Frame DIAG_RBTM_FL_03_ExtCAN_RQ_ST3 then RGS shall respond via DIAG_RBTM_FL_03_ExtCAN_RS_ST3 (Id: 0x18E1A481) frame |  | DAI_EXT_TF_B_2377 |
| ARCH_SW_DIA_0545 | DIAG_RBTM_FL_03_ExtCAN_RS_ST3 Frame size: variable max 8 bytes |  | DAI_EXT_TF_B_2378 |
| ARCH_SW_DIA_0546 | If a diagnostic service request is received via CAN Frame DIAG_RBTM_FR_03_ExtCAN_RQ_ST3 then RGS shall respond via DIAG_RBTM_FR_03_ExtCAN_RS_ST3 (Id: 0x18E1A581) frame |  | DAI_EXT_TF_B_2379 |
| ARCH_SW_DIA_0547 | DIAG_RBTM_FR_03_ExtCAN_RS_ST3 Frame size: variable max 8 bytes |  | DAI_EXT_TF_B_2380 |
| ARCH_SW_DIA_0548 | If a diagnostic service request is received via CAN Frame DIAG_RBTM_FL_03_IntEth_RS_ST3 then RGS shall respond via DIAG_RBTM_FL_03_IntEth_RS_ST3 (Id: 0x18E1A485) frame |  | DAI_EXT_TF_B_2381 |
| ARCH_SW_DIA_0549 | DIAG_RBTM_FL_03_IntEth_RS_ST3 Frame size: variable max 8 bytes |  | DAI_EXT_TF_B_2382 |
| ARCH_SW_DIA_0550 | If a diagnostic service request is received via CAN Frame DIAG_RBTM_FR_03_IntEth_RS_ST3 then RGS shall respond via DIAG_RBTM_FR_03_IntEth_RS_ST3 (Id: 0x18E1A585) frame |  | DAI_EXT_TF_B_2383 |
| ARCH_SW_DIA_0551 | DIAG_RBTM_FR_03_IntEth_RS_ST3 Frame size: variable max 8 bytes |  | DAI_EXT_TF_B_2384 |

### DIA_runDid0210_CAN_output_Read_PS_Curve_XX_Stat_ST3_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0210_CAN_output_Read_PS_Curve_XX_Stat_ST3_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

### DIA_runDid0210_CAN_output_Read_PS_Curve_XX_Stat_ST3_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0210_CAN_output_Read_PS_Curve_XX_Stat_ST3_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the PS_Curve_xy_Stat_ST3 signal |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0512 | Response data shall contain PS_Curve_xy_Stat_ST3 |  | DAI_EXT_TF_E_10475 |

### DIA_runDid0210_CAN_output_Read_RBTMxx_Actv_Lvl_ST35_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0210_CAN_output_Read_RBTMxx_Actv_Lvl_ST35_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

### DIA_runDid0210_CAN_output_Read_RBTMxx_Actv_Lvl_ST35_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0210_CAN_output_Read_RBTMxx_Actv_Lvl_ST35_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the RBTMxy_ACTV_LVL_ST35 signal |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0512 | Response data shall contain RBTMxy_ACTV_LVL_ST35 |  | DAI_EXT_TF_E_10473 |

### DIA_runDid0210_CAN_output_Read_RBTMxx_SP_ActvClient_ST35_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0210_CAN_output_Read_RBTMxx_SP_ActvClient_ST35_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

### DIA_runDid0210_CAN_output_Read_RBTMxx_SP_ActvClient_ST35_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0210_CAN_output_Read_RBTMxx_SP_ActvClient_ST35_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the RBTMxy_SP_ActvClient_ST35 signal |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0513 | Response data shall contain RBTMxy_SP_ActvClient_ST35 |  | DAI_EXT_TF_E_10474 |

### DIA_runDid0210_CAN_output_Read_RBTMxx_SP_FctPrecond_Stat_ST35_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0210_CAN_output_Read_RBTMxx_SP_FctPrecond_Stat_ST35_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

### DIA_runDid0210_CAN_output_Read_RBTMxx_SP_FctPrecond_Stat_ST35_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DIA_runDid0210_CAN_output_Read_RBTMxx_SP_FctPrecond_Stat_ST35_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the RBTMxy_SP_FCTPrecond_Stat_ST35 signal |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0514 | Response data shall contain RBTMxy_SP_FCTPrecond_Stat_ST35 |  | DAI_EXT_TF_E_10472 |

## DIA_runDidFD50_Hardware_Version_Information_Write

### DIA_runDidFD50_Hardware_Version_Information_Write_HW_patch_level

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD50_Hardware_Version_Information_Write_HW_patch_level(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function calculates the |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0515 | Runnable shall return 2 bytes, representing PATCH of HW version |  | DAI_EXT_TF_E_10339 |

### DIA_runDidFD50_Hardware_Version_Information_Write_HW_week

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD50_Hardware_Version_Information_Write_HW_week(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function calculates the |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0516 | Runnable shall return 2 bytes, representing WEEK of HW version |  | DAI_EXT_TF_E_10339 |

### DIA_runDidFD50_Hardware_Version_Information_Write_HW_year

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD50_Hardware_Version_Information_Write_HW_year(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function calculates the |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0517 | Runnable shall return 2 bytes, representing YEAR of HW version |  | DAI_EXT_TF_E_10339 |

## DIA_ runDid0220_HW_input_Read_ConditionCheckRead

### DIA_runDid0220_HW_input_Read_Voltage_KL30t_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0220_HW_input_Read_Voltage_KL30t_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 1st Bitfield of DID 0220 HW Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0555 |  |  |  |

### DIA_runDid0220_HW_input_Read_Voltage_Motor_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0220_HW_input_Read_Voltage_Motor_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 2nd Bitfield of DID 0220 HW Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0555 |  |  |  |

## DIA_ runDid0220_HW_input_Read

### DIA_runDid0220_HW_input_Read_Voltage_KL30t_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0220_HW_input_Read_Voltage_KL30t_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 1st Bitfield for DID 0220 HW Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0555; ARCH_SW_DIA_0556; ARCH_SW_DIA_0557; ARCH_SW_DIA_0558; ARCH_SW_DIA_0559; ARCH_SW_DIA_0560; ARCH_SW_DIA_0561; ARCH_SW_DIA_0562 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0556 | The Read HW Input service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) ⦁Update Session (0x42)Update Session (0x42) |  | DAI_EXT_TF_E_10561 |
| ARCH_SW_DIA_0557 | The Read HW Input service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_10562 |
| ARCH_SW_DIA_0558 | DataIdentifier of the Read HW Input shall be: 0x0220 |  | DAI_EXT_TF_E_10568 |
| ARCH_SW_DIA_0559 | Response data length shall be 5 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10571 |
| ARCH_SW_DIA_0560 | Bytes #2-#3: DataIdentifier of the Read HW Input shall be: 0x0220 |  | DAI_EXT_TF_E_10573 |
| ARCH_SW_DIA_0561 | Byte #4: ⦁bits 0-7: Voltage KL30t Any value from 0x00 -> 0xFF Offset is 0 (i.e. 0x00 = 0V) Resolution is 0.1 (e.g. 0x10 = 1.6 V) Range is 0V -> 25.4V |  | DAI_EXT_TF_E_10574 |
| ARCH_SW_DIA_0562 | Read HW Input service shall return the Voltage measured. |  | DAI_EXT_TF_E_10577 |

### DIA_runDid0220_HW_input_Read_Voltage_Motor_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0220_HW_input_Read_Voltage_Motor_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 2nd Bitfield for DID 0220 HW Input Read |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0555; ARCH_SW_DIA_0556; ARCH_SW_DIA_0557; ARCH_SW_DIA_0558; ARCH_SW_DIA_0559; ARCH_SW_DIA_0560; ARCH_SW_DIA_0561; ARCH_SW_DIA_0563 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0563 | Byte #5: ⦁bits 0-7: Voltage Motor Any value from 0x00 -> 0xFF Offset is 0 (i.e. 0x00 = 0V) Resolution is 0.1 (e.g. 0x10 = 1.6 V) Range is 0V -> 25.4V |  | DAI_EXT_TF_E_10575 |

## DIA_ runDid0240_Internal_ECU_Information_Read_ ConditionCheckRead

### DIA_runDid0240_Internal_ECU_information_Read_Autosar_System_State_EcuM_State_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0240_Internal_ECU_information_Read_Autosar_System_State_EcuM_State_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 1st Bitfield of DID 0240 Internal ECU Information |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0565 |  |  |  |

### DIA_runDid0240_Internal_ECU_information_Read_ECU_Temperature_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0240_Internal_ECU_information_Read_ECU_Temperature_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 2nd Bitfield of DID 0240 Internal ECU Information |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0565 |  |  |  |

### DIA_runDid0240_Internal_ECU_information_Read_MOTOR_Current_ConditionCheckRead

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0240_Internal_ECU_information_Read_MOTOR_Current_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition for the 3rd Bitfield of DID 0240 Internal ECU Information |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0565 |  |  |  |

## DIA_ runDid0240_Internal_ECU_Information_ Read

### DIA_runDid0240_Internal_ECU_information_Read_Autosar_System_State_EcuM_State_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0240_Internal_ECU_information_Read_Autosar_System_State_EcuM_State_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 1st Bitfield for DID 0240 Internal ECU Information |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0565; ARCH_SW_DIA_0566; ARCH_SW_DIA_0567; ARCH_SW_DIA_0568; ARCH_SW_DIA_0569; ARCH_SW_DIA_0570; ARCH_SW_DIA_0571; ARCH_SW_DIA_0572 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0566 | The Read Internal ECU Information service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) ⦁Update Session (0x42) |  | DAI_EXT_TF_E_10582 |
| ARCH_SW_DIA_0567 | The Read Internal ECU Information service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_10583 |
| ARCH_SW_DIA_0568 | DataIdentifier of the Read Internal ECU Information shall be: 0x0240 |  | DAI_EXT_TF_E_10587 |
| ARCH_SW_DIA_0569 | Response data length shall be 6 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10590 |
| ARCH_SW_DIA_0570 | Bytes #2-#3: DataIdentifier of the Read Internal ECU Information shall be: 0x0240 |  | DAI_EXT_TF_E_10592 |
| ARCH_SW_DIA_0571 | Byte #4: ⦁bits 0-7: Autosar System State (EcuM_State) 0x10 - Startup 0x20 - Wakeup 0x30 - Run 0x40 - Shutdown 0x50 - Sleep |  | DAI_EXT_TF_E_10593 |
| ARCH_SW_DIA_0572 | Read Internal ECU Information service shall return the values measured. |  | DAI_EXT_TF_E_10597 |

### DIA_runDid0240_Internal_ECU_information_Read_ECU_Temperature_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0240_Internal_ECU_information_Read_ECU_Temperature_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 2nd Bitfield for DID 0240 Internal ECU Information |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0565; ARCH_SW_DIA_0566; ARCH_SW_DIA_0567; ARCH_SW_DIA_0568; ARCH_SW_DIA_0569; ARCH_SW_DIA_0570; ARCH_SW_DIA_0571; ARCH_SW_DIA_0573 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0573 | Bytes #5-#6: ⦁bits 0-7: ECU Temperature Any value from 0x0A -> 0x043F Offset is -52.7875 (i.e. 0x0A = -50 V) Resolution is 0.2325 (e.g. 0x10 = -49.0675 V) Range is -50 deg C -> 200 deg C Range 0x00 -> 0x09 is Undefined Range 0x0440 -> 0xFFFF is Undefined |  | DAI_EXT_TF_E_10594 |

### DIA_runDid0240_Internal_ECU_information_Read_MOTOR_Current_ReadData

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0240_Internal_ECU_information_Read_MOTOR_Current_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function returns the 3rd Bitfield for DID 0240 Internal ECU Information |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0565; ARCH_SW_DIA_0566; ARCH_SW_DIA_0567; ARCH_SW_DIA_0568; ARCH_SW_DIA_0569; ARCH_SW_DIA_0570; ARCH_SW_DIA_0571; ARCH_SW_DIA_0574 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0574 | Bytes #7-#8: ⦁bits 0-7: Motor Current Any value from 0x00 -> 0xFF Offset is 0 (i.e. 0x00 = 0A) Resolution is 0.01 (e.g. 0x100 = 16 A) Range is 0A -> 163.83A |  | DAI_EXT_TF_E_10595 |

## DIA_runDidFD03_ECUTraceablityNumberConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD03_ECUTraceablityNumberConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of ECU Traceability Number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0575 |  |  |  |

## DIA_runDidFD03_ECUTraceablityNumberRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD03_ECUTraceablityNumberRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the ECU Traceability Number stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0575; ARCH_SW_DIA_0576; ARCH_SW_DIA_0577 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0576 | The Read AEE Traceability Number And PCBA programmed PN shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) ⦁Update Session (0x42) |  | DAI_EXT_TF_E_8900 |
| ARCH_SW_DIA_0577 | The Read AEE Traceability Number And PCBA programmed PN shall be processed without Authentification. |  | DAI_EXT_TF_E_8901 |

## DIA_runDidFD03_PCBAProgrammedPartNumberConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD03_ECUTraceablityNumberConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of PCBA Programmed Part Number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0575 |  |  |  |

## DIA_runDidFD03_PCBAProgrammedPartNumberRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD03_ECUTraceablityNumberRead(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the ECU Traceability Number stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0575; ARCH_SW_DIA_0578; ARCH_SW_DIA_0579; ARCH_SW_DIA_0580; ARCH_SW_DIA_0581; ARCH_SW_DIA_0582 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0578 | Data Identifier of the Read AEE Traceability Number And PCBA programmed PN Service shall be: 0xFD03 |  | DAI_EXT_TF_E_8905 |
| ARCH_SW_DIA_0579 | Response data length shall be 22 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11342 |
| ARCH_SW_DIA_0580 | Bytes #2-#3: DataIdentifier of the Read AEE Traceability Number And PCBA programmed PN shall be: 0xFD03 |  | DAI_EXT_TF_E_8910 |
| ARCH_SW_DIA_0581 | Bytes #4-#15: ⦁AEE Traceability Number [ASCII Coded] |  | DAI_EXT_TF_E_8912 |
| ARCH_SW_DIA_0582 | Bytes #16-#25: ⦁PCBA programmed Part Number [ASCII Coded] |  | DAI_EXT_TF_E_11343 |

## DIA_runDidFD03_ECUTraceablityNumberWrite

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD03_ECUTraceablityNumberWrite(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType_1 OpStatus, P2VAR(Dcm_NegativeResponseCodeType_1, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function writes the Ecu Traceability Number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 |  |  |
| OpStatus | Dcm_OpStatusType_1 |  |  |
| ErrorCode | Dcm_NegativeResponseCodeType_1 |  |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0585; ARCH_SW_DIA_0586; ARCH_SW_DIA_0587; ARCH_SW_DIA_0588 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0586 | The Write AEE Traceability Number And PCBA programmed PN shall be available in the following sessions: ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11352 |
| ARCH_SW_DIA_0587 | The Write AEE Traceability Number And PCBA Programmed PN service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production |  | DAI_EXT_TF_E_11353 |
| ARCH_SW_DIA_0588 | Request data shall be 22 bytes long. Note: Full Request length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11357 |

## DIA_runDidFD03_PCBAProgrammedPartNumberWrite

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD03_PCBAProgrammedPartNumberWrite (P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType_1 OpStatus, P2VAR(Dcm_NegativeResponseCodeType_1, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function writes the PCBA Programmed Part Number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 |  |  |
| OpStatus | Dcm_OpStatusType |  |  |
| ErrorCode | Dcm_NegativeResponseCodeType |  |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0585; ARCH_SW_DIA_0589; ARCH_SW_DIA_0590; ARCH_SW_DIA_0591; ARCH_SW_DIA_0592; ARCH_SW_DIA_0593 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0589 | Bytes #2-#3: DataIdentifier of the Write AEE Traceability Number And PCBA programmed PN shall be: 0xFD03 |  | DAI_EXT_TF_E_11356 |
| ARCH_SW_DIA_0590 | Bytes #4-#15: ⦁AEE Traceability Number [ASCII Coded] |  | DAI_EXT_TF_E_11358 |
| ARCH_SW_DIA_0591 | Bytes #16-#25: ⦁PCBA programmed Part Number [ASCII Coded] |  | DAI_EXT_TF_E_11359 |
| ARCH_SW_DIA_0592 | Byte #1: SID-PR of the WriteDataByIdentifier service shall be 0x6E. |  | DAI_EXT_TF_E_11350 |
| ARCH_SW_DIA_0593 | Bytes #2-#3: Data Identifier of the Write AEE Traceability Number And PCBA programmed PN Service shall be: 0xFD03 |  | DAI_EXT_TF_E_11351 |

## DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of the Reprogramming Attempt Counter. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0595 |  |  |  |

## DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the Reprogramming Attempt Counter stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0595; ARCH_SW_DIA_0596; ARCH_SW_DIA_0597; ARCH_SW_DIA_0598 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0596 | The Read Reprogramming Attempt Counter service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10746 |
| ARCH_SW_DIA_0597 | The Read Reprogramming Attempt Counter service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11217 |
| ARCH_SW_DIA_0598 | DataIdentifier of the Read Reprogramming Attempt Counter shall be: 0x0100 |  | DAI_EXT_TF_E_10757 |

## DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Reprogramming_Attempt_Counter_Read_Reprogramming_Attempts_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the length of the Reprogramming Attempt Counter. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0595; ARCH_SW_DIA_0599; ARCH_SW_DIA_0600; ARCH_SW_DIA_0601; ARCH_SW_DIA_0602 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0599 | Response data length shall be (4 bytes* number of logical blocks) long. Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10759 |
| ARCH_SW_DIA_0600 | Byte #4 #5: ⦁Number Of Reprogramming Attempts default value: 0x0000 if ECU erased and Programmed by the ECU supplier |  | DAI_EXT_TF_E_11221 |
| ARCH_SW_DIA_0601 | Byte #6 #7: ⦁Max. Number Of Reprogramming Attempts |  | DAI_EXT_TF_E_11223 |
| ARCH_SW_DIA_0602 | The reprogramming attempt counter shall be incremented by each reprogramming attempt. |  | DAI_EXT_TF_E_11362 |

## DataServices_Vehicle_Odometer_in_Low_Resolution_Read_Vehicle_Odometer_in_Low_Resolution_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DataServices_Vehicle_Odometer_in_Low_Resolution_Read_Vehicle_Odometer_in_Low_Resolution_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of the Vehicle Odometer in Low Resolution. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0605 |  |  |  |

## DataServices_Vehicle_Odometer_in_Low_Resolution_Read_Vehicle_Odometer_in_Low_Resolution_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DataServices_Vehicle_Odometer_in_Low_Resolution_Read_Vehicle_Odometer_in_Low_Resolution_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the Vehicle Odometer in Low Resolution stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0605; ARCH_SW_DIA_0606; ARCH_SW_DIA_0607; ARCH_SW_DIA_0608; ARCH_SW_DIA_0609; ARCH_SW_DIA_0610; ARCH_SW_DIA_0611 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0606 | The Read Vehicle Odometer in Low Resolution service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11204 |
| ARCH_SW_DIA_0607 | The Read Vehicle Odometer in Low Resolution service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11205 |
| ARCH_SW_DIA_0608 | DataIdentifier of the Read Vehicle Odometer in Low Resolution shall be: 0x010A |  | DAI_EXT_TF_E_11207 |
| ARCH_SW_DIA_0609 | Response data length shall be 2 bytes long. Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11209 |
| ARCH_SW_DIA_0610 | Bytes #2-#3: DataIdentifier of the Read Vehicle Odometer in Low Resolution shall be: 0x010A |  | DAI_EXT_TF_E_11211 |
| ARCH_SW_DIA_0611 | Byte #4 #5: ⦁Vehicle Odometer In Low Resolution Offset is 0 (i.e. 0x00 = 0 km) Resolution is 16 (e.g. 0x10 = 16 km) Range is 0 km -> 1048544 km |  | DAI_EXT_TF_E_11212 |

## DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DataServices_Vehicle_Odometer_in_Low_Resolution_Read_Vehicle_Odometer_in_Low_Resolution_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of the Adjust ISO 15765-2 Block Size and STmin Parameter |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0615 |  |  |  |

## DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the Adjust ISO 15765-2 Block Size and STmin Parameter. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0615; ARCH_SW_DIA_0616; ARCH_SW_DIA_0617; ARCH_SW_DIA_0618; ARCH_SW_DIA_0619 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0616 | Description: Service Read Adjust ISO 15765-2 Block Size and STmin Parameter shall report the ISO 15762-2 Transport Protocol parameters such as BlockSize (BS) and minimum Separation Time which are transmitted within the FlowControl message. |  | DAI_EXT_TF_E_10601 |
| ARCH_SW_DIA_0617 | The Read Adjust ISO 15765-2 Block Size and STmin Parameter service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10604 |
| ARCH_SW_DIA_0618 | The Read Adjust ISO 15765-2 Block Size and STmin Parameter service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_10605 |
| ARCH_SW_DIA_0619 | DataIdentifier of the Read Adjust ISO 15765-2 Block Size and STmin Parameter shall be: 0x010B |  | DAI_EXT_TF_E_10609 |

## DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Read Condition of the Adjust ISO 15765-2 Block Size and STmin Parameter. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0615 |  |  |  |

## DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function reads the Adjust ISO 15765-2 Block Size and STmin Parameter. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0615; ARCH_SW_DIA_0620; ARCH_SW_DIA_0621 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0620 | Response data length shall be 2 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10612 |
| ARCH_SW_DIA_0621 | Bytes #4 #5: ⦁Block Size Value as defined in ISO 15765-2 ⦁STmin Value as defined in ISO 15765-2 |  | DAI_EXT_TF_E_10615 |

## DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_Block_Size_Value_WriteData(P2CONST(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function writes the Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 |  |  |
| OpStatus | Dcm_OpStatusType |  |  |
| ErrorCode | Dcm_NegativeResponseCodeType |  |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0625; ARCH_SW_DIA_0626; ARCH_SW_DIA_0627; ARCH_SW_DIA_0628 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0626 | The Write Adjust ISO 15765-2 Block Size and STmin Parameter service shall be available in the following sessions: ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11075 |
| ARCH_SW_DIA_0627 | The Write Adjust ISO 15765-2 Block Size and STmin Parameter service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11076 |
| ARCH_SW_DIA_0628 | Bytes #2-#3: DataIdentifier of the Write Adjust ISO 15765-2 Block Size and STmin Parameter shall be: 0x010B |  | DAI_EXT_TF_E_11327 |

## DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter_Read_STmin_Value_WriteData (P2CONST(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function writes the Adjust_ISO_15765_2_Block_Size_and_STmin_Parameter. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 |  |  |
| OpStatus | Dcm_OpStatusType |  |  |
| ErrorCode | Dcm_NegativeResponseCodeType |  |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0625; ARCH_SW_DIA_0629; ARCH_SW_DIA_0630; ARCH_SW_DIA_0631 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0629 | Request data length shall be 2 bytes long Note: Full Request length = 3 bytes + Request data length |  | DAI_EXT_TF_E_11078 |
| ARCH_SW_DIA_0630 | Byte #4: ⦁bits 0-7: Block Size Value as defined in ISO 15765-2 |  | DAI_EXT_TF_E_11079 |
| ARCH_SW_DIA_0631 | Byte #5: ⦁bits 0-7: STmin Value as defined in ISO 15765-2 |  | DAI_EXT_TF_E_11080 |

## DataServices_Task_Response_Time_Read_Task_histograms_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_Task_Response_Time_Read_Task_histograms_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Task Response Time Condition. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0635 |  |  |  |

## DataServices_Task_Response_Time_Read_Task_histograms_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_Task_Response_Time_Read_Task_histograms_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that calculates Task Response Time per each task in a histogram type of calculation. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0635; ARCH_SW_DIA_0636; ARCH_SW_DIA_0637; ARCH_SW_DIA_0638 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0636 | The Read Task Response Time service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10620 |
| ARCH_SW_DIA_0637 | The Read Task Response Time service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Internal Diagnostic Test Tool |  | DAI_EXT_TF_E_10621 |
| ARCH_SW_DIA_0638 | DataIdentifier of the Read Task Response Time shall be: 0x01E0 |  | DAI_EXT_TF_E_10625 |

## DataServices_Task_Response_Time_Read_Task_histograms_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_Task_Response_Time_Read_Task_histograms_ReadDataLength (Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the Length of the Task Response Time for each task. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | uint176 | Out | Note: For each task there will be 16 bytes on response meaning that the total length for each task is 16 bytes x 11 tasks = 176 bytes |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0635; ARCH_SW_DIA_0639; ARCH_SW_DIA_0640 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_0639 | Response data length shall be 16 bytes for each task. Note: Full Response length = 3 bytes + Response data length Note: For each task there will be 16 bytes on response meaning that the total length for each task is 16 bytes x 11 tasks = 176 bytes |  | DAI_EXT_TF_E_10628 |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0640 | Bytes #4 - #15 = Task Histogram [16 bytes]: ⦁[Byte0] Task ID ⦁[Byte1] 0-19%, PercentUpdate ⦁[Byte2] 20-39%, PercentUpdate ⦁[Byte3] 40-59%, PercentUpdate ⦁[Byte4] 60-79%, PercentUpdate ⦁[Byte5] 80-99%, PercentUpdate ⦁[Byte6] 100-119%, PercentUpdate ⦁[Byte7] 120-139%, PercentUpdate ⦁[Byte8] 140-159%, PercentUpdate ⦁[Byte9] 160-179%, PercentUpdate ⦁[Byte10] 180-199%, PercentUpdate ⦁[Byte11] >=200%, PercentUpdate ⦁[Byte12-Byte15] Max us, Microseconds |  | DAI_EXT_TF_E_10631 |

## DataServices_System_Time_read_Read_System_time_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_System_Time_read_Read_System_time_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the System Time Read Condition. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0645 |  |  |  |

## DataServices_System_Time_read_Read_System_time_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) DataServices_System_Time_read_Read_System_time_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that retrieves the System Time stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0645; ARCH_SW_DIA_0646; ARCH_SW_DIA_0647; ARCH_SW_DIA_0648; ARCH_SW_DIA_0649; ARCH_SW_DIA_0650 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0646 | The Read System Time service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10778 |
| ARCH_SW_DIA_0647 | The Read System Time service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11228 |
| ARCH_SW_DIA_0648 | DataIdentifier of the Read System Time shall be: 0x02EA |  | DAI_EXT_TF_E_10776 |
| ARCH_SW_DIA_0649 | Response data length shall be 4 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10783 |
| ARCH_SW_DIA_0650 | Bytes #4 to #7 ⦁ bits 0-7: System Time [BCD coded] |  | DAI_EXT_TF_E_11232 |

## DiagCommonData – EF00 / EF01 / EF03 / EF05 / F100 / F104 / F10D / F111 / F121 / F154 / F155

### EF00 - ECU Extract Version

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function that retrieves the version information of the ECU Extract. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0656; ARCH_SW_DIA_0657; ARCH_SW_DIA_0658; ARCH_SW_DIA_0659; ARCH_SW_DIA_0660 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0656 | The Read ECU Extract Version service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10830 |
| ARCH_SW_DIA_0657 | The Read ECU Extract Version service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_10831 |
| ARCH_SW_DIA_0658 | DataIdentifier of the Read ECU Extract Version shall be: 0xEF00 |  | DAI_EXT_TF_E_10828 |
| ARCH_SW_DIA_0659 | Response data length shall be 54 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10835 |
| ARCH_SW_DIA_0660 | Byte #4: ⦁bits 0-7: ECU Extract Major Version Byte #5: ⦁bits 0-7: ECU Extract Minor Version Byte #6: ⦁bits 0-7: ECU Extract Patch Version Bytes #7: ⦁bits 0-7: Identification Type (Constant = 0x00) Note: This is not returned on response, instead it is just for EF00 Identification between the CDD Routines. Byte #8-#17: ⦁bits 0-7: XDIS Version Byte #18-#27: ⦁bits 0-7: Export Date Byte #28-#57: ⦁bits 0-7: ECU Name |  | DAI_EXT_TF_E_11237; DAI_EXT_TF_E_11240; DAI_EXT_TF_E_11241; DAI_EXT_TF_E_11242; DAI_EXT_TF_E_11243; DAI_EXT_TF_E_11244; DAI_EXT_TF_E_11245 |

### EF01 - Diagnostic Extract Version

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function that retrieves the version information of the CANdela File (CDD) and the Diagnostic Extract. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0661; ARCH_SW_DIA_0662; ARCH_SW_DIA_0663; ARCH_SW_DIA_0664; ARCH_SW_DIA_0665 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0661 | The Read Diagnostic Extract Version service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10843 |
| ARCH_SW_DIA_0662 | The Read Diagnostic Extract Version service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11247 |
| ARCH_SW_DIA_0663 | DataIdentifier of the Read Diagnostic Extract Version shall be: 0xEF01 |  | DAI_EXT_TF_E_10841 |
| ARCH_SW_DIA_0664 | Response data length shall be 53 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10848 |
| ARCH_SW_DIA_0665 | Byte #4: ⦁bits 0-7: Diagnostic Extract Major Version Byte #5: ⦁bits 0-7: Diagnostic Extract Minor Version Byte #6: ⦁bits 0-7: Diagnostic Extract Patch Version Bytes #7-#16: ⦁bits 0-7: CANDela Version Bytes #17-#26: ⦁bits 0-7: DDM Version Bytes #27-#56: ⦁bits 0-7: Exported Diagnostic Variant |  | DAI_EXT_TF_E_10849; DAI_EXT_TF_E_10850; DAI_EXT_TF_E_10851; DAI_EXT_TF_E_10852; DAI_EXT_TF_E_10853; DAI_EXT_TF_E_10854 |

### EF03 - Standard Application SW Package Information

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function shall report information regarding the Autosar Platform, Software Package Version, the µC Identification and Supplier Identification of the Application Software Communication stack. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0666; ARCH_SW_DIA_0667; ARCH_SW_DIA_0668; ARCH_SW_DIA_0669; ARCH_SW_DIA_0670 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0666 | The Read Standard Application SW Package Information service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10862 |
| ARCH_SW_DIA_0667 | The Read Standard Application SW Package Information service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁Internal Diagnostic Test Tool |  | DAI_EXT_TF_E_11251 |
| ARCH_SW_DIA_0668 | DataIdentifier of the Read Standard Application SW Package Information shall be: 0xEF03 |  | DAI_EXT_TF_E_10860 |
| ARCH_SW_DIA_0669 | Response data length shall be 17 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10867 |
| ARCH_SW_DIA_0670 | Byte #4: ⦁bits 0-7: Standard Application SW Package Major Version Byte #5: ⦁bits 0-7: Standard Application SW Package Minor Version Byte #6: ⦁bits 0-7: Standard Application SW Package Patch Version Byte #7: ⦁bits 0-7: Standard Application SW Package Build Version Bytes #8-#17: ⦁bits 0-7: µC ID Bytes #18-#19: ⦁bits 0-7: Supplier Identification Byte #20: ⦁bits 0-7: AUTOSAR Platform |  | DAI_EXT_TF_E_11255; DAI_EXT_TF_E_10869; DAI_EXT_TF_E_10870; DAI_EXT_TF_E_10871; DAI_EXT_TF_E_10872; DAI_EXT_TF_E_10873; DAI_EXT_TF_E_10874 |

### EF05 - Minimum SW Version

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function shall read Minimum SW Version for each logical block, that can be flashed to this ECU. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0671; ARCH_SW_DIA_0672; ARCH_SW_DIA_0673; ARCH_SW_DIA_0674; ARCH_SW_DIA_0675 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0671 | The Read Minimum SW Version service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10882 |
| ARCH_SW_DIA_0672 | The Read Minimum SW Version service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁Internal Diagnostic Test Tool |  | DAI_EXT_TF_E_11256 |
| ARCH_SW_DIA_0673 | DataIdentifier of the Read Minimum SW Version shall be: 0xEF05 |  | DAI_EXT_TF_E_10880 |
| ARCH_SW_DIA_0674 | Response data length shall be 3 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10893 |
| ARCH_SW_DIA_0675 | Byte #4: ⦁bits 0-7: SW Year Byte #5: ⦁bits 0-7: SW Week Byte #6: ⦁bits 0-7: SW Patch Level |  | DAI_EXT_TF_E_11260; DAI_EXT_TF_E_11261; DAI_EXT_TF_E_11262 |

### F100 - Active Diagnostic Information

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function shall read information about the Diagnostic Identification (current Diagnostic Status, Diagnostic Variant and Version) and the current Diagnostic Session of an ECU. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0676; ARCH_SW_DIA_0677; ARCH_SW_DIA_0678; ARCH_SW_DIA_0679; ARCH_SW_DIA_0680; ARCH_SW_DIA_0681 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0676 | The Read Active Diagnostic Information shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10906 |
| ARCH_SW_DIA_0677 | The Read Active Diagnostic Information service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11263 |
| ARCH_SW_DIA_0678 | DataIdentifier of the Read Active Diagnostic Information shall be: 0xF100 |  | DAI_EXT_TF_E_10904 |
| ARCH_SW_DIA_0679 | Response data length shall be 4 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10910 |
| ARCH_SW_DIA_0680 | Bytes #2-#3: DataIdentifier of the Read Active Diagnostic Information shall be: 0xF100 |  | DAI_EXT_TF_E_11266 |
| ARCH_SW_DIA_0681 | Byte #4: ⦁bits 0-7: Active Diagnostic Status 0 = Application / 1 = Bootloader Byte #5: ⦁bits 0-7: Active Diagnostic Variant Byte #6: ⦁bits 0-7: Active Diagnostic Version Byte #7: ⦁bits 0-7: Active Diagnostic Sesion -- 0x01: Default -- 0x02: Programming -- 0x04: Extended |  | DAI_EXT_TF_E_10911; DAI_EXT_TF_E_10912; DAI_EXT_TF_E_10913; DAI_EXT_TF_E_11363 |

### F104 - ECU Name

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function is used to retrieve the name of the CANdela file from the ECU. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0682; ARCH_SW_DIA_0683; ARCH_SW_DIA_0684; ARCH_SW_DIA_0685; ARCH_SW_DIA_0686 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0682 | The Read ECU Name shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10935 |
| ARCH_SW_DIA_0683 | The Read ECU Name service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11267 |
| ARCH_SW_DIA_0684 | DataIdentifier of the Read ECU Name shall be: 0xF104 |  | DAI_EXT_TF_E_10933 |
| ARCH_SW_DIA_0685 | Response data length shall be 1 byte long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10939 |
| ARCH_SW_DIA_0686 | Byte #4: Diagnostic ECU Name coded in ASCII Left ECU - RBTMFL174 Right ECU - RBTMFR174 |  | DAI_EXT_TF_E_11270 |

### F10D - DDS Package Release

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function shall provide the release information (YY-MM) of the MBDS Package (Mercedes-Benz Diagnostic Specification) according to which the ECU diagnostic software was developed. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0687; ARCH_SW_DIA_0688; ARCH_SW_DIA_0689; ARCH_SW_DIA_0690; ARCH_SW_DIA_0691 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0687 | The Read DDS Package Release shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10947 |
| ARCH_SW_DIA_0688 | The Read DDS Package Release service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Internal Diagnostic Test Tool |  | DAI_EXT_TF_E_11273 |
| ARCH_SW_DIA_0689 | DataIdentifier of the Read DDS Package Release shall be: 0xF10D |  | DAI_EXT_TF_E_10945 |
| ARCH_SW_DIA_0690 | Response data length shall be 4 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10951 |
| ARCH_SW_DIA_0691 | Byte #4: ⦁bits 0-7: Year Application DDS Package Release Byte #5: ⦁bits 0-7: Month Application DDS Package Release Byte #6: ⦁bits 0-7: Year Boot DDS Package Release Byte #7: ⦁bits 0-7: Month Boot DDS Package Release |  | DAI_EXT_TF_E_11277; DAI_EXT_TF_E_10952; DAI_EXT_TF_E_10953; DAI_EXT_TF_E_11278 |

### F111 - MBC ECU Identification - Hardware Part Number

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function shall report the Hardware Part Number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0692; ARCH_SW_DIA_0693; ARCH_SW_DIA_0694; ARCH_SW_DIA_0695; ARCH_SW_DIA_0696; ARCH_SW_DIA_1040; ARCH_SW_DIA_1041; ARCH_SW_DIA_1042 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0692 | The Read MBC ECU Identification - Hardware Part Number shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_10965 |
| ARCH_SW_DIA_0693 | The Read MBC ECU Identification - Hardware Part Number service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11279 |
| ARCH_SW_DIA_0694 | DataIdentifier of the Read MBC ECU Identification - Hardware Part Number shall be: 0xF111 |  | DAI_EXT_TF_E_10963 |
| ARCH_SW_DIA_0695 | Response data length shall be 10 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_10969 |
| ARCH_SW_DIA_0696 | Bytes #2-#3: DataIdentifier of the Read MBC ECU Identification - Hardware Part Number shall be: 0xF111 Bytes #4-#13: Hardware Part Number [ASCII coded] |  | DAI_EXT_TF_E_11282; DAI_EXT_TF_E_11283 |
| ARCH_SW_DIA_1040 | Hardware Part Number shall not be overwritten during flashing process. |  | DAI_EXT_TF_E_11583 |
| ARCH_SW_DIA_1041 | Both Hardware Part Numbers for Left side and Right side shall be stored in ECU, and based on the ECU location shall be automatically alocated to that ECU |  | DAI_EXT_TF_E_11584 |
| ARCH_SW_DIA_1042 | Default Hardware Part Number if has not been written: Left Hardware Part Number: 0x30 0x30 0x30 0x30 0x30 0x30 0x30 0x30 0x30 0x30 Right Hardware Part Number: 0x30 0x30 0x30 0x30 0x30 0x30 0x30 0x30 0x30 0x30 |  | DAI_EXT_TF_E_11585 |

### F121 - MBC ECU Identification - Software Logical Block Part Number(s)

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function is used to report the Software Part Number(s). |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0697; ARCH_SW_DIA_0698; ARCH_SW_DIA_0699; ARCH_SW_DIA_0700 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0697 | The Read MBC ECU Identification - Software Logical Block Part Number(s) shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11284 |
| ARCH_SW_DIA_0698 | The Read MBC ECU Identification - Software Logical Block Part Number(s) service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11285 |
| ARCH_SW_DIA_0699 | DataIdentifier of the Read MBC ECU Identification - Software Logical Block Part Number(s) shall be: 0xF121 |  | DAI_EXT_TF_E_10976 |
| ARCH_SW_DIA_0700 | ⦁Bytes #4-#13: Software Logical Block # Part Number (shall be repetead for each software logical block) |  | DAI_EXT_TF_E_11289 |

### F154 - Hardware Supplier Identification

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function shall report the Hardware Supplier Identification of an ECU which determines the manufacturer of an ECU's hardware. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0701; ARCH_SW_DIA_0702; ARCH_SW_DIA_0703; ARCH_SW_DIA_0704; ARCH_SW_DIA_0705 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0701 | The Read Hardware Supplier Identification shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E |  | DAI_EXT_TF_E_11029 |
| ARCH_SW_DIA_0702 | The Read Hardware Supplier Identification service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11302 |
| ARCH_SW_DIA_0703 | DataIdentifier of the Read Hardware Supplier Identification shall be: 0xF154 |  | DAI_EXT_TF_E_11027 |
| ARCH_SW_DIA_0704 | Response data length shall be 2 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11033 |
| ARCH_SW_DIA_0705 | Byte #4: ⦁bits 0-7: Hardware Supplier Identification Byte #5: ⦁bits 0-7: Hardware Supplier Identification |  | DAI_EXT_TF_E_11306; DAI_EXT_TF_E_11307 |

### F155 - Software Supplier Identification

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| DiagCommonData {} |  |  |  |
| Object |  |  |  |
| Function shall read the Software Supplier Identification of an ECU. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Returned value |  |  |  |
| Name | Description |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0655; ARCH_SW_DIA_0706; ARCH_SW_DIA_0707; ARCH_SW_DIA_0708; ARCH_SW_DIA_0709; ARCH_SW_DIA_0710; ARCH_SW_DIA_0711 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0706 | DataIdentifier of the Software Supplier Identification shall be: 0xF155 |  | DAI_EXT_TF_E_11039 |
| ARCH_SW_DIA_0707 | The Read Software Supplier Identification shall be available in the following sessions: ⦁ Default Session (0x01) ⦁ Programming Session (0x02) ⦁ Extended Session (0x03) ⦁ Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11308 |
| ARCH_SW_DIA_0708 | The Read Software Supplier Identification service shall be available under the following forms of Authentication: ⦁ Supplier ⦁ Development ENHANCED ⦁ Production ⦁ After-Sales ENHANCED ⦁ After-Sales ⦁ After-Sales BASIC ⦁ Internal Diagnostic Test Tool ⦁ ePTI Test Tool |  | DAI_EXT_TF_E_11309 |
| ARCH_SW_DIA_0709 | Response data length shall be 2 bytes long for eack logical block Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11311 |
| ARCH_SW_DIA_0710 | Bytes #2-#3: DataIdentifier of the Read Software Supplier Identification shall be: 0xF155 |  | DAI_EXT_TF_E_11313 |
| ARCH_SW_DIA_0711 | Byte #4: ⦁bits 0-7: Software Logical Block Supplier Identification Byte #5: ⦁bits 0-7: Software Logical Block Supplier Identification |  | DAI_EXT_TF_E_11314; DAI_EXT_TF_E_11315 |

## DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Reprogramming Capabilities Read Service. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0715 |  |  |  |

## DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that returns the Reprogramming Capabilities stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0715; ARCH_SW_DIA_0716; ARCH_SW_DIA_0717; ARCH_SW_DIA_0718 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0716 | The Read Reprogramming Capabilities shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11290 |
| ARCH_SW_DIA_0717 | The Read Reprogramming Capabilities service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11291 |
| ARCH_SW_DIA_0718 | DataIdentifier of the Read Reprogramming Capabilities shall be: 0xF130 |  | DAI_EXT_TF_E_10992 |

## DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Reprogramming_capabilities_Read_Reprogramming_Capabilities_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the Length of the Reprogramming Capabilities DID. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint32 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0715; ARCH_SW_DIA_0719; ARCH_SW_DIA_0720 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_0719 | Bytes #2-#3: DataIdentifier of the Read Reprograming Capabilities shall be: 0xF130 |  | DAI_EXT_TF_E_11294 |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0720 | Byte #4: ⦁bits 0-7: Security Class Default Value = 0x03 (CCCv2) Byte #5: ⦁bits 0-7: Delta Update Support Byte #6-#35: ⦁bits 0-7: Delta Algorithm Byte #36: ⦁bits 0-7: OFR |  | DAI_EXT_TF_E_10999; DAI_EXT_TF_E_11000; DAI_EXT_TF_E_11295; DAI_EXT_TF_E_11296 |

## DataServices_Supported_configuration_mechanisms_Read_Config_mechanisms_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Supported_configuration_mechanisms_Read_Config_mechanisms_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Supported Configuration Mechanisms Read Service. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0725 |  |  |  |

## DataServices_Supported_configuration_mechanisms_Read_Config_mechanisms_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Supported_configuration_mechanisms_Read_Config_mechanisms_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that returns the Reprogramming Capabilities stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0725; ARCH_SW_DIA_0726; ARCH_SW_DIA_0727; ARCH_SW_DIA_0728; ARCH_SW_DIA_0729; ARCH_SW_DIA_0730 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0726 | The Read Supported Configuration Mechanisms shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11008 |
| ARCH_SW_DIA_0727 | The Read Supported Configuration Mechanisms service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11297 |
| ARCH_SW_DIA_0728 | DataIdentifier of the Read Supported Configuration Mechanisms shall be: 0xF131 |  | DAI_EXT_TF_E_11006 |
| ARCH_SW_DIA_0729 | Response data length shall be 1 byte long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11012 |
| ARCH_SW_DIA_0730 | Byte #4: ⦁bits 0-7: Configuration Mechanisms |  | DAI_EXT_TF_E_11301 |

## DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Software Fingerprint Read Service. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0735 |  |  |  |

## DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that returns the Software Fingerprint stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0735; ARCH_SW_DIA_0736; ARCH_SW_DIA_0737; ARCH_SW_DIA_0738 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0736 | The Read Software Fingerprint shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11059 |
| ARCH_SW_DIA_0737 | The Read Software Fingerprint service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11316 |
| ARCH_SW_DIA_0738 | DataIdentifier of the Read Software Fingerprint shall be: 0xF15B |  | DAI_EXT_TF_E_11057 |

## DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_ReadFingerprint_Read_Software_Fingerprint_s_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the Length of the Software Fingerprint DID. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint32 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0735; ARCH_SW_DIA_0739; ARCH_SW_DIA_0740 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_0739 | Response data length shall be 20 bytes* nb of logical blocks Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11063 |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0740 | Byte #4: ⦁Status Information - Logical Block #0 Bytes #5 -#20: ⦁Diagnostic Authentication Certificate ID Byte #21: ⦁Programming Date Year Byte #22: ⦁Programming Date Month Byte #23: ⦁Programming Date Day |  | DAI_EXT_TF_E_11320; DAI_EXT_TF_E_11321; DAI_EXT_TF_E_11322; DAI_EXT_TF_E_11323; DAI_EXT_TF_E_11324 |

## DataServices_Calibration_Identifications_CAL_ID_Write_CombinedDataElement_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Calibration_Identifications_CAL_ID_Write_CombinedDataElement_WriteData(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, uint16 DataLength, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function writes the Calibration Identifications CombinedDataElement Data. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 |  |  |
| OpStatus | Dcm_OpStatusType |  |  |
| ErrorCode | Dcm_NegativeResponseCodeType |  |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0745; ARCH_SW_DIA_0746; ARCH_SW_DIA_0747; ARCH_SW_DIA_0748 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0746 | The Write Calibration Identifications (CAL ID) service shall be available in the following sessions: ⦁Programming (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11183 |
| ARCH_SW_DIA_0747 | The Write Calibration Identifications (CAL ID) service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD |  | DAI_EXT_TF_E_11184 |
| ARCH_SW_DIA_0748 | Bytes #2-#3: DataIdentifier of the Write Calibration Identifications (CAL ID) shall be: 0xF804 |  | DAI_EXT_TF_E_11337 |

## DataServices_Calibration_Identifications_CAL_ID_Write_Number_of_data_items

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_MBBL_SsaAppl_Coding_CODE) DataServices_Calibration_Identifications_CAL_ID_Write_Number_of_data_items(P2CONST(uint8, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_MBBL_SSAAPPL_CODING_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function writes the Calibration Identifications Number of Data Items. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 |  |  |
| OpStatus | Dcm_OpStatusType |  |  |
| ErrorCode | Dcm_NegativeResponseCodeType |  |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0745; ARCH_SW_DIA_0749; ARCH_SW_DIA_0750 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0749 | Request data shall be 17 bytes long. Note: Full Request length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11186 |
| ARCH_SW_DIA_0750 | Byte #4: ⦁bits 0-7: Number of data items Bytes #5-#21: ⦁bits 0-7: Calibration Identification |  | DAI_EXT_TF_E_11187; DAI_EXT_TF_E_11188 |

## DataServices_Synchronize_to_Non_volatile_Memory_Start

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Synchronize_to_Non_volatile_Memory_Start(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StartDataOut_Routine_Synchronize_to_Non_volatile_Memory_Start_routineInfoType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) routineInfo, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function executes the 0245 Routine Activation. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| routineInfo | Dcm_StartDataOut_Routine_Synchronize_to_Non_volatile_Memory_Start_routineInfoType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0755; ARCH_SW_DIA_0756; ARCH_SW_DIA_0757; ARCH_SW_DIA_0758 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0756 | RoutineIdentifier shall be 0x0245. |  | DAI_EXT_TF_E_11130 |
| ARCH_SW_DIA_0757 | Routine Start 0x01 shall be available in the following diagnostic sessions: ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) Routine Start requires the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD |  | DAI_EXT_TF_E_11143 |
| ARCH_SW_DIA_0758 | Routine Start 0x01 shall synchronize all data currently not synchronized from volatile memory to non-volatile memory. |  | DAI_EXT_TF_E_11145 |

## DataServices_Synchronize_to_Non_volatile_Memory_Stop

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) Synchronize_to_Non_volatile_Memory_Stop(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StopDataOut_Routine_Synchronize_to_Non_volatile_Memory_Start_routineInfoType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) routineInfo, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function stops the execution of the 0245 Routine. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| routineInfo | Dcm_StartDataOut_Routine_Synchronize_to_Non_volatile_Memory_Start_routineInfoType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0755; ARCH_SW_DIA_0759; ARCH_SW_DIA_0760 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0759 | Routine Stop 0x02 shall be available in the following diagnostic sessions: ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) Routine Stop requires the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD |  | DAI_EXT_TF_E_11151 |
| ARCH_SW_DIA_0760 | Routine Stop 0x02 shall be responsible for deactivating the synchronization of all data |  | DAI_EXT_TF_E_11153 |

## DataServices_Synchronize_to_Non_volatile_Memory_RequestResults

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) Synchronize_to_Non_volatile_Memory_Stop(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StopDataOut_Routine_Synchronize_to_Non_volatile_Memory_Start_routineInfoType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) routineInfo, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function stops the execution of the 0245 Routine. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| routineInfo | Dcm_StartDataOut_Routine_Synchronize_to_Non_volatile_Memory_Start_routineInfoType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0755; ARCH_SW_DIA_0761; ARCH_SW_DIA_0762 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0761 | Routine Request Results 0x03 shall be available in the following diagnostic sessions: ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) Routine Stop requires the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD |  | DAI_EXT_TF_E_11157 |
| ARCH_SW_DIA_0762 | RID specific Routine Result shall be represented by the following: ⦁0x00 - Terminated successful. ⦁0x01 - Routine still running. ⦁0x02 - Stopped without result |  | DAI_EXT_TF_E_11162; DAI_EXT_TF_E_11163; DAI_EXT_TF_E_11164 |

## Ssa_ProcCtrl_RoutineServices_Routine_Replace_Trust_model_Certificates

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ssa_ProcCtrl_CODE) Ssa_ProcCtrl_RoutineServices_Routine_Replace_Trust_model_Certificates(P2CONST(uint8, AUTOMATIC, RTE_SSA_PROCCTRL_APPL_DATA) Trust_Certificates_In,Dcm_OpStatusType OpStatus,P2VAR(uint8, AUTOMATIC, RTE_SSA_PROCCTRL_APPL_VAR) routineInfo_Out,P2VAR(uint8, AUTOMATIC, RTE_SSA_PROCCTRL_APPL_VAR) Verification_Result_Out,uint16 DataLength,P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_SSA_PROCCTRL_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function executes the 0277 Routine. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Trust_Certificates_In | Uint8 | In |  |
| OpStatus | Dcm_OpStatusType | Out |  |
| routineInfo_Out | Uint8 | Out |  |
| Verification_Result_Out | Uint8 | Out |  |
| DataLength | Uint16 | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0765; ARCH_SW_DIA_0766; ARCH_SW_DIA_0767; ARCH_SW_DIA_0768; ARCH_SW_DIA_0769; ARCH_SW_DIA_0770 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0766 | RoutineIdentifier shall be 0x0277. |  | DAI_EXT_TF_E_11170 |
| ARCH_SW_DIA_0767 | The folowing request shall be available for this RID: - Start Routine |  | DAI_EXT_TF_E_11171 |
| ARCH_SW_DIA_0771 | Routine Start shall be available in the following diagnostic sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) ⦁Update Session (0x42) Routine Start requires the following forms of Authentication: ⦁Production |  | DAI_EXT_TF_E_11173 |
| ARCH_SW_DIA_0768 | Routine Start shall replace the currently stored Trust model ECU Root Certificate |  | DAI_EXT_TF_E_11175 |
| ARCH_SW_DIA_0769 | RID specific Control Options Record shall be represented by the following: ⦁Byte 0: Signature new Root ⦁Byte 64: Signature ⦁Byte128: Certificates |  | DAI_EXT_TF_E_11178 |
| ARCH_SW_DIA_0770 | RID specific Routine Start Result shall be represented by the following: ⦁Byte 0: Verification Result |  | DAI_EXT_TF_E_11177 |

## DataServices_Check_Reprogramming_Preconditions_Routine_Start

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Check_Reprogramming_Preconditions_Routine_Start(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StartDataOut_Routine_Check_Reprogramming_Preconditions_Start_routineInfoType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) routineInfo, P2VAR(Dcm_StartDataOut_Routine_Check_Reprogramming_Preconditions_Start_Number_of_Unfulfilled_PreconditionsType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Number_of_Unfulfilled_Preconditions, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) List_of_Preconditions, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function executes the 0203 Routine. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| routineInfo | Dcm_StartDataOut_Routine_Check_Reprogramming_Preconditions_Start_routineInfoType | Out |  |
| Number_of_Unfulfilled_Preconditions | Dcm_StartDataOut_Routine_Check_Reprogramming_Preconditions_Start_Number_of_Unfulfilled_PreconditionsType | Out |  |
| List_of_Preconditions | Uint8 | Out |  |
| DataLength | Uint16 | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0805; ARCH_SW_DIA_0806; ARCH_SW_DIA_0807; ARCH_SW_DIA_0808; ARCH_SW_DIA_0809; ARCH_SW_DIA_0810; ARCH_SW_DIA_0811; ARCH_SW_DIA_0812; ARCH_SW_DIA_0813; ARCH_SW_DIA_0814 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0806 | RoutineIdentifier shall be 0x0203. |  | DAI_EXT_TF_E_11085 |
| ARCH_SW_DIA_0807 | The folowing request shall be available for this RID: - Start Routine (01) |  | DAI_EXT_TF_E_11087 |
| ARCH_SW_DIA_0808 | If the ECU does not need any preconditions to be fulfilled for being reprogrammed or all preconditions are fulfilled, it shall return 0x00 as Number of Unfulfilled Preconditions. |  | DAI_EXT_TF_E_11088 |
| ARCH_SW_DIA_0809 | If there are unfulfilled preconditions, the ECU shall include each of them in the List of Unfulfilled Preconditions. |  | DAI_EXT_TF_E_11089 |
| ARCH_SW_DIA_0810 | If there are additional preconditions necessary which are not contained in the List of Unfulfilled Preconditions, these shall be added in consultation with the Diagnostic Development Team. |  | DAI_EXT_TF_E_11090 |
| ARCH_SW_DIA_0811 | Routine Start shall be available in the following diagnostic sessions: ⦁Programming Session ⦁Extended Session ⦁Development Session |  | DAI_EXT_TF_E_11094 |
| ARCH_SW_DIA_0812 | Routine Start shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED |  | DAI_EXT_TF_E_11364 |
| ARCH_SW_DIA_0813 | Routine Start shall be responsible to check if any system or ECU conditions to successfully reprogram the ECU are not fulfilled. |  | DAI_EXT_TF_E_11096 |
| ARCH_SW_DIA_0814 | RID specific Routine Start Result shall be represented by the following: ⦁Byte 0: Number of Unfulfilled Preconditions. ⦁Byte 1: List of Unfulfilled Preconditions |  | DAI_EXT_TF_E_11099 |
| ARCH_SW_DIA_1019 | The programming shall still be possible in the folowing cases ⦁vehicle_speed information or Impact_x information (CAN signals) not received since last power on/wake-up. ⦁vehicle_speed information or Impact_x information is SNA ⦁if vehicle_speed information or Impact_x information is lost during the active diagnostic session, the ECU shall use the last known valid preconditions to determine if a transition from Application Software to Reprogramming Software is allowed. |  | DAI_EXT_TF_E_11499 |

## DataServices_PreCheckProgrammingDependencies_Start

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_PreCheckProgrammingDependencies_Start(P2CONST(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_DATA) Pre_Check_Informations, Dcm_OpStatusType OpStatus, P2VAR(Dcm_StartDataOut_Routine_PreCheckProgrammingDependencies_Start_routineInfoType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) routineInfo, P2VAR(Dcm_StartDataOut_Routine_PreCheckProgrammingDependencies_Start_Dependencies_Verification_Check_ResultType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Dependencies_Verification_Check_Result, uint16 DataLength, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function executes the 0207 Routine. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Pre_Check_Informations | Uint8 | Out |  |
| OpStatus | Dcm_OpStatusType | Out |  |
| routineInfo | Dcm_StartDataOut_Routine_PreCheckProgrammingDependencies_Start_routineInfoType | Out |  |
| Dependencies_Verification_Check_Result | Dcm_StartDataOut_Routine_PreCheckProgrammingDependencies_Start_Dependencies_Verification_Check_ResultType | Out |  |
| DataLength | Uint16 | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0815; ARCH_SW_DIA_0816; ARCH_SW_DIA_0817; ARCH_SW_DIA_0818; ARCH_SW_DIA_0819; ARCH_SW_DIA_0820; ARCH_SW_DIA_0821; ARCH_SW_DIA_0822; ARCH_SW_DIA_0823 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0816 | RoutineIdentifier shall be 0x0207. |  | DAI_EXT_TF_E_11101 |
| ARCH_SW_DIA_0817 | The folowing request shall be available for this RID: - Start Routine (01) |  | DAI_EXT_TF_E_11102 |
| ARCH_SW_DIA_0818 | Regarding the SW/SW and HW/SW compatibility checks the same algorithms shall be used like for RoutineControl "checkProgrammingDependencies |  | DAI_EXT_TF_E_11103 |
| ARCH_SW_DIA_0819 | Contrary to RoutineControl "checkProgrammingDependencies", this service shall not change any ECU flags |  | DAI_EXT_TF_E_11104 |
| ARCH_SW_DIA_0820 | Routine Start shall be available in the following diagnostic sessions: ⦁Programming Session ⦁Extended Session ⦁Development Session |  | DAI_EXT_TF_E_11107 |
| ARCH_SW_DIA_0821 | Routine Start shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED |  | DAI_EXT_TF_E_11365 |
| ARCH_SW_DIA_0822 | Routine Start shall be responsible to check if any system or ECU conditions to successfully reprogram the ECU are not fulfilled. RID specific Control Options Record shall be represented by the following: Byte 0: Precheck Informations |  | DAI_EXT_TF_E_11109 |
| ARCH_SW_DIA_0823 | RID specific Routine Start Result shall be represented by the following: ⦁Byte 0: Dependencies Verification Check Result |  | DAI_EXT_TF_E_11111 |

## DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that Reads the Temperature Sensor Calibration Point. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0830; ARCH_SW_DIA_0831; ARCH_SW_DIA_0832; ARCH_SW_DIA_0833 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0831 | The Read Temperature Sensor Calibration Point shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11540 |
| ARCH_SW_DIA_0832 | The Read Temperature Sensor Calibration Point service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11541 |
| ARCH_SW_DIA_0833 | Data Identifier of the Read Temperature Sensor Calibration Point Service shall be: 0xFD30 |  | DAI_EXT_TF_E_11545 |

## DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_ConditionCheckRead.(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0830; ARCH_SW_DIA_0834; ARCH_SW_DIA_0835 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0834 | Response data length shall be 1 byte long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11548 |
| ARCH_SW_DIA_0835 | Byte #4: ⦁ Offset Point (signed value) |  | DAI_EXT_TF_E_11551 |

## DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFD30_Temperature_Sensor_Calibration_Point_Read_Offset_Point_WriteData(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function that writes the Temperature Sensor Calibration Point |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 | Out |  |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0840; ARCH_SW_DIA_0841; ARCH_SW_DIA_0842; ARCH_SW_DIA_0843; ARCH_SW_DIA_0844 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0841 | The Write Temperature Sensor Calibration Point Service shall be available in the following sessions: ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11555 |
| ARCH_SW_DIA_0842 | The Write Temperature Sensor Calibration Point service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD |  | DAI_EXT_TF_E_11556 |
| ARCH_SW_DIA_0843 | Request data shall be 1 byte long. Note: Full Request length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11559 |
| ARCH_SW_DIA_0844 | Byte #4: ⦁Offset Point (signed value) |  | DAI_EXT_TF_E_11562 |

## DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that Reads the Velocity PI Regulation Coefficient. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | Out | Data to send to DiagOnCAN tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0845; ARCH_SW_DIA_0846; ARCH_SW_DIA_0847; ARCH_SW_DIA_0848 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0846 | The Read Velocity PI Regulation Coefficients Service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E |  | DAI_EXT_TF_E_11379 |
| ARCH_SW_DIA_0847 | The Read Velocity PI Regulation Coefficients service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11380 |
| ARCH_SW_DIA_0848 | Data Identifier of the Read Velocity PI Regulation Coefficients Service shall be: 0xFDDF |  | DAI_EXT_TF_E_11384 |

## DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0845; ARCH_SW_DIA_0849; ARCH_SW_DIA_0850 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0849 | Response data length shall be 30 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11387 |
| ARCH_SW_DIA_0850 | Bytes #4-#33: ⦁Input Velocity Coefficient |  | DAI_EXT_TF_E_11390 |

## DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDidFDDF_Velocity_PI_Regulation_Coefficient_Read_Input_Velocity_Coefficient_WriteData(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function that writes the Temperature Sensor Calibration Point |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 | Out |  |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0855; ARCH_SW_DIA_0856; ARCH_SW_DIA_0857; ARCH_SW_DIA_0858; ARCH_SW_DIA_0859 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0856 | The Write Velocity PI Regulation Coefficients Service shall be available in the following sessions: ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11396 |
| ARCH_SW_DIA_0857 | The Write Velocity PI Regulation Coefficients service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD |  | DAI_EXT_TF_E_11397 |
| ARCH_SW_DIA_0858 | Request data shall be 30 bytes long. Note: Full Request length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11399 |
| ARCH_SW_DIA_0859 | Bytes #4-#33: ⦁Input Velocity Coefficient |  | DAI_EXT_TF_E_11402 |

## RoutineServices_Routine_Erase_All_Start

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) RoutineServices_Routine_Erase_All_Start(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StartDataOut_Routine_Erase_All_Start_routineInfoType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) routineInfo, P2VAR(Dcm_StartDataOut_Routine_Erase_All_Start_StatusType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Status, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function executes the F782 Routine. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| routineInfo | Dcm_StartDataOut_Routine_ DataServices_Erase_All_Start_Start_routineInfoType | Out |  |
| Status | Dcm_StartDataOut_Routine_Erase_All_Start_StatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0860; ARCH_SW_DIA_0861; ARCH_SW_DIA_0862; ARCH_SW_DIA_0863; ARCH_SW_DIA_0864; ARCH_SW_DIA_0865 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0861 | RoutineIdentifier shall be 0xF782. |  | DAI_EXT_TF_E_11439 |
| ARCH_SW_DIA_0862 | The folowing request shall be available for this RID: - Start Routine |  | DAI_EXT_TF_E_11440 |
| ARCH_SW_DIA_0863 | Routine Start shall be available in the following diagnostic sessions: ⦁Supplier Session/Development Session (0x7E) Routine Start requires the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD |  | DAI_EXT_TF_E_11443 |
| ARCH_SW_DIA_0864 | Routine Start shall be used to reset/clear the following NVM data: ⦁All cycle counters (EOL counters, Request Counters) ⦁System context ⦁System time ⦁Deficiency level ⦁Warm Reset data |  | DAI_EXT_TF_E_11446 |
| ARCH_SW_DIA_0865 | RID specific Routine Status Record Start shall be represented by the following: The following NvM Blocks shall be erased: ⦁EOL Counters NVP_BLOCK_ID_EXECUTION_COUNTERS ⦁Request Counters NVP_BLOCK_ID_MEASUREMENT_FRAME_12_CONFIG ⦁System Context NVP_BLOCK_ID_SYSTEM_CONTEXT_0 NVP_BLOCK_ID_SYSTEM_CONTEXT_1 NVP_BLOCK_ID_SYSTEM_CONTEXT_2 ⦁System Time NVP_BLOCK_ID_PRE_SAFE_RECORDER_1 NVP_BLOCK_ID_PRE_SAFE_RECORDER_2 ⦁Deficiency Level NVP_BLOCK_ID_TESTS_PARAM ⦁Warm Reset Data NVP_BLOCK_ID_WARM_RESET |  | DAI_EXT_TF_E_11448 |

## RoutineServices_Routine_Force_External_WDG_Start

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) RoutineServices_Routine_Force_External_WDG_Start(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StartDataOut_Routine_Force_External_WDG_Start_routineInfoType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) routineInfo, P2VAR(Dcm_StartDataOut_Routine_Force_External_WDG_Start_StatusType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Status, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function executes the F791 Routine. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| routineInfo | Dcm_StartDataOut_Routine_Force_External_WDG_Start_routineInfoType | Out |  |
| Status | Dcm_StartDataOut_Routine_Force_External_WDG_Start_StatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0870; ARCH_SW_DIA_0871; ARCH_SW_DIA_0872; ARCH_SW_DIA_0873; ARCH_SW_DIA_0874; ARCH_SW_DIA_0875 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0871 | RoutineIdentifier shall be 0xF791 |  | DAI_EXT_TF_E_11572 |
| ARCH_SW_DIA_0872 | The folowing request shall be available for this RID: - Start Routine |  | DAI_EXT_TF_E_11573 |
| ARCH_SW_DIA_0873 | Routine Start shall be available in the following diagnostic sessions: ⦁Supplier Session/Development Session (0x7E) Routine Start requires the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD |  | DAI_EXT_TF_E_11575 |
| ARCH_SW_DIA_0874 | Routine Start shall be used to test the external watchdog by stopping its refresh and causing a reset. |  | DAI_EXT_TF_E_11577 |
| ARCH_SW_DIA_0875 | RID specific Routine Status Record Start shall be represented by the following: ⦁Byte 6: Watchdog Status ⦁WDG_RESET_NOT_EXECUTED = 0x00 ⦁WDG_RESET_BY_DIAG = 0xAA ⦁WDG_NOT_RESET_BY_DIAG = 0x55 |  | DAI_EXT_TF_E_11579 |

## DataServices_Customer_Settings_Read_Customer_Settings_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Customer_Settings_Read_Customer_Settings_ConditionCheckRead (Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Customer Settings Read Service. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0880 |  |  |  |

## DataServices_Customer_Settings_Read_Customer_Settings_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Customer_Settings_Read_Customer_Settings_ReadData (Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that returns the Customer Settings stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0880; ARCH_SW_DIA_0881; ARCH_SW_DIA_0882; ARCH_SW_DIA_0883 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0881 | The Read Customer Settings service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11523 |
| ARCH_SW_DIA_0882 | The Read Customer Settings service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales |  | DAI_EXT_TF_E_11524 |
| ARCH_SW_DIA_0883 | DataIdentifier of the Read Customer Settings shall be: 0x0138 |  | DAI_EXT_TF_E_11528 |

## DataServices_Customer_Settings_Read_Customer_Settings_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Customer_Settings_Read_Customer_Settings_ReadDataLength (Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the Length of the Customer Settings DID. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint32 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0880; ARCH_SW_DIA_0884; ARCH_SW_DIA_0885; ARCH_SW_DIA_0886 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_0884 | Response data length shall be 2 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11531 |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0885 | Byte #4: BltSlckDec_Md_xy_Stat_ST3 obs: Service shall not report the current value of signal, but the value of parameter stored in NVM xy = FL or FR |  | DAI_EXT_TF_E_11534 |
| ARCH_SW_DIA_0886 | Byte #5: PS_Curve_xy_Stat_ST3 obs: Service shall not report the current value of signal, but the value of parameter stored in NVM xy = FL or FR |  | DAI_EXT_TF_E_11535 |

## DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Trust Model CSR Supplier Signature Read Service. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0890 |  |  |  |

## DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that returns the Trust Model CSR Supplier Signature stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0890; ARCH_SW_DIA_0891; ARCH_SW_DIA_0892; ARCH_SW_DIA_0893 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0891 | The Read Trust Model CSR Supplier Signature service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11410 |
| ARCH_SW_DIA_0892 | The Read Trust Model CSR Supplier Signature service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED |  | DAI_EXT_TF_E_11411 |
| ARCH_SW_DIA_0893 | DataIdentifier of the Read Trust Model CSR Supplier Signature shall be: 0x01A2 |  | DAI_EXT_TF_E_11415 |

## DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the Length of the Trust Model CSR Supplier Signature DID. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint258 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0890; ARCH_SW_DIA_0894; ARCH_SW_DIA_0895; ARCH_SW_DIA_0896 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_0894 | Response data length shall be 258 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11418 |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0895 | Bytes #4 - #131: ⦁Supplier Public Key Supplier Public Key Format[1 leading size byte + 128 bytes]: 1 leading size byte is automatically calculated by CANoe after the input of the Supplier Public Key. Receiving an example of 128 bytes value in CANoe will be sufficient to test: 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 |  | DAI_EXT_TF_E_11421 |
| ARCH_SW_DIA_0896 | Bytes #132 - #261: ⦁Supplier Signature Supplier Signature Format [1 leading size byte + 128 bytes]: 1 leading size byte is automatically calculated by CANoe after the input of the Supplier Signature Receiving an example of 128 bytes value in CANoe will be sufficient to test: 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x12 |  | DAI_EXT_TF_E_11422 |

## DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_WriteData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Trust_Model_CSR_Supplier_Signature_Read_Trust_Model_CSR_Supplier_Signature_WriteData(P2CONST(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_DATA) Data, uint16 DataLength, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function that writes the Trust Model CSR Supplier Signature |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Uint8 | Out |  |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint16 | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_0900; ARCH_SW_DIA_0901; ARCH_SW_DIA_0902; ARCH_SW_DIA_0903; ARCH_SW_DIA_0904; ARCH_SW_DIA_0905; ARCH_SW_DIA_0906 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_0901 | The Write Trust Model CSR Supplier Signature service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11471 |
| ARCH_SW_DIA_0902 | The Write Trust Model CSR Supplier Signature service shall be available under the following forms of Authentication: ⦁Supplier |  | DAI_EXT_TF_E_11472 |
| ARCH_SW_DIA_0903 | Bytes #2-#3: DataIdentifier of the Write Trust Model CSR Supplier Signature shall be: 0x01A2 |  | DAI_EXT_TF_E_11476 |
| ARCH_SW_DIA_0904 | Request data length shall be 258 bytes long Note: Full Request length = 3 bytes + Request data length |  | DAI_EXT_TF_E_11477 |
| ARCH_SW_DIA_0905 | Bytes #4 - #131: ⦁Supplier Public Key Supplier Public Key Format [1 leading size byte + 128 bytes]: 1 leading size byte is automatically calculated by CANoe after the input of the Supplier Public Key. Inserting an example of 128 bytes value in CANoe will be sufficient to test: 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 0x0B 0x0C 0x0D 0x0E 0x0F 0x10 0x11 0x12 |  | DAI_EXT_TF_E_11478 |
| ARCH_SW_DIA_0906 | Bytes #132 - #261: ⦁Supplier Signature Supplier Signature Format [1 leading size byte + 128 bytes]: 1 leading size byte is automatically calculated by CANoe after the input of the Supplier Signature Inserting an example of 128 bytes value in CANoe will be sufficient to test: 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x0B 0x12 0x11 0x10 0x0F 0x0E 0x0D 0x0C 0x12 |  | DAI_EXT_TF_E_11479 |

## DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_DID_01A2_Trust_model_CSR_Supplier_Signature_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Calibration Identifications Read Service. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_1000 |  |  |  |

## DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that returns the Calibration Identifications stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_1000; ARCH_SW_DIA_1001; ARCH_SW_DIA_1002; ARCH_SW_DIA_1003 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_1001 | The Read Calibration Identifications (CAL ID) service shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11426 |
| ARCH_SW_DIA_1002 | The Read Calibration Identifications (CAL ID) service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales STANDARD ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11427 |
| ARCH_SW_DIA_1003 | DataIdentifier of the Read Calibration Identifications (CAL ID) shall be: 0xE104 |  | DAI_EXT_TF_E_11431 |

## DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Calibration_Identifications_CAL_ID_Read_CombinedDataElement_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the Length of the Calibration Identifications DID. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint16 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_1000; ARCH_SW_DIA_1004; ARCH_SW_DIA_1005 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_1004 | Response data length shall be 16 bytes long Note: Full Response length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11434 |
| --- | --- | --- | --- |
| ARCH_SW_DIA_1005 | Bytes #4-#19: ⦁Calibration Identification |  | DAI_EXT_TF_E_11437 |

## DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ConditionCheckRead

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ConditionCheckRead(Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function checks the Dump of Vedoc Relevant Information Read Service. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_1010 |  |  |  |

## DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ReadData

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ReadData(Dcm_OpStatusType OpStatus, P2VAR(uint8, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) Data) |  |  |  |
| Object |  |  |  |
| Function that returns the Dump of Vedoc Relevant Information stored. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| Data | Uint8 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_1010; ARCH_SW_DIA_1011; ARCH_SW_DIA_1012; ARCH_SW_DIA_1013 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_1011 | The Read Vedoc Relevant Information shall be available in the following sessions: ⦁Default Session (0x01) ⦁Programming Session (0x02) ⦁Extended Session (0x03) ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11452 |
| ARCH_SW_DIA_1012 | The Read Vedoc Relevant Information service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production ⦁After-Sales ENHANCED ⦁After-Sales ⦁After-Sales BASIC ⦁Internal Diagnostic Test Tool ⦁ePTI Test Tool |  | DAI_EXT_TF_E_11453 |
| ARCH_SW_DIA_1013 | DataIdentifier of the Read Vedoc Relevant Information shall be: 0xF103 |  | DAI_EXT_TF_E_11457 |

## DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ReadDataLength

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DiagFunction_CODE) DataServices_Dump_of_Vedoc_Relevant_Information_Read_PatchedObject_ReadDataLength(Dcm_OpStatusType OpStatus, P2VAR(uint16, AUTOMATIC, RTE_DIAGFUNCTION_APPL_VAR) DataLength) |  |  |  |
| Object |  |  |  |
| Function calculates the Length of the Dump of Vedoc Relevant Information DID. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| DataLength | Uint16 | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_1010; ARCH_SW_DIA_1014; ARCH_SW_DIA_1015; ARCH_SW_DIA_1016; ARCH_SW_DIA_1017; ARCH_SW_DIA_1018 |  |  |  |

### Data flow / Parameters

| ARCH_SW_DIA_1014 | Response data length shall be 16 byte long Note: Full Response length = 3 bytes + Response data length |  |  |
| --- | --- | --- | --- |
| ARCH_SW_DIA_1015 | Bytes #4-#13: Hardware Part Number [ASCII] |  |  |
| ARCH_SW_DIA_1016 | Byte #14: Constant [ASCII] |  | DAI_EXT_TF_E_11464 |
| ARCH_SW_DIA_1017 | Bytes #15-#18: Hardware Supplier Identification [Hex to ASCII] |  | DAI_EXT_TF_E_11465 |
| ARCH_SW_DIA_1018 | Byte #19: ECU Serial Number [ASCII] |  | DAI_EXT_TF_E_11466 |

## DIA_runDID0340_UniqueSeatbeltSerialNumberWrite

### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, DIA_AC_DiagOnCAN_CODE) DIA_runDid0340_UniqueSeatbeltSerialNumberWrite(P2CONST(uint8, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_DATA) Data, Dcm_OpStatusType OpStatus, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_DIA_AC_DIAGONCAN_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| This function shall write the Seat Belt Serial Number. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| Data | Array of unsigned 8-bits | In | Data sent by the tester |
| OpStatus | Dcm_OpStatusType | Out | Negative response code (if applicable) |
| ErrorCode | Dcm_NegativeResponseCodeType | Out | Negative response code (if applicable) |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Dynamic aspect |  |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_1020; ARCH_SW_DIA_1021; ARCH_SW_DIA_1022; ARCH_SW_DIA_1023; ARCH_SW_DIA_1024; ARCH_SW_DIA_1025 |  |  |  |

### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_1021 | The Write Seat Belt Serial Number service shall be available in the following sessions: ⦁Supplier Session/Development Session (0x7E) |  | DAI_EXT_TF_E_11487 |
| ARCH_SW_DIA_1022 | The Write Seat Belt Serial Number service shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED ⦁Production |  | DAI_EXT_TF_E_11488 |
| ARCH_SW_DIA_1023 | Bytes #2-#3: DataIdentifier of the Write Seat Belt Serial Number shall be: 0x0340 |  | DAI_EXT_TF_E_11492 |
| ARCH_SW_DIA_1024 | Request data shall be 28 bytes long. Note: Full Request length = 3 bytes + Response data length |  | DAI_EXT_TF_E_11493 |
| ARCH_SW_DIA_1025 | Bytes #4 - #31: Unique Seatbelt Serial Number [ASCII] |  | DAI_EXT_TF_E_11494 |

## RoutineServices_Routine_Clear_Resource_Consumption_Data_Start_Start

#### Definition

| Prototype |  |  |  |
| --- | --- | --- | --- |
| FUNC(Std_ReturnType, Ct_ResourceMeasurement_CODE) RoutineServices_Routine_Clear_Resource_Consumption_Data_Start_Start(Dcm_OpStatusType OpStatus, P2VAR(Dcm_StartDataOut_Routine_Clear_Resource_Consumption_Data_Start_routineInfoType, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) routineInfo, P2VAR(Dcm_NegativeResponseCodeType, AUTOMATIC, RTE_CT_RESOURCEMEASUREMENT_APPL_VAR) ErrorCode) |  |  |  |
| Object |  |  |  |
| Function executes the 0211 Routine. |  |  |  |
| Parameters |  |  |  |
| Name | Type | Direction | Description |
| OpStatus | Dcm_OpStatusType | Out |  |
| routineInfo | Dcm_StartDataOut_Routine_Clear_Resource_Consumption_Data_Start_routineInfoType | Out |  |
| ErrorCode | Dcm_NegativeResponseCodeType | Out |  |
| Returned value |  |  |  |
| Name | Description |  |  |
| E_OK | DiagOnCAN request is processed, positive answer can be sent |  |  |
| E_NOT_OK | DiagOnCAN request cannot be processed, negative answer has to be sent |  |  |
| Synchronous server operation Non-Reentrant |  |  |  |
| Covered requirements |  |  |  |
| ARCH_SW_DIA_1030; ARCH_SW_DIA_1031; ARCH_SW_DIA_1032; ARCH_SW_DIA_1033; ARCH_SW_DIA_1034; ARCH_SW_DIA_1035; ARCH_SW_DIA_1036 |  |  |  |

#### Data flow / Parameters

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_1031 | RoutineIdentifier shall be 0x0211. |  | DAI_EXT_TF_E_11501 |
| ARCH_SW_DIA_1032 | The folowing request shall be available for this RID: - Start Routine (01) |  | DAI_EXT_TF_E_11502 |
| ARCH_SW_DIA_1033 | Routine Start shall be available in the following diagnostic sessions: ⦁Extended Session ⦁Development Session |  | DAI_EXT_TF_E_11504 |
| ARCH_SW_DIA_1034 | Routine Start shall be available under the following forms of Authentication: ⦁Supplier ⦁Development ENHANCED |  | DAI_EXT_TF_E_11505 |
| ARCH_SW_DIA_1035 | Routine Start shall be responsible to reset all resource data that is provided by the DIDs 0x01E0, 0x01E1, 0x01E2 and 0x01E3. |  | DAI_EXT_TF_E_11568 |
| ARCH_SW_DIA_1036 | RID specific Routine Start Result shall be represented by the following: ⦁Byte 0: Routine Info |  | DAI_EXT_TF_E_11570 |

# MCU resources

The following requirements on resource consumption objectives apply to the module/package:

| Requirements | Criteria | Levels/Tolerances | Source |
| --- | --- | --- | --- |
| ARCH_SW_DIA_9997 | The ROM size consumed by this component shall not exceed 18K bytes. |  |  |
| ARCH_SW_DIA_9998 | The RAM size consumed by this component shall be 700 bytes. |  |  |