| ERH Communication Interaction Layer |  |
| --- | --- |
| Summary | This is the Software Detailed Design Document for the DAIMLER MMA Project. |

| Author | Review | Approval |
| --- | --- | --- |
| Title: Obada Mirela Ioana | See Project Master Document for the roles and Project Members List for the name of people | See Project Master Document for the roles and Project Members List for the name of people |
| Distribution |  |  |
| See Project Master Document for the roles and Project Members List for the name of people | See Project Master Document for the roles and Project Members List for the name of people | See Project Master Document for the roles and Project Members List for the name of people |

Table of contents

General Information	5

1.1.	Revision history *	5

1.2.	Purpose and Scope	5

1.3.	Referenced documents	5

1.3.1.	External  documents	5

1.3.2.	Internal Documents	5

1.3.3.	Terminology and definitions	6

2.	SW Module Detailed Design	7

2.1.	Overview	7

2.2.	Traceability	7

3.	Features	21

3.1.	Services	21

3.1.1.	ERH_cyclic	21

3.1.2.	ERH_Init	22

3.1.3.	erh_CheckAecId	22

3.1.4.	erh_ManageFailedCounterEvent	24

3.1.5.	erh_ManagePassedCounterEvent	25

3.1.6.	erh_ManageAECCommand	25

3.1.7.	ERH_runGetAecStatus	26

3.1.8.	ERH_runGetAecGroupStatus	27

3.1.9.	ERH_ResetAllAECStatus	28

3.1.10.	erh_UpdateGroup	29

3.1.11.	ERH_runSetAecEvent	29

3.2.	Types	30

3.2.1.	erh_stAECConfigurationType	30

3.3.	Variabiles	31

3.3.1.	erh_au16AecCounter	31

3.3.2.	erh_u32GroupsStatus	31

3.3.3.	erh_au8AecStatus	31

3.3.4.	erh_u8PendingEventAddIndex	31

3.3.5.	erh_u8PendingEventRunIndex	31

3.3.6.	erh_au8PendingEventCommand	32

3.3.7.	erh_au8PendingEventAecIdentifier	32

3.3.8.	erh_u8StAECWriteStatusResult	32

3.3.9.	erh_bNvmRecordRequested	32

3.3.10.	erh_u8OtherGroupValue	32

3.3.11.	erh_cpstAecData	32

3.3.12.	ERH_astAECsDefinitions	33

3.3.13.	erh_au32GroupCounter	33

3.3.14.	erh_u16DebounceCounter	33

3.3.15.	ERH_u16AECsForControlDTC_StorageGroup[KU8_EIGHT]	33

3.3.16.	ERH_u16AECsForECULevel_StorageGroup[KU8_FIFTEEN]	33

3.3.17.	ERH_u16AECsForNetworkCom_StorageGroup[KU8_FIFTEEN]	34

3.3.18.	ERH_u16AECsForPowerDistribution_StorageGroup[KU8_SIX]	34

3.3.19.	ERH_u16DebounceCounter_NetworkComunication	35

3.3.20.	ERH_u16DebounceCounter_NetworkComunication15	35

3.3.21.	ERH_u16DebounceCounter_PowerDistribution	35

3.3.22.	ERH_u16StorageConditionStatus	35

3.3.23.	ERH_u8Index_ECULevel	35

3.3.24.	ERH_u8Index_NetworkCom	36

3.3.25.	ERH_u8Index_NoStorageCondition	36

3.3.26.	ERH_u8Index_PowerDistribution	36

3.4.	Macros	36

3.4.1.	KU8_MAX_PENDING_EVENT	36

3.4.2.	KU8_NUMBER_OF_GROUPS	36

3.4.3.	KU8_NVP_AEC_RECORDING_OK	37

3.4.4.	KU8_NVP_AEC_RECORDING_NOK	37

3.4.5.	KU8_NVP_AEC_RECORDING_IN_PROGRESS	37

3.4.6.	ERH_KU8_NUMBER_OF_AEC	37

3.4.7.	KU8_NVP_BLOCK_JOB_PENDING	37

3.4.8.	NVP_BLOCK_ID_AEC_STATUS	37

3.4.9.	KU16_COUNTER_TRESHOLD_IGN	38

3.4.10.	KU16_ERH_MASK_DTC_CONFIRMED	38

3.4.11.	BOOL_ERH_CLEAR_STORAGE_CONDITION	38

3.4.12.	BOOL_ERH_SET_STORAGE_CONDITION	38

3.4.13.	ERH_SET_STORAGE_CONDITION_MASK_OFF(mask)	38

3.4.14.	ERH_SET_STORAGE_CONDITION_MASK_ON(mask)	39

3.4.15.	KU16_ERH_MASK_CONTROL_DTC	39

3.4.16.	KU16_ERH_MASK_DEBOUNCE_TIMER_NC	39

3.4.17.	KU16_ERH_MASK_DEBOUNCE_TIMER_NC15	39

3.4.18.	KU16_ERH_MASK_DEBOUNCE_TIMER_PD	39

3.4.19.	KU16_ERH_MASK_DTC_CONFIRMED	39

3.4.20.	KU16_ERH_MASK_ECU_LEVEL	40

3.4.21.	KU16_ERH_MASK_NETWORK_COM	40

3.4.22.	KU16_ERH_MASK_NETWORK_COM15	40

3.4.23.	KU16_ERH_MASK_POWER_DISTRIBUTION	40

3.4.24.	KU16_ERH_SC10_0_debounceTimer	40

3.4.25.	KU16_ERH_SC10_1_debounceTimer	40

3.4.26.	KU16_ERH_SC10_2_debounceTimer	41

3.4.27.	KU16_ERH_SC11_VehicleStartup	41

3.4.28.	KU16_ERH_SC12_ProductionMode	41

3.4.29.	KU16_ERH_SC1_ControlDTC	41

3.4.30.	KU16_ERH_SC2_IgnitonStatus	41

3.4.31.	KU16_ERH_SC3_LocalVoltage	41

3.4.32.	KU16_ERH_SC4_SystemVoltage	42

3.4.33.	KU16_ERH_SC5_TransportationMode	42

3.4.34.	KU16_ERH_SC8_CommunicationStatus	42

3.4.35.	KU16_ERH_TIMER_TRESHOLD	42

4.	EEPROM	42

5.	Configuration	42

6.	Compilation Options	43

Table of figures

Figure 1 ERH_cyclic	22

Figure 2: ERH_Init	22

Figure 3: erh_CheckAecId	24

Figure 4: erh_ManageFailedCounterEvent	25

Figure 5: erh_ManagePassedCounterEvent	25

Figure 6: erh_ManageAECCommand	26

Figure 7: ERH_runGetAecStatus	27

Figure 8: ERH_runGetAecGroupsStatus	28

Figure 9: ERH_ResetAllAECStatus	29

Figure 10: erh_UpdateGroup	29

# General Information

## Revision history *

| Revision | Date | Author(s) | Description/comment |
| --- | --- | --- | --- |
| 1.1.5.1 | 9/9/2022 | Mirela Obada | First revision on Daimler MMA |
| 1.1.5.2 | 9/9/2022 | Mirela Obada | Update for 3.0 release |
| 1.1.5.3 | 12/9/2022 | Mirela Obada | Update traceability after SRM |
| 1.1.5.4 | 14/11/2022 | Mirela Obada | Update document for 4.0 |
| 1.1.5.5 | 15/11/2022 | Mirela Obada | Update with figures for all functions |
| 1.1.5.6 | 21/11/22 | Mirela Obada | Update traceability |
| 1.1.5.7 | 22/11/22 | Mirela Obada | Update after review |
| 1.1.5.8 | 20/12/2022 | Madalina Serban | Update after disable AECs implementation |
| 1.1.5.9 | 09/02/2023 | Madalina Serban | Update for R5.0 |
| 1.1.5.10 | 16/02/2023 | Madalina Serban | Fixes after review |
| 1.1.5.11 | 08/03/2023 | Mirela Obada | Update ERH_Init() diagaram |
| 1.1.5.12 | 08/03/2023 | Mirela Obada | Fix finding after review |
| 1.1.5.13 | 18/04/2023 | Mirela Obada | Update for R6.0 |
| 1.1.5.14 | 05/05/2023 | Mirela Obada | Update after SRM |
| 1.1.5.15 | 08/05/2023 | Mirela Obada | Update diagram for erh_UpdateGroup |
| 1.1.5.16 | 15/05/2023 | Mirela Obada | Update traceability |
| 1.1.5.17 | 21/06/2023 | Madalina Serban | Update for R6.1 |
| 1.1.5.18 | 22/06/2023 | Madalina Serban | Updated figure |
| 1.1.5.19 | 23/08/2023 | Mirela Obada | Update document for R07.0 |
| 1.1.5.20 | 25/08/2023 | Mirela Obada | Update traceability |
| 1.1.5.21 | 01/09/2023 | Mirela Obada | Update after review |
| 1.1.5.22 | 12/01/2024 | Madalina Serban | Updated document for R08.1 |
| 1.1.5.23 | 23.01.2024 | Mirela Obada | Update for 8.1 |
| 1.1.5.24 | 04/04/2024 | Mirela Obada | Update for 9.0 |
| 1.1.5.25 | 04/04/2024 | Mirela Obada | Update after reqtify |

\

* Template history is found in the CM tool used for templates

## Purpose and Scope

The purpose of this document is to provide an overview of the ERH operation principle, and to present the implementation choices in terms of module and function splitting.

## Referenced documents

### External  documents

| Id | Title | Reference |
| --- | --- | --- |

### Internal Documents

| Id | Title | Reference |
| --- | --- | --- |
|  | DAIMLER_MMA_SWarchitectureDesignInterfaceDescription.docx |  |
|  | ERH - Design Interface Description.docx |  |
|  | SBE_4G_NVP_layout.xls |  |

### Terminology and definitions

| Terminology | Meaning |
| --- | --- |
| AAU | Atomic architectural unit |
| SW | software |

# SW Module Detailed Design

## Overview

The aim of the “ERH” component is to receive error notifications and proper functioning notifications from other modules and filter them to keep an up to date a list of error status.

Error can be handled on two different ways:

- Timer based: An internal timer is managed by the ERH for each AEC. The timer is started when a failed event is received and stopped when the AEC qualification time reached, or a passed event is received.

- Counter base: An internal counter is managed by the ERH for each AEC. The counter is incremented/decremented depending on the received events (failed or passed).

Also the ERH module is responsible for manage  DTC qualification and disqualification depending on status of the AECs groups.

## Traceability

| Requirements | Criteria | Linked Runnable | Source |
| --- | --- | --- | --- |
| DSG_ERH_00001 | The ERH component shall check the validity of the current AEC identifier. The AEC identifier should be in the following range to be valid: 0x00 - 0x30. | Requirement covered by erh_CheckAecId function | ARCH_SW_ERH_0050; ARCH_SW_ERH_0074; ARCH_SW_ERH_0064; ARCH_SW_ERH_0063; ARCH_SW_ERH_0070; |
| DSG_ERH_00002 | The erh_CheckAecId function shall return B_TRUE (0xAA) value if the AEC identifier is valid, if not it shall return B_FALSE (0x55) value. | Requirement covered by erh_CheckAecId function | ARCH_SW_ERH_0050; ARCH_SW_ERH_0074; ARCH_SW_ERH_0064; ARCH_SW_ERH_0063; ARCH_SW_ERH_0070; |
| DSG_ERH_00003 | The ERH component shall convert the current AEC identifier to an index if it is valid | Requirement covered by erh_CheckAecId function | ARCH_SW_ERH_0050; ARCH_SW_ERH_0074; ARCH_SW_ERH_0064; ARCH_SW_ERH_0063; ARCH_SW_ERH_0070; |
| DSG_ERH_00004 | The ERH component shall update the AEC index and the corresponding debouncing/deskill thresholds if the AEC identifier is valid. | Requirement covered by erh_CheckAecId function | ARCH_SW_ERH_0050; ARCH_SW_ERH_0074; ARCH_SW_ERH_0064; ARCH_SW_ERH_0063; ARCH_SW_ERH_0070; |
| DSG_ERH_00005 | The ERH component shall manage the current AEC Status if a failed event occurred. | Requirement covered by erh_ManageFailedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048;ARCH_SW_ERH_0080; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00006 | The ERH component shall set the current AEC status to “qualified” if the qualification threshold has been reached. | Requirement covered by erh_ManageFailedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048; ARCH_SW_ERH_0080; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00007 | The ERH component shall increment with one step the current debounce counter if the AEC qualification threshold has not been reached. | Requirement covered by erh_ManageFailedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00008 | If the qualification threshold has been reached the ERH component shall perform the following actions to update the AEC status: -set ever tested status, -set ever qualified status, -set qualified status, -clear in progress status | Requirement covered by erh_ManageFailedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00009 | The ERH component shall update the group status if the qualification threshold has been reached. | Requirement covered by erh_ManageFailedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00010 | The ERH component shall store the current AEC status in NVP variable. | Requirement covered by erh_ManageFailedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00011 | The ERH component shall reset all AEC status and keep only “ever qualified” and “ever failed” status if the AEC deskill threshold has been reached. | Requirement covered by erh_ManageFailedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048; ARCH_SW_ERH_0080; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00012 | The ERH component shall manage the current AEC Status if a passed event occurred. | Requirement covered by erh_ManagePassedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048; ARCH_SW_ERH_0080; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00013 | The ERH component shall update the group status if the AEC deskill threshold has been reached and the qualified status has been set for current AEC. | Requirement covered by erh_ManagePassedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00014 | The ERH component shall store the current AEC status in NVP. | Requirement covered by erh_ManagePassedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048; ARCH_SW_ERH_0080; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00015 | The ERH component shall decrement with one step the current debounce counter if the AEC deskill threshold has not been reached. | Requirement covered by erh_ManagePassedCounterEvent function | ARCH_SW_ERH_0001; ARCH_SW_ERH_0004; ARCH_SW_ERH_0006; ARCH_SW_ERH_0008; ARCH_SW_ERH_0010; ARCH_SW_ERH_0012; ARCH_SW_ERH_0014; ARCH_SW_ERH_0016; ARCH_SW_ERH_0018; ARCH_SW_ERH_0019; ARCH_SW_ERH_0020; ARCH_SW_ERH_0021; ARCH_SW_ERH_0022; ARCH_SW_ERH_0023; ARCH_SW_ERH_0024; ARCH_SW_ERH_0025; ARCH_SW_ERH_0026; ARCH_SW_ERH_0027; ARCH_SW_ERH_0028; ARCH_SW_ERH_0029; ARCH_SW_ERH_0030; ARCH_SW_ERH_0031; ARCH_SW_ERH_0032; ARCH_SW_ERH_0033; ARCH_SW_ERH_0034; ARCH_SW_ERH_0035; ARCH_SW_ERH_0036; ARCH_SW_ERH_0037; ARCH_SW_ERH_0038;ARCH_SW_ERH_0039; ARCH_SW_ERH_0040; ARCH_SW_ERH_0041; ARCH_SW_ERH_0042; ARCH_SW_ERH_0043; ARCH_SW_ERH_0044; ARCH_SW_ERH_0045;ARCH_SW_ERH_0046; ARCH_SW_ERH_0047;ARCH_SW_ERH_0048; ARCH_SW_ERH_0086; ARCH_SW_ERH_0087; ARCH_SW_ERH_0088; ARCH_SW_ERH_0089; ARCH_SW_ERH_0090; ARCH_SW_ERH_0092; ARCH_SW_ERH_0157; |
| DSG_ERH_00016 | If at least one AEC in a group is currently qualified the qualification bit for this group shall be set true. | Requirement covered by erh_UpdateGroup function | ARCH_SW_ERH_0074; |
| DSG_ERH_00017 | The ERH component shall update the current group counter if at least one AEC in a group has been qualified or deskilled. | Requirement covered by erh_UpdateGroup function | ARCH_SW_ERH_0074; |
| DSG_ERH_00018 | If there are no more qualified AECs in a group, the qualification bit for this group shall be set to false. | Requirement covered by erh_UpdateGroup function | ARCH_SW_ERH_0074; |
| DSG_ERH_00019 | The ERH component shall initialize the AECs counters to 0x00 | Requirement covered by ERH_Init () function | ARCH_SW_ERH_0060; |
| DSG_ERH_00020 | The ERH component shall initialize the status of AECs to 0x00 | Requirement covered by ERH_Init () function | ARCH_SW_ERH_0060; |
| DSG_ERH_00021 | The ERH component shall manage the DTC Status if an AEC passed event occurred depending on the Group of the AEC. | Requirement covered by erh_UpdateGroup function | ARCH_SW_ERH_0083; ARCH_SW_ERH_0084; ARCH_SW_ERH_0085; ARCH_SW_ERH_0081; |
| DSG_ERH_00022 | The ERH component shall manage the DTC Status if an AEC failed event occurred depending on the Group of the AEC. | Requirement covered by erh_UpdateGroup function | ARCH_SW_ERH_0083; ARCH_SW_ERH_0084; ARCH_SW_ERH_0085; ARCH_SW_ERH_0081; |
| DSG_ERH_00025 | If there are no more qualified AECs in a group, the qualification bit for this group shall be set to false. | Requirement covered by erh_UpdateGroup function | ARCH_SW_ERH_0073; |
| DSG_ERH_00026 | The ERH component shall return the DCM_E_OK status if the Nvm writing process is successfully completed. | Requirement covered by ERH_ResetAllAECStatus service | ARCH_SW_ERH_0075,ARCH_SW_ERH_0076, |
| DSG_ERH_00027 | The ERH component shall return DCM_E_NOT_OK status if the Nvm writing process is unsuccessfully completed. | Requirement covered by ERH_ResetAllAECStatus service | ARCH_SW_ERH_0075,ARCH_SW_ERH_0076, |
| DSG_ERH_00028 | The ERH component shall enable conditions for error handler mechanism | Requirement covered by ERH_Init() | ARCH_SW_ERH_0100; |
| DSG_ERH_00029 | The ERH component shall provide the AEC status of a given AEC | ERH_runGetAecStatus() | ARCH_SW_ERH_0064; |
| DSG_ERH_00030 | The ERH component shall provide the AECs group status. | ERH_runGetAecGroupsStatus() | ARCH_SW_ERH_0090; |
| DSG_ERH_00031 | Each error code shall have the following properties: - a single Autoliv Error Code - an associated Status - an Aec Qualification Count - an chrono stack priority | Requirement covered by erh_ManageAECCommand function | ARCH_SW_ERH_0050; ARCH_SW_ERH_0091; ARCH_SW_BSW_0004; ARCH_SW_BSW_0005 |
| DSG_ERH_00032 | The ERH component shall be able to update a given AEC status applying the denouncing mechanism based on previous status of the AEC. | ERH_runSetAecEvent() | ARCH_SW_ERH_0080; ARCH_SW_ERH_0081; ARCH_SW_ERH_0083; ARCH_SW_ERH_0050; |
| DSG_ERH_00033 | The ERH component shall read QUALIFIED status in NVP and update status tables according its value. | Requirement covered by ERH_Init() | ARCH_SW_ERH_0084; ARCH_SW_ERH_0085; |
| DSG_ERH_00034 | The ERH component shall update the AEC status block in NVM | ERH_runSetAecEvent() | ARCH_SW_ERH_0077 |
| DSG_ERH_00035 | The ERH component shall be able to read through Rte_Read_prrIgnitionStatus_b8TriggerIgnitionStatus the status(TRUE/FALSE) coming from CIL. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0093; ARCH_SW_ERH_0094; ARCH_SW_ERH_0095; ARCH_SW_ERH_0110; ARCH_SW_ERH_0118; |
| DSG_ERH_00036 | The ERH component shall be able to start a debounce counter if IGN status is “TRUE” and shall reset the counter if status is set to “FALSE”. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0093; ARCH_SW_ERH_0094; ARCH_SW_ERH_0095; ARCH_SW_ERH_0110 |
| DSG_ERH_00037 | The debounce counter erh_u16DebounceCounter shall be used to set a 10s delay before enable Dem Ignition cycle. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0093; ARCH_SW_ERH_0094; ARCH_SW_ERH_0095; ARCH_SW_ERH_0110 |
| DSG_ERH_00038 | The ERH component shall be able to read through Rte_Read_prrIgnitionStatus_b8TriggerIgnitionStatusEnableStorageCondition the status(TRUE/FALSE) coming from CIL in order to set storage condition. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0117; |
| DSG_ERH_00039 | If ignition status is TRUE then SC2 shall be SET. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0117; |
| DSG_ERH_00040 | The ERH component shall get the status of “ControlDTCSetting” through Rte_Mode_prrControlAECSetting_DcmControlDtcSetting . | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0119; |
| DSG_ERH_00041 | If u8DtcState state from “ ControlDTCSetting” is enabled then status for SC1 shall be set. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0119; |
| DSG_ERH_00042 | The ERH component shall be able to read through Rte_Read_isrSupBatStatus_b8SupBatStatus the status(TRUE/FALSE) coming from CIL in order to set storage condition. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0111 |
| DSG_ERH_00043 | If u8SupBatStatus is TRUE then for SC4 shall be checked OV&UV autotest status else System OV&UV autotest shall be checked. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0111 |
| DSG_ERH_00044 | The ERH component shall be able to read through Rte_Call_pcsGetComMStatus_GetCurrentComMode the status(NO_COM/FULL_COM) in order to set storage condition. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0112 |
| DSG_ERH_00045 | If ComM_Status status is FULL_COM then SC8 shall be set | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0112 |
| DSG_ERH_00046 | The ERH component shall be able to read through Rte_Read_prrEngStartStatus_b8EngStartStatus the status(TRUE/FALSE) coming from CIL in order to set storage condition. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0113 |
| DSG_ERH_00047 | If u8EngStartStatus status is TRUE then SC11 shall be set | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0113 |
| DSG_ERH_00048 | The ERH component shall be able to read through Rte_Read_prrProdModeStatus_b8TriggerProdModeStatus the status(TRUE/FALSE) coming from CIL in order to set storage condition. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0114 |
| DSG_ERH_00049 | If u8ProductionMode status is TRUE then SC12 shall be set | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0114 |
| DSG_ERH_00050 | The ERH component shall be able to read through Rte_Read_prrTransportationModeStatus_b8TransportationModeStatus the status(TRUE/FALSE) coming from CIL in order to set storage condition. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0115 |
| DSG_ERH_00051 | If u8TransportationMode status is TRUE then SC5 shall be set | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0115 |
| DSG_ERH_00052 | The ERH component shall be able to read through Rte_Call_pcsAutotestServices_GetTestResult the status(TRUE/FALSE) coming from CIL in order to set storage condition. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0116 |
| DSG_ERH_00053 | If autotest status for UV or OV is passed then SC3 shall be set. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0116 |
| DSG_ERH_00054 | If ECU Level storage condition group is not set then the status for the following AECs shall be inhibited: - ERH_KU8_ENABLE_OUT_OF_ORDER_AEC_CFG, - ERH_KU8_MOSFET_HIGH_SC_AEC_CFG, - ERH_KU8_MOSFET_LOW_SC_AEC_CFG, - ERH_KU8_MOTOR_DISCONNECTED_AEC_CFG, - ERH_KU8_HALL_SENSOR_AEC_CFG, - ERH_KU8_TEMPERATURE_SENSOR_FAILURE_AEC_CFG, - ERH_KU8_EXT_WDG_OUT_OF_ORDER_AEC_CFG, - ERH_KU8_HW_SELF_PROTECTION_AEC_CFG, - ERH_KU8_SW_SELF_PROTECTION_AEC_CFG, - ERH_KU8_END_OF_LIFE_COMFORT_AEC_CFG, - ERH_KU8_END_OF_LIFE_LOW_AEC_CFG, - ERH_KU8_END_OF_LIFE_HIGH_AEC_CFG, - ERH_KU8_END_OF_LIFE_MAX_FORCE_AEC_CFG, - ERH_KU8_STEERING_CONFIG_AEC_CFG, - ERH_KU8_VARIANT_CODING_AEC_CFG | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0096 |
| DSG_ERH_00055 | If Network Communication storage condition group is not set then the status for the following AECs shall be inhibited: - ERH_KU8_TIMEOUT_PRESAFE_AEC_CFG, - ERH_KU8_TIMEOUT_BUCKLE_AEC_CFG, - ERH_KU8_IMPLAUSIBLE_DATA_PRESAFE_AEC_CFG, - ERH_KU8_IMPLAUSIBLE_DATA_IGNITION_AEC_CFG, - ERH_KU8_IMPLAUSIBLE_DATA_BUCKLE_AEC_CFG - ERH_KU8_IMPLAUSIBLE_DATA_ORC_AEC_CFG, - ERH_KU8_IMPLAUSIBLE_DATA_POWERTRAIN_RDY_AEC_CFG, - ERH_KU8_IMPLAUSIBLE_DATA_POWERTRAIN_DRV_AEC_CFG, - ERH_KU8_TIMEOUT_BLTHD_R_AEC_CFG, - ERH_KU8_TIMEOUT_BLTHD_L_AEC_CFG, - ERH_KU8_TIMEOUT_POWERTRAIN_AEC_CFG, - ERH_KU8_TIMEOUT_ODOSPEEDOMETER_AEC_CFG, - ERH_KU8_IMPLAUSIBLE_DATA_API_AEC_CFG, - ERH_KU8_TIMEOUT_BELT_ADJ_AEC_CFG, - ERH_KU8_TIMEOUT_API | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0097 |
| DSG_ERH_00056 | If Network Communication without Clam 15 storage condition group is not set then the status for the following AECs shall be inhibited: - ERH_KU8_TIMEOUT_IGNITION_AEC_CFG | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0098 |
| DSG_ERH_00057 | If Power Distribution storage condition group is not set then the status for the following AECs shall be inhibited: - ERH_KU8_OVO_POWER_AEC_CFG, - ERH_KU8_UVO_POWER_AEC_CFG, - ERH_KU8_UVO_TENSIONING_POWER_AEC_CFG, - ERH_KU8_OVO_TENSIONING_POWER_AEC_CFG, - ERH_KU8_SYSTEM_OV_AEC_CFG, - ERH_KU8_SYSTEM_UV_AEC_CFG | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0099 |
| DSG_ERH_00058 | If SC1 is not set then yhe following AECs shall be inhibited: - ERH_KU8_SPI_ERROR_AEC_CFG, - ERH_KU8_HIGH_SIDE_SWITCH_AEC_CFG, - ERH_KU8_MOSFET_OC_AEC_CFG, - ERH_KU8_WARM_RESET_AEC_CFG, - ERH_KU8_MOTOR_CURRENT_AEC_CFG, - ERH_KU8_MOTOR_BLOCKED_AEC_CFG, - ERH_KU8_MOTOR_ORDER_AEC_CFG, - ERH_KU8_MOTOR_IN_SC_AEC_CFG | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0150 |
| DSG_ERH_00059 | If - SC1: Control DTC settings - SC3: Local Voltage (KL30 input) - SC11: Vehicle Startup Are SET then "ECU level" group shall be SET | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0151 |
| DSG_ERH_00060 | If - SC1: Control DTC settings - SC2: Ignition Status - SC4: System Voltage - SC5: Transportation mode - SC8: Communication Status - SC10: De-bounce Timer network - SC11: Vehicle Startup - SC12: production mode Are SET then "Network Communication" group shall be SET | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0152 |
| DSG_ERH_00061 | If - SC1: Control DTC settings - SC4: System Voltage - SC5: Transportation mode - SC8: Communication Status - SC10: De-bounce Timer Network w/o KL15 - SC11: Vehicle Startup - SC12: production mode Are SET then "Network Communication - Timeout monitoring without Clamp 15" group shall be SET | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0153 |
| DSG_ERH_00062 | If - SC1: Control DTC settings - SC2: Ignition Status - SC5: Transportation mode - SC10: De-bounce Timer Power - SC11: Vehicle Startup - SC12: production mode Are SET then "Power Distribution" group shall be SET | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0154 |
| DSG_ERH_00063 | A debounce timer of 5s shall be applicable for each of the following storage condition groups Power Distribution, Network Communication without Clam 15 and Network Communication | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0154 |
| DSG_ERH_00064 | For Power Distribution SC group if all SCs (except SC10) are set a debounce timer of 5s shall start. Only if the 5s are elapsed the SC group shall be SET. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0156 |
| DSG_ERH_00065 | For Network Communication without Clam 15 SC group if all SCs (except SC10) are set a debounce timer of 5s shall start. Only if the 5s are elapsed the SC group shall be SET. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0156 |
| DSG_ERH_00066 | For Network Communication SC group if all SCs (except SC10) are set a debounce timer of 5s shall start. Only if the 5s are elapsed the SC group shall be SET. | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0156 |
| DSG_ERH_00067 | Dem_GetEventUdsStatus is called to read status for all DTCs | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0120 |
| DSG_ERH_00068 | If DTC status has bit 3 set then Rte_Write_ERH_AC_ErrorHandler_psrDTCConfirmed_b8DTCConfirmed shall write TRUE for DIA | Requirement covered by ERH_cyclic() | ARCH_SW_ERH_0121 |
| DSG_ERH_00069 | erh_UpdateGroup shall be called at init in order to update the status of the DTCs after a reset. (Avoinding unwanted 0x10 status after reset) | Requirement covered by ERH_Init() | ARCH_SW_ERH_0158 |
| DSG_ERH_00070 | When update group occurs erh module shall send snapshot data AEC to Dem module | Requirement covered by ERH_runSetAecEvent() | ARCH_SW_ERH_0159 |

# Features

## Services

### ERH_cyclic

| Object |  |  |
| --- | --- | --- |
| This service shall be used to set the debounce counter for Ignistion cycle start This service shall be called periodically at 10ms. |  |  |
| Prototype |  |  |
| void ERH_cycle(void) |  |  |
| Input parameters |  |  |
| Name | Type | Description |
| NA | NA | NA |
| Output parameters |  |  |
| Name | Type | Description |
| NA | NA | NA |
| Return value |  |  |
| Type | Description |  |
| NA | void |  |
| Dynamic aspect |  |  |
| Who(callers) | Description |  |
| * | * |  |
| Static aspect |  |  |
| NA |  |  |
| Constrains |  |  |
| NA |  |  |

Figure 1 ERH_cyclic

### ERH_Init

| Object |  |  |
| --- | --- | --- |
| This service shall initialize all AEC status and associated counters to default values. The “EVER QUALIFIED” status should be updated according to its value from NVM. This service shall be called only once during MCU initialization phase. |  |  |
| Prototype |  |  |
| void ERH_Init(void) |  |  |
| Input parameters |  |  |
| Name | Type | Description |
| NA | NA | NA |
| Output parameters |  |  |
| Name | Type | Description |
| NA | NA | NA |
| Return value |  |  |
| Type | Description |  |
| NA | void |  |
| Dynamic aspect |  |  |
| Who(callers) | Description |  |
| * | * |  |
| Static aspect |  |  |
| NA |  |  |
| Constrains |  |  |
| NA |  |  |

Figure 2: ERH_Init

### erh_CheckAecId

| Object |  |  |
| --- | --- | --- |
| This function shall check the validity of the current AEC identifier. |  |  |
| Prototype |  |  |
| LOCAL Boolean erh_CheckAecId (const uint8 cu8AecId, uint8 *const cpu8Index, erh_stAECConfigurationType* const cpstAecData) |  |  |
| Input Parameters |  |  |
| Name | Type | Description |
| Cu8AecId | uint8 | The AEC identifier which must be checked |
| cpu8Index | uint8* | The output array index |
| cpstAecData | stAECConfigurationType* | The address of the AEC data structure which has the qualification/deskill thresholds and the AEC group data |
| Output Parameters: |  |  |
| Name | Type | Description |
| cpu8Index | uint8* | The address where shall be stored the output index |
| cpstAecData | stAECConfigurationType* | The address of the AEC data structure where are stored the qualification/deskill thresholds and the group status |
| Return value |  |  |
| Type | Description |  |
| Boolean | - B_TRUE - if the given AEC identifier is in range: 0x0 - 0x37 - B_FALSE - if the given AEC identifier is out of range |  |
| Dynamic aspect |  |  |
| Who (callers) | Description |  |
| ERH component | ERH_runGetAecStatus – interface used to get the AEC status to DemIf module ERH_runSetAecEvent – interface used to set the AEC status ERH_ResetAllAECStatus – interface used to clear all AEC status |  |
| Static aspect |  |  |
| This function shall check the validity of the current AEC identifier and if it is valid, this function shall convert this identifier to an index used to access each AEC status. This function shall send by the output parameters the converted index and the corresponding debouncing threshold only if the AEC identifier is valid. If the AEC identifier is out of range, this function shall return B_FALSE (0x55) value. |  |  |
| Constraints |  |  |
| The AEC identifier should be in the following range to be valid: |  |  |

Figure 3: erh_CheckAecId

### erh_ManageFailedCounterEvent

| Object |  |  |
| --- | --- | --- |
| This function shall manage the current AEC counter on a failed event and update the AEC status |  |  |
| Prototype |  |  |
| LOCAL void erh_ManageFailedCounterEvent(const uint8 cu8Index, const erh_stAECConfigurationType* const cpstAecData) |  |  |
| Input Parameters |  |  |
| Name | Type | Description |
| cu8Index | Uint8 | AEC ID related to the passed event |
| CpstAecData | stAECConfigurationType* | Pointer to the corresponding AEC configuration |
| Output Parameters : |  |  |
| Name | Type | Description |
| NA | NA | NA |
| Return value |  |  |
| Type | Description |  |
| NA | NA |  |
| Dynamic aspect |  |  |
| Who (callers) | Description |  |
| ERH component | ERH_runSetAecEvent |  |
| Static aspect |  |  |
| This function firstly checks if the AEC qualification threshold is not inhibited for the current AEC. In the second step if the status of the current AEC is not qualified the debouncing counter shall be incremented and if the qualification limit has been reached, it shall be set to deskill value. The AEC status and group status shall be updated for the current AEC and stored in NVM. In the last step if the AEC qualification threshold is inhibited the current AEC Status shall be updated with “AEC ever tested” status. |  |  |
| Constraints |  |  |
| None |  |  |

Figure 4: erh_ManageFailedCounterEvent

### erh_ManagePassedCounterEvent

| Object |  |  |
| --- | --- | --- |
| This function shall manage the current AEC debounce counter and update the current AEC status on a Passed event |  |  |
| Prototype |  |  |
| LOCAL void erh_ManagePassedCounterEvent (const uint8 cu8Index, const erh_stAECConfigurationType * const cpstAecData) |  |  |
| Input Parameters |  |  |
| Name | Type | Description |
| cu8Index | Uint8 | AEC ID related to the passed event |
| CpstAecData | stAECConfigurationType* | Pointer to the corresponding AEC configuration |
| Output Parameters: |  |  |
| Name | Type | Description |
| NA | NA | NA |
| Return value |  |  |
| Type | Description |  |
| NA | NA |  |
| Dynamic aspect |  |  |
| Who (callers) | Description |  |
| ERH component | ERH_runSetAecEvent |  |
| Static aspect |  |  |
| This function firstly checks if deskill threshold is set for the current AEC and debouncing counter was incremented. In the second step the current debouncing counter shall be decremented with one step and the AEC status shall be updated. The AEC status shall be set to “in progress state” if the fault threshold was reached during the qualification phase otherwise “in progress status” shall be cleared. In the next step if the current debouncing counter reaches the zero value the AEC status for current AEC shall be updated. At each step, the AEC local status shall be stored in NVM. |  |  |
| Constraints |  |  |
| None |  |  |

Figure 5: erh_ManagePassedCounterEvent

### erh_ManageAECCommand

| Object |  |  |
| --- | --- | --- |
| This function shall execute the request received through the input parameter “cu8Command”. |  |  |
| Prototype |  |  |
| LOCAL void erh_ManageAECCommand(const u8AecCommandType cu8Command, const uint8 cu8Index, const erh_stAECConfigurationType* const cpstAecData) |  |  |
| Input Parameters |  |  |
| Name | Type | Description |
| cu8Command | u8AecCommandType | The received request |
| cu8Index | uint8 | AEC Index in the configuration table |
| cpstAecData | erh_stAECConfigurationType* | Pointer to the corresponding AEC data structure where are stored the qualification/deskill thresholds and the AEC group status |
| Output Parameters: |  |  |
| Name | Type | Description |
| cpstAecData | erh_stAECConfigurationType* | The address of the AEC data structure where are stored the qualification/deskill thresholds and the AEC group status |
| Return value |  |  |
| Type | Description |  |
| NA | NA |  |
| Dynamic aspect |  |  |
| Who (callers) | Description |  |
| ERH component | ERH_runSetAecEvent – called to update the AEC status |  |
| Static aspect |  |  |
| This function manages 4 cases. In the first two cases if the input parameter “cu8Command” is ‘FAILED’ (0x00) or ‘PASSED’ (0x01) the AEC qualification counter and the AEC status shall be updated. In another case if the input parameter “cu8Command” is ‘AEC_RST_COUNTER’ (0x02) the following actions shall be performed: - the qualification counter shall be reset, - only “ever tested” and “ever qualified” status shall be kept, the “qualified” and “in progress” AEC status shall be cleared, - all AEC parameters will be stored in NVP In the last case if the input parameter cu8Command will be ‘AEC_RST_ALL’ (0x03) the following actions will be performed: - the qualification counter shall be cleared - each AEC status shall be cleared except ‘ever tested bit’ - each AEC status shall be stored in NVP |  |  |
| Constraints |  |  |
| None |  |  |

Figure 6: erh_ManageAECCommand

### ERH_runGetAecStatus

| Object |  |  |
| --- | --- | --- |
| This service shall supply the AEC status to DemIf module if the AEC identifier is valid. f the AEC identifier transmitted by the input parameter “u8AecIdentifier” is between 0x00 and 0x33 this service shall send by the output parameter “pu8AecStatus” the current AEC status, and if the AEC identifier is out of range the transmitted AEC status shall be set to "UNPLAUSIBLE_STATUS". |  |  |
| Prototype |  |  |
| void ERH_runGetAecStatus(u8AecIdentifierType u8AecIdentifier, u8AecStatusType * pu8AecStatus) |  |  |
| Input parameters |  |  |
| Name | Type | Description |
| u8AecIdentifier | u8AecIdentifierType | The AEC Identifier |
| Output parameters |  |  |
| Name | Type | Description |
| pu8AecStatus | u8AecStatusType | The value for AEC status |
| Return value |  |  |
| Type | Description |  |
| NA | void |  |
| Dynamic aspect |  |  |
| Who(callers) | Description |  |
| * | * |  |
| Static aspect |  |  |
| NA |  |  |
| Constrains |  |  |
| NA |  |  |

Figure 7: ERH_runGetAecStatus

### ERH_runGetAecGroupStatus

| Object |  |  |
| --- | --- | --- |
| This service shall provide the the current AEC groups status bitfield. |  |  |
| Prototype |  |  |
| void ERH_runGetAecGroupStatus(u8AecIdentifierType u8AecIdentifier, u8AecStatusType * pu8AecStatus) |  |  |
| Input parameters |  |  |
| Name | Type | Description |
| NA |  |  |
| Output parameters |  |  |
| Name | Type | Description |
| pu32AecGroupsStatus | u32AecGroupsType | The value for AECs group status |
| Return value |  |  |
| Type | Description |  |
| NA | void |  |
| Dynamic aspect |  |  |
| Who(callers) | Description |  |
| * | * |  |
| Static aspect |  |  |
| NA |  |  |
| Constrains |  |  |
| NA |  |  |

Figure 8: ERH_runGetAecGroupsStatus

### ERH_ResetAllAECStatus

| Object |  |  |
| --- | --- | --- |
| This service shall reset all AECs status. |  |  |
| Prototype |  |  |
| Std_ReturnType ERH_ResetAllAECStatus(void) |  |  |
| Input parameters |  |  |
| Name | Type | Description |
| NA | NA | NA |
| Output parameters |  |  |
| Name | Type | Description |
| NA | NA | NA |
| Return value |  |  |
| Type | Description |  |
| Std_ReturnType Result | NA |  |
| Dynamic aspect |  |  |
| Who(callers) | Description |  |
| * | * |  |
| Static aspect |  |  |
| NA |  |  |
| Constrains |  |  |
| NA |  |  |

Figure 9: ERH_ResetAllAECStatus

### erh_UpdateGroup

| Object |  |  |
| --- | --- | --- |
| This function shall update the group status and the number of qualified AECs in the group. |  |  |
| Prototype |  |  |
| LOCAL void erh_UpdateGroup(const uint8 cu8AecId, const erh_stAECConfigurationType* const cpstAecData) |  |  |
| Input Parameters |  |  |
| Name | Type | Description |
| cu8AecId | uint8 | The AEC identifier to update the right group |
| cpstAecData | erh_stAECConfigurationType* | Pointer to the corresponding AEC configuration |
| Output Parameters: |  |  |
| Name | Type | Description |
| NA | NA | NA |
| Return value |  |  |
| Type | Description |  |
| NA | NA |  |
| Dynamic aspect |  |  |
| Who (callers) | Description |  |
| ERH component | ERH_Init - interface used to initialize the all internal data to default value erh_ManageAECCommand - used to execute the received request erh_ManageFailedCounterEvent - used to manage the AEC counter on a failed event erh_ManagePassedCounterEvent - used to manage the AEC counter on a passed event ERH_ResetAllAECStatus - interface called to clear all AEC status |  |
| Static aspect |  |  |
| The group status shall be set to “qualified” if at least one AEC in the current group is qualified, and if there is no more any qualified AEC in the group, the group status shall be set to disqualified. |  |  |
| Constraints |  |  |
| None |  |  |

Figure 10: erh_UpdateGroup

### ERH_runSetAecEvent

| Object |  |  |
| --- | --- | --- |
| o Basing of the previous AEC status. |  |  |
| Prototype |  |  |
| void ERH_runSetAecEvent (u8AecIdentifierType u8AecIdentifier, u8AecCommandType u8Command) |  |  |
| Input parameters |  |  |
| Name | Type | Description |
| u8AecIdentifier | u8AecIdentifierType | u8AecIdentifierType u8AecIdentifier: the AEC identifier u8AecCommandType u8Command: KU8_AEC_FAILED: Update the error status regarding a test failed result KU8_AEC_PASSED : Update the error status regarding a test passed result |
| u8Command | u8AecCommandType | u8AecCommandType cu8Command: uint8* cpu8Index: AEC Index in the configuration table stAECConfigurationType* cpstAecData: Pointer to the corresponding AEC data structure where are stored the qualification/deskill thresholds and the AEC group status |
| Output parameters |  |  |
| Name | Type | Description |
| NA | NA | NA |
| Return value |  |  |
| Type | Description |  |
| NA | None. |  |
| Dynamic aspect |  |  |
| Who(callers) | Description |  |
| * | * |  |
| Static aspect |  |  |
| * |  |  |
| Constrains |  |  |

Figure 10: ERH_runSetAecEvent

## Types

### erh_stAECConfigurationType

| Field Name | Field Type | Field description |
| --- | --- | --- |
| u16AECDeskillThrs | uint16 | To store the AEC deskill threshold |
| u16AECQualificationThrs | uint16 | To store the AEC qualification threshold |
| u8AECGroup | uint8 | To store the AEC group (HW, battery, motor etc.) |
| u8ReservedByte1 | uint8 | Reserved Byte for future update |
| u8ReservedByte2 | uint8 | Reserved Byte for future update |
| u8ReservedByte3 | uint8 | Reserved Byte for future update |

## Variabiles

### erh_au16AecCounter

| Type | Value |  |
| --- | --- | --- |
| uint16[] | NA |  |
| Description |  |  |
| Array of current debouncing counters for AEC’s |  |  |
| Definition |  |  |
| LOCAL uint16 erh_au16AecCounter[ERH_KU8_NUMBER_OF_AEC]; |  |  |

### erh_u32GroupsStatus

| Type | Value |  |
| --- | --- | --- |
| Uint32 | KU32_ZERO |  |
| Description |  |  |
| Status of AEC group |  |  |
| Definition |  |  |
| LOCAL uint16 erh_u16GroupsStatus; |  |  |

### erh_au8AecStatus

| Type | Value |  |
| --- | --- | --- |
| uint8 | NA |  |
| Description |  |  |
| Array of AEC status |  |  |
| Definition |  |  |
| LOCAL uint8 erh_au8AecStatus[ERH_KU8_NUMBER_OF_AEC]; |  |  |

### erh_u8PendingEventAddIndex

| Type | Value |  |
| --- | --- | --- |
| uint8 | KU8_ZERO |  |
| Description |  |  |
| Index pointing to the free spot of the pending event list |  |  |
| Definition |  |  |
| LOCAL uint8 erh_u8PendingEventAddIndex; |  |  |

### erh_u8PendingEventRunIndex

| Type | Value |  |
| --- | --- | --- |
| uint8 | KU8_ZERO |  |
| Description |  |  |
| Index pointing to the current item to process in the pending list |  |  |
| Definition |  |  |
| LOCAL uint8 erh_u8PendingEventRunIndex; |  |  |

### erh_au8PendingEventCommand

| Type | Value |  |
| --- | --- | --- |
| u8AecCommandType | NA |  |
| Description |  |  |
| Array of u8AecCommandType |  |  |
| Definition |  |  |
| LOCAL u8AecCommandType erh_au8PendingEventCommand[KU8_MAX_PENDING_EVENT]; |  |  |

### erh_au8PendingEventAecIdentifier

| Type | Value |  |
| --- | --- | --- |
| u8AecIdentifierType | NA |  |
| Description |  |  |
| Array of u8AecIdentifierType |  |  |
| Definition |  |  |
| LOCAL u8AecIdentifierType erh_au8PendingEventAecIdentifier[KU8_MAX_PENDING_EVENT]; |  |  |

### erh_u8StAECWriteStatusResult

| Type | Value |  |
| --- | --- | --- |
| uint8 | KU8_ZERO |  |
| Description |  |  |
| Result of the AEC status write process |  |  |
| Definition |  |  |
| LOCAL uint8 erh_u8StAECWriteStatusResult; |  |  |

### erh_bNvmRecordRequested

| Type | Value |  |
| --- | --- | --- |
| boolean | B_FALSE |  |
| Description |  |  |
| Switch used to indicate when the condition is met to store data in NVP |  |  |
| Definition |  |  |
| LOCAL boolean erh_bNvmRecordRequested; |  |  |

### erh_u8OtherGroupValue

| Type | Value |  |
| --- | --- | --- |
| uint8 | NA |  |
| Description |  |  |
| The number of qualified AEC in the group |  |  |
| Definition |  |  |
| LOCAL uint8 erh_u8OtherGroupValue; |  |  |

### erh_cpstAecData

| Type | Value |  |
| --- | --- | --- |
| erh_stAECConfigurationType | NA |  |
| Description |  |  |
| Indirection pointer to access AEC configuration data |  |  |
| Definition |  |  |
| LOCAL const erh_stAECConfigurationType * const erh_cpstAecData |  |  |

### ERH_astAECsDefinitions

| Type | Value |  |
| --- | --- | --- |
| uint8 | NA |  |
| Description |  |  |
| Array of AECs configuration |  |  |
| Definition |  |  |
| const extern uint8 ERH_astAECsDefinitions[]; |  |  |

### erh_au32GroupCounter

| Type | Value |  |
| --- | --- | --- |
| uint32 | NA |  |
| Description |  |  |
| Array of counters for AEC group. |  |  |
| Definition |  |  |
| LOCAL uint32 erh_au32GroupCounter[((uint8) 30)] |  |  |
| Remarks |  |  |
| None. |  |  |

### erh_u16DebounceCounter

| Type | Value |  |
| --- | --- | --- |
| Uint16 | NA |  |
| Description |  |  |
| Counter for debounce before start of Ign cycle |  |  |
| Definition |  |  |
| LOCAL uint16 erh_au16DebounceCounter [((uint16) 0)] |  |  |
| Remarks |  |  |
| None. |  |  |

### ERH_u16AECsForControlDTC_StorageGroup[KU8_EIGHT]

| Type | Value |  |
| --- | --- | --- |
| uint16 | NA |  |
| Description |  |  |
| Initial value: |  |  |
| Definition |  |  |
| LOCAL uint16 ERH_u16AECsForControlDTC_StorageGroup[KU8_EIGHT] |  |  |
| = { ERH_KU8_SPI_ERROR_AEC_CFG, |  |  |
| ERH_KU8_HIGH_SIDE_SWITCH_AEC_CFG, ERH_KU8_MOSFET_OC_AEC_CFG, ERH_KU8_WARM_RESET_AEC_CFG, ERH_KU8_MOTOR_CURRENT_AEC_CFG, ERH_KU8_MOTOR_BLOCKED_AEC_CFG, ERH_KU8_MOTOR_ORDER_AEC_CFG, ERH_KU8_MOTOR_IN_SC_AEC_CFG} |  |  |

### ERH_u16AECsForECULevel_StorageGroup[KU8_FIFTEEN]

| Type | Value |  |
| --- | --- | --- |
| uint16 | NA |  |
| Description |  |  |
| Initial value: |  |  |
| Definition |  |  |
| LOCAL uint16 ERH_u16AECsForECULevel_StorageGroup[KU8_FIFTEEN] |  |  |
| = {ERH_KU8_ENABLE_OUT_OF_ORDER_AEC_CFG, |  |  |
| ERH_KU8_MOSFET_HIGH_SC_AEC_CFG, ERH_KU8_MOSFET_LOW_SC_AEC_CFG, ERH_KU8_MOTOR_DISCONNECTED_AEC_CFG, ERH_KU8_HALL_SENSOR_AEC_CFG, ERH_KU8_TEMPERATURE_SENSOR_FAILURE_AEC_CFG, ERH_KU8_EXT_WDG_OUT_OF_ORDER_AEC_CFG, ERH_KU8_HW_SELF_PROTECTION_AEC_CFG, ERH_KU8_SW_SELF_PROTECTION_AEC_CFG, ERH_KU8_END_OF_LIFE_COMFORT_AEC_CFG, ERH_KU8_END_OF_LIFE_LOW_AEC_CFG, ERH_KU8_END_OF_LIFE_HIGH_AEC_CFG, ERH_KU8_END_OF_LIFE_MAX_FORCE_AEC_CFG, ERH_KU8_STEERING_CONFIG_AEC_CFG, ERH_KU8_VARIANT_CODING_AEC_CFG} |  |  |

### ERH_u16AECsForNetworkCom_StorageGroup[KU8_FIFTEEN]

| Type | Value |  |
| --- | --- | --- |
| uint16 | NA |  |
| Description |  |  |
| Initial value: |  |  |
| Definition |  |  |
| LOCAL uint16 ERH_u16AECsForNetworkCom_StorageGroup[KU8_FIFTEEN] |  |  |
| = {ERH_KU8_TIMEOUT_PRESAFE_AEC_CFG, |  |  |
| ERH_KU8_TIMEOUT_BUCKLE_AEC_CFG, ERH_KU8_IMPLAUSIBLE_DATA_PRESAFE_AEC_CFG, ERH_KU8_IMPLAUSIBLE_DATA_IGNITION_AEC_CFG, ERH_KU8_IMPLAUSIBLE_DATA_BUCKLE_AEC_CFG, ERH_KU8_IMPLAUSIBLE_DATA_ORC_AEC_CFG, ERH_KU8_IMPLAUSIBLE_DATA_POWERTRAIN_RDY_AEC_CFG, ERH_KU8_IMPLAUSIBLE_DATA_POWERTRAIN_DRV_AEC_CFG, ERH_KU8_TIMEOUT_BLTHD_R_AEC_CFG, ERH_KU8_TIMEOUT_BLTHD_L_AEC_CFG, ERH_KU8_TIMEOUT_POWERTRAIN_AEC_CFG, ERH_KU8_TIMEOUT_ODOSPEEDOMETER_AEC_CFG, ERH_KU8_IMPLAUSIBLE_DATA_API_AEC_CFG, ERH_KU8_TIMEOUT_BELT_ADJ_AEC_CFG, ERH_KU8_TIMEOUT_API} |  |  |

### ERH_u16AECsForPowerDistribution_StorageGroup[KU8_SIX]

| Type | Value |  |
| --- | --- | --- |
| uint16 | NA |  |
| Description |  |  |
| Initial value: |  |  |
| Definition |  |  |
| LOCAL uint16 ERH_u16AECsForPowerDistribution_StorageGroup[KU8_SIX] |  |  |
| = {ERH_KU8_OVO_POWER_AEC_CFG, |  |  |
| ERH_KU8_UVO_POWER_AEC_CFG, ERH_KU8_UVO_TENSIONING_POWER_AEC_CFG, ERH_KU8_OVO_TENSIONING_POWER_AEC_CFG, ERH_KU8_SYSTEM_OV_AEC_CFG, ERH_KU8_SYSTEM_UV_AEC_CFG} |  |  |

### ERH_u16DebounceCounter_NetworkComunication

| Type | Value |  |
| --- | --- | --- |
| uint16 | KU16_ZERO |  |
| Description |  |  |
| The Counter of Network Comunication storage Group. |  |  |
| Definition |  |  |
| LOCAL uint16 ERH_u16DebounceCounter_NetworkComunication = KU16_ZERO |  |  |
| Remarks |  |  |
| None. |  |  |

### ERH_u16DebounceCounter_NetworkComunication15

| Type | Value |  |
| --- | --- | --- |
| uint16 | KU16_ZERO |  |
| Description |  |  |
| The Counter of Network Comunication 15 storage Group. |  |  |
| Definition |  |  |
| LOCAL uint16 ERH_u16DebounceCounter_NetworkComunication15 = KU16_ZERO |  |  |
| Remarks |  |  |
| None. |  |  |

### ERH_u16DebounceCounter_PowerDistribution

| Type | Value |  |
| --- | --- | --- |
| uint16 | KU16_ZERO |  |
| Description |  |  |
| The Counter of Power Distribution storage Group. |  |  |
| Definition |  |  |
| LOCAL uint16 ERH_u16DebounceCounter_PowerDistribution = KU16_ZERO |  |  |
| Remarks |  |  |
| None. |  |  |

### ERH_u16StorageConditionStatus

| Type | Value |  |
| --- | --- | --- |
| uint16 | NA |  |
| Description |  |  |
| The Status mask used for Storage Conditions. |  |  |
| Definition |  |  |
| LOCAL uint16 ERH_u16StorageConditionStatus |  |  |
| Remarks |  |  |
| None. |  |  |

### ERH_u8Index_ECULevel

| Type | Value |  |
| --- | --- | --- |
| uint8 | KU8_ZERO |  |
| Description |  |  |
| This variable is used to ... |  |  |
| Definition |  |  |
| LOCAL uint8 ERH_u8Index_ECULevel = KU8_ZERO |  |  |

### ERH_u8Index_NetworkCom

| Type | Value |  |
| --- | --- | --- |
| uint8 | KU8_ZERO |  |
| Description |  |  |
| This variable is used to ... |  |  |
| Definition |  |  |
| LOCAL uint8 ERH_u8Index_NetworkCom = KU8_ZERO |  |  |

### ERH_u8Index_NoStorageCondition

| Type | Value |  |
| --- | --- | --- |
| uint8 | KU8_ZERO |  |
| Description |  |  |
| This variable is used to ... |  |  |
| Definition |  |  |
| LOCAL uint8 ERH_u8Index_NoStorageCondition = KU8_ZERO |  |  |

### ERH_u8Index_PowerDistribution

| Type | Value |  |
| --- | --- | --- |
| uint8 | KU8_ZERO |  |
| Description |  |  |
| This variable is used to ... |  |  |
| Definition |  |  |
| LOCAL uint8 ERH_u8Index_PowerDistribution = KU8_ZERO |  |  |

## Macros

### KU8_MAX_PENDING_EVENT

| Name | Value |
| --- | --- |
| KU8_MAX_PENDING_EVENT | 8 |
| Definition |  |
| #define KU8_MAX_PENDING_EVENT ((uint8)8) |  |
| Description |  |
| Maximum number of pending events allowed |  |

### KU8_NUMBER_OF_GROUPS

| Name | Value |
| --- | --- |
| KU8_NUMBER_OF_GROUPS | 30 |
| Definition |  |
| #define KU8_NUMBER_OF_GROUPS ((uint8)30) |  |
| Description |  |
| Size array with the number of AEC groups |  |

### KU8_NVP_AEC_RECORDING_OK

| Name | Value |
| --- | --- |
| KU8_NVP_AEC_RECORDING_OK | 0 |
| Definition |  |
| #define KU8_NVP_AEC_RECORDING_OK ((uint8)0) |  |
| Description |  |
| The NVP Recording OK value of "Ever qualified" AEC informations |  |

### KU8_NVP_AEC_RECORDING_NOK

| Name | Value |
| --- | --- |
| KU8_NVP_AEC_RECORDING_NOK | 0xFF |
| Definition |  |
| #define KU8_NVP_AEC_RECORDING_NOK ((uint8)0xFF) |  |
| Description |  |
| The NVP Recording “Not OK” value of "Ever qualified" AEC informations |  |

### KU8_NVP_AEC_RECORDING_IN_PROGRESS

| Name | Value |
| --- | --- |
| KU8_NVP_AEC_RECORDING_IN_PROGRESS | 0x55 |
| Definition |  |
| #define KU8_NVP_AEC_RECORDING_IN_PROGRESS ((uint8)0x55) |  |
| Description |  |
| The NVP Recording “In Progress” value of "Ever qualified" AEC informations |  |

### ERH_KU8_NUMBER_OF_AEC

| Name | Value |
| --- | --- |
| ERH_KU8_NUMBER_OF_AEC | ((uint8)0x36) |
| Definition |  |
| #define ERH_KU8_NUMBER_OF_AEC ((uint8)0x36) |  |
| Description |  |
| Number of all AECs |  |

### KU8_NVP_BLOCK_JOB_PENDING

| Name | Value |
| --- | --- |
| KU8_NVP_BLOCK_JOB_PENDING | (4U) |
| Definition |  |
| #define KU8_NVP_BLOCK_JOB_PENDING (4U) |  |
| Description |  |
| Define for block job pending |  |

### NVP_BLOCK_ID_AEC_STATUS

| Name | Value |
| --- | --- |
| NVP_BLOCK_ID_AEC_STATUS | ( 25u ) |
| Definition |  |
| #define NVP_BLOCK_ID_AEC_STATUS ( 25u ) |  |
| Description |  |
| Define for NVP block where are IDs of AEC status |  |

### KU16_COUNTER_TRESHOLD_IGN

| Name | Value |
| --- | --- |
| KU16_COUNTER_TRESHOLD_IGN | ( 1000u ) |
| Definition |  |
| #define KU16_COUNTER_TRESHOLD_IGN ( 1000u ) |  |
| Description |  |
| Define for debounce counter. |  |

### KU16_ERH_MASK_DTC_CONFIRMED

| Name | Value |
| --- | --- |
| KU16_ERH_MASK_DTC_CONFIRMED | ( 0x008 ) |
| Definition |  |
| #define KU16_ERH_MASK_DTC_CONFIRMED ((uint16)0x008) |  |
| Description |  |
| Define for DTC Confirmed bit status |  |

### BOOL_ERH_CLEAR_STORAGE_CONDITION

| Name | Value |
| --- | --- |
| BOOL_ERH_CLEAR_STORAGE_CONDITION | (0u) |
| Definition |  |
| #define BOOL_ERH_CLEAR_STORAGE_CONDITION (0u) |  |
| Description |  |
| Storage Condition off |  |

### BOOL_ERH_SET_STORAGE_CONDITION

| Name | Value |
| --- | --- |
| BOOL_ERH_SET_STORAGE_CONDITION | (1u) |
| Definition |  |
| #define BOOL_ERH_SET_STORAGE_CONDITION (1u) |  |
| Description |  |
| Storage Condition on |  |

### ERH_SET_STORAGE_CONDITION_MASK_OFF(mask)

| Name | Value |
| --- | --- |
| ERH_SET_STORAGE_CONDITION_MASK_OFF(mask) | (mask) U16_BIT_CLEAR(ERH_u16StorageConditionStatus, (mask)) |
| Definition |  |
| #define ERH_SET_STORAGE_CONDITION_MASK_OFF(mask) U16_BIT_CLEAR(ERH_u16StorageConditionStatus, (mask)) |  |
| Description |  |
| Macro used for bit clearing inhibition status. |  |

### ERH_SET_STORAGE_CONDITION_MASK_ON(mask)

| Name | Value |
| --- | --- |
| ERH_SET_STORAGE_CONDITION_MASK_ON(mask) | (mask) U16_BIT_SET(ERH_u16StorageConditionStatus, (mask)) |
| Definition |  |
| #define ERH_SET_STORAGE_CONDITION_MASK_ON(mask) U16_BIT_SET(ERH_u16StorageConditionStatus, (mask)) |  |
| Description |  |
| Macro used for bit setting inhibition status. |  |

### KU16_ERH_MASK_CONTROL_DTC

| Name | Value |
| --- | --- |
| KU16_ERH_MASK_CONTROL_DTC | (1u) |
| Definition |  |
| #define KU16_ERH_MASK_CONTROL_DTC (1u) |  |
| Description |  |
| CONTROL_DTC mask |  |

### KU16_ERH_MASK_DEBOUNCE_TIMER_NC

| Name | Value |
| --- | --- |
| KU16_ERH_MASK_DEBOUNCE_TIMER_NC | ((uint16) 0x63B) |
| Definition |  |
| #define KU16_ERH_MASK_DEBOUNCE_TIMER_NC ((uint16) 0x63B) |  |
| Description |  |
| DEBOUNCE_TIMER_NC mask |  |

### KU16_ERH_MASK_DEBOUNCE_TIMER_NC15

| Name | Value |
| --- | --- |
| KU16_ERH_MASK_DEBOUNCE_TIMER_NC15 | ((uint16) 0x639) |
| Definition |  |
| #define KU16_ERH_MASK_DEBOUNCE_TIMER_NC15 ((uint16) 0x639) |  |
| Description |  |
| DEBOUNCE_TIMER_NC15 mask |  |

### KU16_ERH_MASK_DEBOUNCE_TIMER_PD

| Name | Value |
| --- | --- |
| KU16_ERH_MASK_DEBOUNCE_TIMER_PD | ((uint16) 0x633) |
| Definition |  |
| #define KU16_ERH_MASK_DEBOUNCE_TIMER_PD ((uint16) 0x633) |  |
| Description |  |
| NA |  |

### KU16_ERH_MASK_DTC_CONFIRMED

| Name | Value |
| --- | --- |
| KU16_ERH_MASK_DTC_CONFIRMED | ((uint16)0x008) |
| Definition |  |
| #define KU16_ERH_MASK_DTC_CONFIRMED ((uint16)0x008) |  |
| Description |  |
| DTC_CONFIRMED mask |  |

### KU16_ERH_MASK_ECU_LEVEL

| Name | Value |
| --- | --- |
| KU16_ERH_MASK_ECU_LEVEL | (517u) |
| Definition |  |
| #define KU16_ERH_MASK_ECU_LEVEL (517u) |  |
| Description |  |
| ECU_LEVEL storage condition group mask |  |

### KU16_ERH_MASK_NETWORK_COM

| Name | Value |
| --- | --- |
| KU16_ERH_MASK_NETWORK_COM | ((uint16)0x6BB) |
| Definition |  |
| #define KU16_ERH_MASK_NETWORK_COM ((uint16)0x6BB) |  |
| Description |  |
| NETWORK_COM storage condition group mask |  |

### KU16_ERH_MASK_NETWORK_COM15

| Name | Value |
| --- | --- |
| KU16_ERH_MASK_NETWORK_COM15 | ((uint16)0x739) |
| Definition |  |
| #define KU16_ERH_MASK_NETWORK_COM15 ((uint16)0x739) |  |
| Description |  |
| NETWORK_COM15 storage condition group mask |  |

### KU16_ERH_MASK_POWER_DISTRIBUTION

| Name | Value |
| --- | --- |
| KU16_ERH_MASK_POWER_DISTRIBUTION | ((uint16)0x673) |
| Definition |  |
| #define KU16_ERH_MASK_POWER_DISTRIBUTION ((uint16)0x673) |  |
| Description |  |
| POWER_DISTRIBUTION mask |  |

### KU16_ERH_SC10_0_debounceTimer

| Name | Value |
| --- | --- |
| KU16_ERH_SC10_0_debounceTimer | (64u) |
| Definition |  |
| #define KU16_ERH_SC10_0_debounceTimer (64u) |  |
| Description |  |
| SC10_0_debounceTimer mask |  |

### KU16_ERH_SC10_1_debounceTimer

| Name | Value |
| --- | --- |
| KU16_ERH_SC10_1_debounceTimer | (128u) |
| Definition |  |
| #define KU16_ERH_SC10_1_debounceTimer (128u) |  |
| Description |  |
| SC10_1_debounceTimer mask |  |

### KU16_ERH_SC10_2_debounceTimer

| Name | Value |
| --- | --- |
| KU16_ERH_SC10_2_debounceTimer | (256u) |
| Definition |  |
| #define KU16_ERH_SC10_2_debounceTimer (256u) |  |
| Description |  |
| SC10_2_debounceTimer mask |  |

### KU16_ERH_SC11_VehicleStartup

| Name | Value |
| --- | --- |
| KU16_ERH_SC11_VehicleStartup | (512u) |
| Definition |  |
| #define KU16_ERH_SC11_VehicleStartup (512u) |  |
| Description |  |
| VehicleStartup mask |  |

### KU16_ERH_SC12_ProductionMode

| Name | Value |
| --- | --- |
| KU16_ERH_SC12_ProductionMode | (1024u) |
| Definition |  |
| #define KU16_ERH_SC12_ProductionMode (1024u) |  |
| Description |  |
| ProductionMode mask |  |

### KU16_ERH_SC1_ControlDTC

| Name | Value |
| --- | --- |
| KU16_ERH_SC1_ControlDTC | (1u) |
| Definition |  |
| #define KU16_ERH_SC1_ControlDTC (1u) |  |
| Description |  |
| ControlDTC mask |  |

### KU16_ERH_SC2_IgnitonStatus

| Name | Value |
| --- | --- |
| KU16_ERH_SC2_IgnitonStatus | (2u) |
| Definition |  |
| #define KU16_ERH_SC2_IgnitonStatus (2u) |  |
| Description |  |
| IgnitonStatus mask |  |

### KU16_ERH_SC3_LocalVoltage

| Name | Value |
| --- | --- |
| KU16_ERH_SC3_LocalVoltage | (4u) |
| Definition |  |
| #define KU16_ERH_SC3_LocalVoltage (4u) |  |
| Description |  |
| LocalVoltage mask |  |

### KU16_ERH_SC4_SystemVoltage

| Name | Value |
| --- | --- |
| KU16_ERH_SC4_SystemVoltage | (8u) |
| Definition |  |
| #define KU16_ERH_SC4_SystemVoltage (8u) |  |
| Description |  |
| SystemVoltage mask |  |

### KU16_ERH_SC5_TransportationMode

| Name | Value |
| --- | --- |
| KU16_ERH_SC5_TransportationMode | (16u) |
| Definition |  |
| #define KU16_ERH_SC5_TransportationMode (16u) |  |
| Description |  |
| TransportationMode mask |  |

### KU16_ERH_SC8_CommunicationStatus

| Name | Value |
| --- | --- |
| KU16_ERH_SC8_CommunicationStatus | (32u) |
| Definition |  |
| #define KU16_ERH_SC8_CommunicationStatus (32u) |  |
| Description |  |
| CommunicationStatus mask |  |

### KU16_ERH_TIMER_TRESHOLD

| Name | Value |
| --- | --- |
| KU16_ERH_TIMER_TRESHOLD | (500u) |
| Definition |  |
| #define KU16_ERH_TIMER_TRESHOLD (500u) |  |
| Description |  |
| Counter used for SC10 debounce timer |  |

# EEPROM

The EEPROM parameters are all specified in [Doc3 = SBE_4G_NVP_layout.xls].

Refer to this document for more details.

# Configuration

No special configuration for ERH software component.

# Compilation Options

No compilation options for ERH software component.